Appendix A - Error Code Explanations

For any other error codes - please contact Radius Product Services.

Code #		Problem Description	Probable Cause			Solution

1		Parity Error		1. Poor or no connection to		1. Check connections

radio

2. Defective radio			2. Replace logic board on the repeater?

microprocessor system.		    base station or return radio to depot

for servicing or replacement

mobile or portable radio.

2		No Response after Send	1. Poor or no connection to radio.	1. Check connections to radio.

2. Poor or no connection to rib.	2. Check connections to RIB.

3. Defective radio			3. Replace logic board on the repeater/

microprocessor system.		    base station or return radio 	to depot							4. Radio or RIB off.			    for servicing or replacement for a

mobile or portable radio.

4. Turn radio and or RlB on.

3		Innerbyte Delay Error	Defective radio			Replace logic board on the repeater?

microprocessor system.		base station or return radio to depot

for servicing or replacement for a

mobile or portable radio.

4		Checksum Error		1. Detective radio	                             	1. Replace logic board on the repeater/

microprocessor system		    base or return radio to depot,

2. Codeplug data bad.		    for servicing or replacement for a

mobile or portable radio.

2. Use default archive file.

5		No Power			No DC power to RIB or Radio		1. Check power

2. Try other COM port.

6		Multiple Collisions		Defective radio			1. Try other COM port.

microprocessor system		2. Replace logic board on the repeater/

base station or return to depot,

for servicing or replacement for a

mobile or portable radio.

7		Serial Bus Error		1. Invalid opcode			1. Try other COM port.

2. Wrong COM Port			2. Check connections.

3. Poor connections.

8		Negative Acknowledge	Radio or RSS version		Use latest version of RSS.

incompatible.

15		Model number do not	Radio model info does not		1. Read connected radio then

match.			match computer workspace		    program.

model information.			2. Try cloning Codeplug from

another radio.

20		RSS version does not					Use correct RSS Version for

match Radio Codeplug.					the model information.

21		Radio not Supported	Loaded the wrong software		Load Correct RSS.

22		Blank Board - radio not	An attempt to read blank		initialize logic board.

initialized			EEprom.

30		Serial Number mismatch	Attempting to program the		1. Read connected radio,

Model Number mismatch	Codeplug with Codeplug data		    program as desired.

Configuration Register	from another radio.			2. Use latest RSS version.

mismatch.

51-58		Radio Codeplug Error	Bad Codeplug data.			Codeplug is corrupted -

Call Product Services.