# Nucleus SCM AIPS/FIPS Reference Manual

Version: 1.30

Last Revised: Feb. 16, 1998

Revision History:

Version	Date	Author	Change History

1.10	01/09/98	K. Simonelic	Add changes for 4.030 and 4.100 versions.  Added this rev. history, not previously part of this document.

1.20	01/12/98	K. Simonelic	Various review updates. Change name of document.

1.30	02/16/98	K. Simonelic	Fixes from 4.100 integration testing.

1.

OVERVIEW

There are two serial port interfaces for communicating to the SCM board in the Nucleus transmitter; AIPS and FIPS.

AIPS, (ASCII Integrated Paging System) is used by the NIU (Network Interface Unit) or other controller to transfer programming, alarm, and maintenance information to/from the SCM. AIPS was intended to be a machine to machine interface. AIPS is accessible via the NIU front panel 9-pin D-connector, or via a dial up modem connection. The External NIU does not communicate to the SCM via an AIPS interface.

FIPS (Friendly Integrated Paging System) is normally used by Motorola’s factory during station programming, or for debugging/programming by development or field engineers. FIPS is accessed via the SCM’s front panel 9-pin D-connector.

CAUTION !

AIPS and FIPS were originally created to aid in the programming, alignment, and development of the Nucleus transmitter software and hardware, and for communicating alarm and diagnostic information to the internal NIU. AIPS and FIPS affords the experienced user/technician the ability to diagnose station operation, or modify programming parameters locally or remotely. With this ability comes inherent dangers. Modifying certain parameters may adversely affect station operation or alignment. It is important that care be taken when using AIPS/FIPS commands, as there is limited error checking performed by the station software. Mis-typing commands, or entering commands that do not appear in “ACTION COMMANDS” on page 8 could have catastrophic results on station operation; it could take the station off the air and result in the need for a site visit, and potentially, require the replacement of station hardware. Similarly, writing a value to one of the read/write parameters may also cause catastrophic results.

2. FIPS COMMUNICATIONS

2.1. Connecting via FIPS

Normally, when you are servicing a station at the site, you will want to use FIPS. Connect your dumb terminal (PC running terminal emulator software) through a NULL MODEM to the 9-pin D-connector on the front of the Nucleus SCM board (not the NIU). The default settings for the terminal should be 9600 baud, 8 bits, and No Parity bit.

Press the ENTER key on the terminal several times until ENTER PROTOCOL is displayed on the screen. If you do not see ENTER PROTOCOL, check your connection. If you see some characters, the baud rate is probably incorrect. If you see nothing, you probably need to install or remove a NULL MODEM (Some cables have a NULL MODEM built in).

Once you see the ENTER PROTOCOL prompt,. type:

FIPS<CR> (you can use upper or lower case)

If the Station Password is ENABLED you will see:

ENTER PASSWORD

Type:

xxxx<CR> (where xxxx is the station password, default is 6000)

You should now see the following displayed on the terminal screen:

FIPS:

You are now ready to send/receive FIPS commands.

2.2. FIPS Message Format

This FIPS handler in the SCM accepts commands or responses of the following form:

   [R | r | W | w | A | a]   [ID code] [argument]<CR>

The first field is the command/response type:

R or r = Read parameter
	W or w = Write parameter
	A or a = perform an Action

There are three types of FIPS commands; Read, Write, or Action. The Read command is used to read a value from the SCM parameter database. Similarly, the Write command is used to write a value to the parameter database. There are several Action commands, which cause the station to perform some type of action.

The ID code field is 1-3 ASCII characters which represents the parameter number for read and write commands, or the action ID for action commands.

Many commands and responses require an argument. The argument format is command-dependent, but is always composed of ASCII characters. Write parameter arguments range from a single ASCII number to a string of ASCII numbers representing a floating point number, such as 900.1000. Action commands may have numeric or string arguments.

All commands are terminated with an ASCII carriage return, <CR>.

FIPS commands are NOT case-sensitive, unlike AIPS commands which must be entered in all capital letters.

2.3. Commonly Used FIPS Command Examples

2.3.1. Reading Current Forward Power via FIPS

From Table 2, we determine that the parameter number for the current forward power reading is 131 (FORWARD_POWER).

The following command will retrieve the current forward power reading:

	R 131 <CR>

The response to this command is:

	RR 131 xxx (where xxx is the current forward power in Watts)

NOTE: The FORWARD_POWER parameter gives the latest KEYED power reading. If the station is dekeyed it will not return 0 Watts (unless the station has not keyed since it was last reset).

2.3.2. Changing SCM Front Panel Serial Port to 19200 Baud via FIPS

The parameter number associated with the front panel serial port (FPSP) is 140 (FPSP_BAUD_RATE). We determine that 5 corresponds to 19200 baud. To change the baud rate to 19200 type the following:

W 140 5<CR>

NOTE: The Front Panel Serial Port baud rate is only set during station power up. For the change above to take effect, reset the station (via A 117 FIPS command, or by pressing the 1 and 3 buttons on the front panel simultaneously). You will also need to change the baud rate of the terminal you are using to communicate with the SCM.

2.3.3. Reading the SCM Flash ROM Bank Software Versions via FIPS

From Table 1 below, we determine that action command 33 (RSDM_GET_VERSION, Remote Software Download Manager Version) is the command used to retrieve the Flash ROM Bank Versions. Type the following to get the versions of software residing in the Active and Inactive Flash ROM banks:

A 34<CR>

The SCM responds after approximately 7 seconds with a message similar to:

RA 34 3.210 2.920

NOTE: The Active Software Flash Bank is left-most version number. If there is only one valid Flash bank, only one version will be returned. Read parameter 901 to determine whether Bank A or Bank B is currently active.

3. AIPS COMMUNICATIONS

3.1. Connecting via AIPS

Normally, AIPS is used when communicating with the SCM via a dumb terminal through the NIU. This is normally done remotely via a dial up modem connected or installed in the Internal NIU. See the Internal NIU Configuration of the Nucleus Paging Station User’s Guide for details on connecting to the NIU.

Once you have connected to the NIU and the “NUCNIU>” prompt is displayed, enter the following to begin communicating to the SCM via AIPS:

NUCNIU> scm cmd nucleus<CR>     (Not case-sensitive)

The following prompt will be displayed:

Enter SCM command:

The NIU is now ready to transfer AIPS commands to the SCM. All AIPS commands must be in UPPER-case.

3.2. AIPS Message Format

This AIPS handler accepts commands or responses of the following form:

      [CR | CW | CA]   [ID code] [argument]<CR>

The second field is the command/response type:
	CR = Command Read parameter
	CW = Command Write parameter
	CA = Command perform IPS Action

The ID code field is 1-3 ASCII characters which represents the parameter number for read and write commands, or the IPS action ID for action commands.

Many commands and responses require an argument. The argument format is command-dependent, but is always composed of ASCII characters. Write parameter arguments range from a single ASCII number to a string of ASCII numbers representing a floating point number, such as 900.1000. Action commands may have numeric or string arguments. All commands are terminated with an ASCII carriage return, <CR>.

NOTE: When using AIPS via the NIU certain commands may not be supported due to limitations on the NIU user interface.

3.3. Common AIPS Command Examples

3.3.1. Reading Output Power via AIPS

From Table 2, we determine that the parameter number for the current output power setting (assuming Channel-mapped power is disabled) is 88 (NORMAL_POWER_LEVEL).

The following command will retrieve the power output setting:

	CR 88<CR>

The response to this command is:

	RR 88 xxx<CR> (where xxx is the current operating power in Watts)

3.3.2. Changing Nucleus Operating Power via AIPS

The following example will change the operating power of the station to 100 Watts:

	CW 88 100<CR>

NOTE: The SCM does a minimal amount of error checking when commands are entered via FIPS or AIPS. If you try to write a value that is not valid, the SCM will not accept the change. To be certain that the SCM accepted the change, you may want to read the parameter immediately after writing it.

3.3.3. Resetting the Nucleus SCM via AIPS

From Table 1 below, we determine that the command to reset the Nucleus is 117. To execute the command type:

CA 117 <CR>

NOTE: After executing this command, the SCM will go through a reset. You will not be able to communicate to the NIU (or SCM) until the SCM completes resetting.

4.

Table 1: FIPS/AIPS Action Commands

| Command Abbreviation | Action ID | t | Software Version | Description |
| --- | --- | --- | --- | --- |
| RSDM_SWITCH_BANKS | 33 | 7 | 2.340+ | Causes Station to Switch Active Flash Bank, then resets the station. |
| RSDM_GET_VERSION | 34 |  | 2.340+ | Returns Active and Inactive Flash Bank Version numbers. Active bank is displayed first. This action requires approx. 7 seconds to complete. |
| RSDM_UPDATE_DORMANT_BANK | 35 |  | 4.000+ | Copies the content of the Active Flash Bank into the Inactive Flash Bank. Returns Inactive Flash Bank Version numbers. This action requires approx. 70 seconds to complete. |
| READ_ALL_ALARMS | 99 |  | 2.340+ | Displays Active Alarm 4-digit codes. (use 162 to interpret alarms) |
| CLEAR_SINGLE_ACTUAL_ALARM | 102 |  | 2.340+ | Clears specified alarm from log. Will only clear the alarm if it is “clearable”. Format: A 102 FFFF (where FFFF is the 4-digit alarm code to be cleared). |
| CLEAR_ALL_ACTUAL_ALARMS | 103 |  | 2.340+ | Clears Alarm Log (ALARM LED on front of station will go off after this command is sent) |
| INITIATE_SOFTWARE_DOWNLOAD | 114 | 7 | 2.340+ | Command used to initiate software download into Bank B. If the station does not receive Flashdownload information within 20 seconds, the station will reset. |
| KEY_STATION | 115 | 7 | 2.340+ | Causes the station to key on the current channel |
| DEKEY_STATION | 116 | 7 | 2.340+ | Dekeys the station |
| INITIATE_STATION_RESET | 117 | 7 | 2.340+ | Causes station reset |
| READ_ACTUAL_INT_WM_POWER | 153 |  | 2.340+ | Returns Forward followed by Reflect Power in Watts. Will return 0 0 if the station is dekeyed. |
| LIST_ALL_ALARM_IDS | 161 |  | 2.410+ | Lists all current supported Alarm IDs. The first two characters of the value returned represents the number of alarms supported (in hexadecimal). Every 4 hex characters thereafter represents the Alarm IDs. |
| XLATE_ALARM_ID_TO_STRING | 162 |  | 2.410+ | Returns the verbose interpretation of the Alarm IDs. For example, “a 162 F881” returns “HIGH REFLECTED POWER”. |
| STATION_MINIMAL_MODE_RESET | 177 | 7 | 3.210+ | Allows remote resetting of a station that has gone into Minimal Mode. Older software requires that the station power be turned off, then on, to remove it from Minimal Mode. The station enters Minimal Mode when the station experiences several resets within a short period of time, usually indicating a serious station failure condition. |
| AUTO_PA_CAL_START | 180 | 7 | 3.210+ | Causes the station equipped with a Nucleus II PA to key and calibrate the power out. NOTICE: The Calibration is done on the Mean Frequency. In multi-freq stations this may not be a valid FCC approved frequency, therefore the station must be connected to an RF Load (other than the transmit antenna) during the PA calibration procedure. |
| A 7 in the t column means that paging traffic will be interrupted for some period of time if the corresponding action command is executed. Extra caution should be exercised when using these commands. |  |  |  |  |

ACTION COMMANDS

5.

Read/Write Parameters

Table 2: Nucleus SCM Parameter Database ID’s

| Parameter Abbreviation | Param ID | t | Software Version | Description/ Front Panel Menu Location |
| --- | --- | --- | --- | --- |
| LOW_SPEED_OFFSET | 70 |  | 2.340+ | TX/TX CHN OFFSETS/LOW SPEED OFFSET (Hz) |
| HIGH_SPEED_OFFSET | 71 |  | 2.340+ | TX/TX CHN OFFSET/HIGH SPEED OFFSET (Hz) |
| INT_WM_FWD_PWR_LIMIT | 75 |  | 2.340+ | ASET/FWD PWR ALM PT (Watts) NOTE: Parameters 75 and 77 are used when Channel Mapped Power is DISABLED. When channel mapped power is ENABLED, parameters 472-478 and 503-510 should be used to set internal wattmeter forward and reflected alarm trip points. |
| INT_WM_RFL_PWR_LIMIT | 77 |  | 2.340+ | ASET/RFL PWR ALM PT (Watts) |
| RATED_POWER_LEVEL | 78 |  | 2.340+ | CNFG/MAX PWR (Watts) |
| FIXD_CUTBK_PWR_REDUC_REQ | 87 |  | 2.340+ | CNFG/BATTERY REVERT SETUP/FIXED CUTBK RED % (%) |
| NORMAL_POWER_LEVEL | 88 |  | 2.340+ | TX/TX CHN PWR/OPERATING PWR NOTE: Parameters 88 is used when Channel Mapped Power is DISABLED. When channel mapped power is ENABLED, parameters 633-641 should be used to set individual channel power settings. |
| TX_FREQ_BAND | 89 |  | 2.700+ | STN/TX FREQ RANGE Exciter Frequency Band 0=VHF R1, 1=VHF R2, 2=UHF R1, 3=UHF R2, 4=900 R1, 5=280MHz, 6=900 R2, 7=VHF H1, 8=VHF H2, 9=VHF H3, 10=VHF H4,11=322MHz, 12=UHF R2,13=UHF R4 |
| T_EQ_R_FREQ_SHIFT_DISABLED | 92 |  | 2.710+ | TX/SPECIAL TX SETUP/TX=RX 0=DISABLED, 1=ENABLED |
| PS_BATTERY_TYPE | 96 |  | 2.340+ | CNFG/BATTERY REVERT SETUP/BATTERY TYPE 2 = Sealed Lead Calcium 5 = DISABLED |
| PS_BATTERY_CHARGING | 98 |  | 2.600+ | CNFG/BATTERY REVERT SETUP/BATTERY TYPE 0=DISABLED, 1=ENABLED |
| PAGING_ACCESS_DISABLED | 99 | 7 | 2.340+ | DIS/PAGING ACCESS 0=ENABLED,1=DISABLED |
| MAINT_ACCESS_DISABLED | 100 | 7 | 2.340+ | DIS/MAINT ACCESS 0=DISABLED, 1=ENABLED |
| TX_EQ_RX_CHANNEL | 101 |  | 2.900+ | TX/SPECIAL TX SETUP/TX=RX CHANNEL 0=Chan 1, 1=Chan 2,.... |
| STARTUP_CHANNEL | 103 |  | 2.340+ | NO MENU ITEM Channel changed to after power reset (Default 0 = Channel 1) |
| CURRENT_CHANNEL_NUMBER | 105 |  | 2.340+ | STN/CURRENT TX CHN 0=Chan 1, 1=Chan 2,... |
| DEMODULATOR_INVERT | 107 | 7 | 2.340+ | RX/RX OUTPUT 0=Not Inverted, 1=Inverted |
| SYSTEM_TIMER_ALARM_VALUE | 108 |  | 2.340+ | STN/SYS TIMER ALARM Unit - Minutes Must be 2,15,30,60,90,120 or 180. |
| STATION_PTT_STATUS | 119 |  | 2.340+ | NO MENU ITEM 0=No PTT Req, 1=Line, 2=Local, 3=Diagnostics,4=Service |
| STATION_MODULATION_TYPE | 120 |  | 2.340+ | NO MENU ITEM 0=Analog 1=Binary |
| WILDCARD_1 | 121 |  | 2.340+ | NO MENU ITEM Alarm mapped to Wildcard Output 1 (Default = 64064, or hex FA40, PA FAIL) |
| WILDCARD_2 | 122 |  | 2.340+ | Alarm mapped to Wildcard Output 2 (Default = 63625, or hex F889, SYNTH OUT OF LOCK) |
| WILDCARD_3 | 123 |  | 2.340+ | Alarm mapped to Wildcard Output 3 (Default = 63488, or hex F800, NOT DEFINED) |
| WILDCARD_4 | 124 |  | 2.340+ | Alarm mapped to Wildcard Output 4 (Default = 63647, or hex F89F) When either Internal or External Wattmeter Low Forward Power Alarm occurs, WC4 output is activated. |
| WILDCARD_5 | 125 |  | 2.340+ | Alarm mapped to Wildcard Output 5 (Default = 63648, or hex F8A0) When either Internal or External Wattmeter High Reflected Power Alarm occurs, WC5 is activated. |
| WILDCARD_6 | 126 |  | 2.340+ | Alarm mapped to Wildcard Output 6 (Default = 63622, or hex F886, BATTERY REVERT) |
| WILDCARD_7 | 127 |  | 2.340+ | Alarm mapped to Wildcard Output 7 (Default = 63627, or hex F88B, PA FAN) |
| WILDCARD_8 | 128 |  | 2.340+ | Alarm mapped to Wildcard Output 8 (Default = 63642, or hex F89A, SYS TIMER EXPIRED) |
| PS_BATTERY_REVERT_ACT | 130 |  | 2.340+ | NO MENU ITEM 1=Battery Revert Active 0=Inactive |
| FORWARD_POWER | 131 |  | 2.340+ | STAT/FWD PWR |
| REFLECTED_POWER | 132 |  | 2.340+ | STAT/RFL PWR |
| PSTN_PORT_MODE | 133 | 7 | 2.340+ | NO MENU ITEM Internal NIU/SCM Comm Port 12=FIPS, 15=AIPS(default) |
| FPSP_BAUD_RATE | 140 |  | 2.340+ | NO MENU ITEM Front Panel Serial Port Baud Rate 1=1200, 2=2400, 3=4800, 4=9600, 5=19200, 6=38400 (Default 9600) |
| EXT_WM_FWD_PWR_LIMIT | 141 |  | 2.340+ | ASET/EXT WM FWD PWRALM PT (Watts) NOTE: Parameters 141 and 142 are used when Channel Mapped Power is DISABLED. When channel mapped power is ENABLED, parameters 535-542and 567-574 should be used to set internal wattmeter forward and reflected alarm trip points. |
| EXT_WM_RFL_PWR_LIMIT | 142 |  | 2.340+ | ASET/EXT WM RFL PWR ALM PT (Watts) |
| VSWR | 143 |  | 2.340+ | STAT/VSWR |
| EXT_WM_VSWR | 144 |  | 2.340+ | STAT/EXT WM VSWR |
| EXT_WM_FORWARD_POWER | 145 |  | 2.340+ | STAT/EXT WMFWD PWR |
| EXT_WM_REFLECTED_POWER | 146 |  | 2.340+ | STAT/EXT WM RFL PWR |
| BOOT_SW_VERSION_ID | 147 |  | 2.340+ | STAT/SOFTWARE VERSIONS/BOOT |
| ACTIVE_APPL_SW_VERSION_ID | 148 |  | 2.340+ | STAT/SOFTWARE VERSIONS/APPLICATION |
| TX_EXCITER_SW_VERSION_ID | 149 |  | 2.340+ | STAT/SOFTWARE VERSIONS/EXCITER |
| DORMANT_APPL_SW_VERSION_ID | 152 |  | 2.340+ | NO MENU Inactive Flash Bank Software Version |
| WIB_HW_VERSION_ID | 156 |  | 2.340+ | Wireline Interface Board HW ID |
| SW_VERSION_CHECKING | 157 |  | 2.340+ | NO MENU ITEM 2=DO NOT Download Exciter Software at startup (Run exciter software out of EPROM) 1=Download Exciter Software from SIMM, if Exciter is downloadable. |
| P_TX_ALIGNMENT_ID | 158 |  | 2.340+ | NO MENU ITEM Holds the 4-level alignment ID contained in the exciter. This should match the SCM ID, parameter 887, when the station is properly aligned. |
| EXT_WM_TYPE | 159 |  | 2.340+ | CNFG/EXT WATTMETER TYPE 0=None, 1=Class 1, 2=Class 2, 3=Class 3 4=Class 4 |
| TX_EXCITER_HW_VERSION_ID | 160 |  | 2.340+ | Exciter Hardware ID 0,1 = Non-Downloadable 2+ = Software Downloadable |
| MONITOR_RX_OUTPUT | 161 | 7 | 2.340+ | RX/MONITOR RX OUTPUT 1=Analoog 2=TTL/C-NET |
| REAL_TIME_CLOCK_YEAR | 162 |  | 2.340+ | STN/SET STATION TIME/YEAR |
| REAL_TIME_CLOCK_MONTH | 163 |  | 2.340+ | STN/SET STATION TIME/MONTH |
| REAL_TIME_CLOCK_DAY | 164 |  | 2.340+ | STN/SET STATION TIME/DAY |
| REAL_TIME_CLOCK_HOUR | 165 |  | 2.340+ | STN/SET STATION TIME/HOUR |
| REAL_TIME_CLOCK_MINUTE | 166 |  | 2.340+ | STN/SET STATION TIME/MINUTE |
| REAL_TIME_CLOCK_SECOND | 167 |  | 2.340+ | STN/SET STATION TIME/SECONDS |
| EXT_CIRC_PRESENT | 168 |  | 2.340+ | OPT1/EXT CIRCULATOR 0=Not Present 1=Present |
| STATION_TYPE | 170 |  | 2.340+ | NO MENU ITEM 1=High Speed Nucleus |
| INTERNAL_RX_TYPE | 171 | 7 | 2.340+ | CNFG/RX TYPE 0=None, 1= Internal Link, 2=Internal Monitor |
| DE_EMPHASIS_ENABLE | 175 | 7 | 2.340+ | RX/RX DEEMPHASIS 0=Disabled, 1=Enabled |
| RX_BAND | 176 |  | 2.340+ | RX/RX FREQ RANGE 0=Midband, 1=VHF R1, 2=VHF R2, 3=280 MHz, 4=322 MHz, 5=UHF R1, 6=UHF R2, 7=UHF R3, 8=UHF R4, 9=800 MHz, 10=900 MHz, 11=900 R1, 12=900 R2 |
| FREQ_REFERENCE_TYPE | 180 | 7 | 4.000+ | REFERENCE FREQ SOURCE 0 = Internal UHSO, 1 = Internal HSO, 2 = External 5 MHz, 3 = External 10 MHz, 4= Internal NIU |
| LOW_SPEED_SPLATTER_FILTER | 182 | 7 | 2.340+ | TX/LOW SPEED SPLATTER FILER 0=88 usec, 2=140 usec, 3=250 usec, 8=160 usec |
| HIGH_SPEED_SPLATTER_FILTER | 183 | 7 | 2.340+ | TX/HIGH SPEED SPLATTER FILTER 0=88 usec, 8=160 usec |
| RX_SQUELCH_RESP | 184 |  | 2.340+ | NO MENU ITEM 0=Internal Rx Squelched, 1=Unsquelched |
| IDLE_DEVIATION | 186 | 7 | 2.340+ | TX/IDLE DEVIATION 0=Null, 1=Space, 2=Mark |
| KEY_SYMBOL | 187 |  | 2.700+ | SERV/SELECT SYMBOL 1=Staircase, 2=DADA, 3=EBEB, 4=Continuous A, 5=Cont B, 6=Cont D, 7=Cont E, 8=Cont C |
| P_CURRENT_MODULATION_TYPE | 225 | 7 | 4.000+ | NO MENU ITEM 0=ERMES, 1=POCSAG_250, 2=POCSAG_125, 3=POCSAG_150, 4=FLEX, 5=POCSAG_88, 6=POCSAG_100, 7=FLEX_1600_2L, 8=REFLEX Note: The DSP uses only 3 different protocol modes, ERMES, ReFlex, and Flex/POCSAG. Thus, settings 1-7 are functionally equivalent. The numbers are only included for Ericsson I20 spec. compatibility, they do not have meaning for a Nucleus station. |
| SCM_HW_VERSION_ID | 229 |  | 4.000+ | STAT/SCM HARDWARE VERSION 0=Non I20 SCM 1=I20 Compatible SCM |
| I20_DSP_ENABLE | 230 | 7 | 4.000+ | NO MENU ITEM 0=SYLC_INTERFACE 1=I20_INTERFACE Changes to this parameter will cause the station to reset. |
| P_FLEX1600_LOW_SPEED | 231 |  | 4.000+ | NO MENU ITEM FLEX_BAUD_1600_FLEX = 0, FLEX_BAUD_1600_POCSAG=1 |
| RXDSP_SANITY_CHECK_ENABLED | 235 |  | 4.000+ | NO MENU ITEM 0=disabled, 1=enabled This parameter is not stored in NVM. It is always enabled after a station reset. This parameter provides a means to temporarily disable this function for testing purposes. |
| P_CHN_MAP_I20_PROTOCOL | 236 | 7 | 4.100+ | NO MENU ITEM 0=CHN_MAP_DISABLED 1=REFLEX_ONLY 2=ERMES_ONLY |
| P_CHN_MAP_I20_REFLEX_CNFG | 237 |  | 4.100+ | NO MENU ITEM Holds the channel numbers which are mapped to ReFlex-only. Each bit corresponds to a channel number, bit 0 = channel 1, bit 19 = chn 20. A set bit maps ReFlex to the corresponding channel. |
| P_CHN_MAP_I20_ERMES_CNFG | 238 |  | 4.100+ | NO MENU ITEM Holds the channel numbers which are mapped to ERMES-only. Each bit corresponds to a channel number, bit 0 = channel 1, bit 19 = chn 20. A set bit maps ERMES to the corresponding channel. |
| P_HW_SELECT_PROTOCOL_CNTL | 239 | 7 | 4.100+ | NO MENU ITEM 0=HW_PROTOCOL_CNTL 1=XM_CHN_MAP_PROTOCOL_CNTL |
| P_EPIC_TEMPERATURE | 240 |  | 4.100+ | NO MENU ITEM Holds the current temperature from the Epic IC i n Celsius. |
| CHN_1_LOW_FWD_PWR_PT to CHN_32_LOW_FWD_PWR_PT | 471 to 502 |  | 2.410 to 3.200 | CHN 1 - 32 FWD CHAN MAPPED ALM PT (Watts) |
| CHN_1_HIGH_RFL_PWR_PT to CHN_32_HIGH_RFL_PWR_PT | 503 to 534 |  | 2.410 to 3.200 | CHN 1 - 32 HIGH REFLECTED CHAN MAPPED ALM PT (Watts) |
| CHN_1_EXT_LOW_FWD_PWR_PT to CHN_32_EXT_LOW_FWD_PWR_PT | 535 to 566 |  | 2.410 to 3.200 | CHN 1 - 32 EXT FWD ALM PT (Watts) |
| CHN_1_EXT_HIGH_RFL_PWR_PT to CHN_32_EXT_HIGH_RFL_PWR_PT | 567 to 698 |  | 2.410 to 3.200 | CHN 1 - 32 EXT RFL ALM PT (Watts) |
| CHANNEL_1_FREQUENCY_VALUE to CHANNEL_32_FREQUENCY_VALUE | 601 to 632 | 7 | 2.410 to 3.200 | TX/CHN FREQS CHN 1 - 32 FREQ (MHz) |
| CHN_1_LOW_FWD_PWR_PT to CHN_8_LOW_FWD_PWR_PT | 471 to 478 |  | 3.300 to 3.400 | CHN 1 - 8 FWD CHAN MAPPED ALM PT (Watts) |
| CHN_1_HIGH_RFL_PWR_PT to CHN_8_HIGH_RFL_PWR_PT | 503 to 510 |  | 3.300 to 3.400 | CHN 1 - 8 HIGH REFLECTED CHAN MAPPED ALM PT (Watts) |
| CHN_1_EXT_LOW_FWD_PWR_PT to CHN_8_EXT_LOW_FWD_PWR_PT | 535 to 542 |  | 3.300 to 3.400 | CHN 1 - 8 EXT FWD ALM PT (Watts) |
| CHN_1_EXT_HIGH_RFL_PWR_PT to CHN_8_EXT_HIGH_RFL_PWR_PT | 567 to 574 |  | 3.300 to 3.400 | CHN 1 - 8 EXT RFL ALM PT (Watts) |
| CHANNEL_1_FREQUENCY_VALUE to CHANNEL_8_FREQUENCY_VALUE | 601 to 608 | 7 | 3.300 to 3.400 | TX/CHN FREQS CHN 1 - 8 FREQ (MHz) |
| CHN_1_LOW_FWD_PWR_PT to CHN_20_LOW_FWD_PWR_PT | 471 to 490 |  | 4.000+ | CHN 1 - 20 FWD CHAN MAPPED ALM PT (Watts) |
| CHN_1_HIGH_RFL_PWR_PT to CHN_20_HIGH_RFL_PWR_PT | 503 to 522 |  | 4.000+ | CHN 1 - 20 HIGH REFLECTED CHAN MAPPED ALM PT (Watts) |
| CHN_1_EXT_LOW_FWD_PWR_PT to CHN_20_EXT_LOW_FWD_PWR_PT | 535 to 554 |  | 4.000+ | CHN 1 - 20 EXT FWD ALM PT (Watts) |
| CHN_1_EXT_HIGH_RFL_PWR_PT to CHN_20_EXT_HIGH_RFL_PWR_PT | 567 to 586 |  | 4.000+ | CHN 1 - 20 EXT RFL ALM PT (Watts) |
| CHANNEL_1_FREQUENCY_VALUE to CHANNEL_20_FREQUENCY_VALUE | 601 to 620 | 7 | 4.000+ | TX/CHN FREQS CHN 1 - 20 FREQ (MHz) |
| ARITHMETIC_MEAN_CHANNEL | 633 |  | 2.340+ | TX/MEAN FREQ (MHz) |
| ARITHMETIC_MEAN_CHANNEL | 621 |  | 4.000+ | TX/MEAN FREQ (MHz) |
| CHANNEL_1_POWER_VALUE to CHANNEL_32_POWER_VALUE | 634 to 665 |  | 2.410 to 3.200 | TX/TX CHN PWR CHN 1 - 32 PWR (Watts) |
| CHANNEL_1_POWER_VALUE to CHANNEL_8_POWER_VALUE | 634 to 641 |  | 3.300 to 3.400 | TX/TX CHN PWR CHN 1 - 8 PWR (Watts) |
| CHANNEL_1_POWER_VALUE to CHANNEL_20_POWER_VALUE | 634 to 643 |  | 4.000+ | TX/TX CHN PWR CHN 1 - 20 PWR (Watts) |
| ARITH_MEAN_CHN_POWER_VALUE | 666 |  | 2.410+ | TX/MEAN FREQ PWR (Watts) |
| ARITH_MEAN_CHN_POWER_VALUE | 654 |  | 4.000+ | TX/MEAN FREQ PWR (Watts) |
| CHN_MAPPED_PWR_CNFG | 667 | 7 | 2.410+ | OPT1/CHN MAPPED PWR 0=Disable, 1=Enabled |
| RX_CHN_1_FREQUENCY_VALUE | 681 |  | 2.340+ | RX/RX CHN FREQ (MHz) |
| NOMINAL_BINARY_DEVIATION | 700 | 7 | 2.340+ | TX/NOMINAL BINARY DEVIATION (Hz) |
| TX_DATA_POLARITY | 701 | 7 | 2.340+ | TX/SPECIAL TX SETUP/TX DATA INVERT 0=DIsabled, 1=Enabled |
| SPECIAL_KEY_OPTION | 704 | 7 | 2.340+ | CNFG/SPECIAL KEY SELECT 0=None, 1=CD INT, 2=CD EXT, 3=Special, 4=FAST LOW, 5=FAST HIGH, 6=EXT LOW, 7=EXT HIGH (Normal C-Net systems use EXT LOW) |
| ANTENNA_RELAY_STATE | 705 | 7 | 2.340+ | OPT1/ANTENNA RELAY 0=Disabled,1=Enabled |
| STATION_CONFIGURATION | 706 | 7 | 2.340+ | CNFG/CONTROL 1=Internal CNET (Internal NIU) 2=External SyLC |
| USER_PASSWORD | 707 | 7 | 2.340+ | STN/PASSWORD |
| RX_CHANNEL_SPACING | 708 | 7 | 2.340+ | RX/CHN SPACING 0=12.5 KHz, 1=20 KHz, 2=25 KHz |
| ANTENNA_RELAY_MODE | 710 | 7 | 3.210+ | NO MENU ITEM 0=Normal (Switch Relay on every key/dekey) 1=Relay Saver Mode (Only switch to Rx when instructed by the NIU--during Maintenance) |
| P_ALIGNMENT_ID | 887 |  | 2.340+ | STAT/SOFTWARE VERSIONS/ALIGNMENT ID/SCM Holds the 4-level alignment ID contained in the SCM. This should match the exciter ID, parameter 158, when the station is properly aligned. |
| P_ALIGNMENT_TEMPERATURE | 888 |  | 2.340+ | NO MENU ITEM Temperature in Celsius when the 4-level alignment was performed. |
| ACTIVE_FLASH_BANK | 901 |  | 2.340+ | NO MENU ITEM (Use in conjunction with “A 114" - which always downloads to Bank B). 1=Flash Bank A is active 2=Flash Bank B is active |
| A 7 in the t column signifies that changing that parameter could adversely affect paging performance, or possibly damage the transmitter. Extreme caution should be used when writing to these parameters. |  |  |  |  |