External Repeater Controller Interface for MASTR-III Stations

HARDWARE:

All necessary I/O signals can be found on the back-plane interface connectors as follows:

Connector #	Pin #		Function

P5		1		+12V DC

P5		4		DC Ground

P5		9		Receiver Audio Low

P5		10		Receiver Vol/Sq High (this is raw discriminator audio that

is NOT de-emphasized)

P5		17		CAS  (carrier active squelch, active HIGH) (aka: COR)

P5		20		RUS  (receiver un-squelched, active HIGH) (aka: CTCSS & COR)

# P2		8		Transmit Audio Hi

P2		9		Transmit Audio Low

P2		14		Transmit PTT (active LOW)

Set jumper J104 to short pins 2 and 3.  (Located on T/R interface board)

Set jumper J105 to short pins 2 and 3.  (Located on T/R interface board)

SOFTWARE:

Use standard MASTR-III programming software to set the following configurations:

- Single channel, DC remote

- RF Simplex = NO

- RPT = NO

- PTT options:	set all CCT and DODT to “NO”

set all CG encode to “YES”

- Pot settings	TX pot = 200

CG pot = 120

Line out = 0

RPTR gain = 1023

Line in = 0

DSP line cancel = 0

DSP line in = 20

DSP compressor = 1023

DSP Comp Thres = 32767

Squelch pot = manual

- Morse code = “disabled”

- F7 options = set all to “disabled” except Control Line = “4 wire”

- Program TX/RX frequencies as desired

Align and tune the RF portion of the station as call for in the installation/tuning instruction manual.

de WA1ZMS      Oct ‘99