TECHNICAL SERVICE MANUAL

WITH

ILLUSTRATED PARTS BREAKDOWN

RACAL 25 PORTABLE RADIO

(PRC 6894)

RACAL COMMUNICATIONS, INC.

Figure 1-1

5 RESEARCH PLACE

ROCKVILLE, MD 20850

This device made under license under one or more of the following U.S. Patents: 5,146,497; 5,185,795; 4,636,791; 4,590,473; 5,185,796; 5,148,482; 5,271,017; 5,377,229; 4,833,701; 4,972,460.

The IMBETM voice coding technology embodied in this product is protected by intellectual property rights including patent rights, copyrights, and trade secrets of Digital Voice Systems, Inc.  The voice coding technology can only be used as part of the North American land mobile radio communication system for the APCO Project 25.  The user of this technology is explicitly prohibited from attempting to decompile, reverse engineer, or disassemble the Object Code, or in any way convert the Object Code into a human-readable form.

FCC ID:	OKC4101256

MODEL:	PRC6894

CANADA:	473195576A

RACAL COMMUNICATIONS, INC.

MADE IN USA

NOTICE

This Technical Service Manual contains information that is current as of the date shown below.  The information contained in this document is subject to change at any time, without prior notice

The Racal 25 Radio is a portable, handheld, battery operated radio capable of providing both secure and non-secure communications.  Immersible and weighing only 17 oz., the radio operates in the 136-174 Megahertz (MHz) band with adjustable Radio Frequency (RF) power output up to 5 Watts.  The Racal 25 portable conforms to the Project 25 Common Air Interface (CAI) standard.  A display and keypad provide easy programming and feature selection for the 256 available channels.  The radio has a separate internal speaker and microphone for improved audio fidelity.  A side connector is available for radio-to-radio cloning, programming from a personal computer (including key fill), and incorporating software upgrades.  The Racal 25 radio uses Digital Signal Processing (DSP) and microprocessor control for easy evolution towards advanced capabilities.

30 NOVEMBER 1999

This page left blank intentionally.

table of contents

FIGURES	ix

TABLES	ix

RACAL 25 PORTABLE RADIO (WITH BATTERY AND ANTENNA)	x

INTRODUCTION	xi

CHAPTER 1.0.  INSTALLATION, ADJUSTMENT AND OPERATION	1-1

1.1	Radio Configuration.	1-1

1.1.1	Channels.	1-1

1.1.2	Zones.	1-1

1.1.3	Banks.	1-1

1.2	Installation and Adjustment	1-1

1.2.1	Hardware	1-1

1.2.2	Software	1-1

1.3	Operating Procedures.	1-1

1.3.1	Connect the Battery	1-1

1.3.2	Connect the Antenna	1-1

1.3.3	Radio Programming	1-1

1.3.4	Radio Initialization.	1-1

1.3.5	Choose a Channel.	1-1

1.3.6	Transmit a Voice Message.	1-1

1.3.7	Receive a Voice Message.	1-2

CHAPTER 2.0.  RADIO SET AND ACCESSORIES	2-1

2.1	System Description.	2-1

2.1.1	Portable Radio.	2-1

2.1.2	Antennas.	2-1

2.1.2.1	Standard Broadband Antenna.	2-1

2.1.2.2	Low-Band Narrowband Antenna.	2-1

2.1.2.3	Mid-Band Narrowband Antenna.	2-1

2.1.2.4	High-Band Narrowband Antenna.	2-1

2.1.3	Battery.	2-1

2.1.3.1	Li-ion Rechargeable Battery	2-1

2.1.3.2	Alkaline Clamshell Battery Pack	2-2

2.1.4	Battery Chargers.	2-2

2.1.5	Surveillance Harness.	2-2

2.1.6	MA6941C PC Programmer	2-2

2.1.7	Cloning Cable	2-3

2.1.8	Belt Clip	2-3

2.1.9	Nylon and Leather Bags	2-3

2.1.10	Side Connector Assembly	2-3

2.2	Controls, Indicators, and Connectors.	2-3

2.2.1	Controls.	2-3

2.2.1.1	On/Off/Volume Rotary Knob	2-3

2.2.1.2	16 Channel Rotary Knob.	2-3

2.2.1.3	Three Position Toggle Switch.	2-3

2.2.1.4	Push-To-Talk (PTT) Switch.	2-3

2.2.1.5	Three Programmable-Function Side Keys.	2-3

2.2.1.6	Emergency Push Button.	2-3

2.2.1.7	16-Button DTMF Keypad.	2-4

2.2.2	Indicators.	2-4

2.2.2.1	Liquid Crystal Display (LCD).	2-4

2.2.2.2	LED Status Indicator.	2-4

2.2.2.3	Audible Tones.	2-4

2.2.3	Connectors.	2-4

2.2.3.1	Side Connector.	2-4

2.2.3.2	Antenna Connector.	2-4

2.2.3.3	Battery Connector.	2-4

2.3	Transceiver Characteristics.	2-4

2.3.1	Transmitter Characteristics.	2-4

2.3.1.1	Transmitter Output.	2-4

2.3.1.2	Transmit Squelch.	2-4

2.3.2	Receiver Characteristics.	2-5

2.3.2.1	Receiver Performance.	2-5

2.3.2.2	Receive Squelch.	2-5

2.4	Communication Security.	2-5

2.4.1	Algorithms.	2-5

2.4.2	Key-Fill.	2-5

2.4.3	Zeroization.	2-5

CHAPTER 3.0.  SOFTWARE THEORY OF OPERATION	3-1

3.1	Functional System(s) Operation.	3-1

3.1.1	Architecture.	3-1

3.1.2	Module Identification.	3-1

3.1.3	Power-On Self Test (POST).	3-1

3.1.4	Flash Software Upgrades.	3-2

3.1.5	Voice Coder/Decoder (VOCODER).	3-2

3.2	Radio Control Software.	3-2

3.2.1	Audio Control.	3-2

3.2.2	DSP Control Software.	3-2

3.2.3	Transceiver Module.	3-3

3.2.3.1	Transceiver Module Mode Control.	3-3

3.2.3.2	Transceiver Frequency Control.	3-3

3.2.3.3	12-Bit DAC	3-3

3.2.3.4	Transceiver Reference Oscillator Temperature Compensation.	3-3

3.2.3.5	Transceiver Receiver Control.	3-3

3.2.3.6	Transceiver Transmitter Control.	3-3

3.2.3.7	Transceiver Tx/Rx Switching.	3-4

3.2.3.8	Receiver Scanning.	3-4

3.2.4	DC Power Control.	3-4

3.2.4.1	Power Supply Frequency Control.	3-4

3.2.4.2	Battery Interface	3-4

3.2.5	Monitoring	3-4

3.2.6	Radio Control Drivers.	3-5

3.2.6.1	Audio/Power Supply Unit (PSU) Driver.	3-5

3.2.6.2	Transceiver Serial Bus Driver.	3-5

3.2.6.3	Digital Signal Processing (DSP) Host Driver.	3-5

3.2.6.4	BATBUS Driver.	3-5

3.2.6.5	IIC Bus Driver.	3-5

3.3	Digital Signal Processing.	3-5

3.3.1	DSP Transmit Chain.	3-5

3.3.1.1	Audio Coder/Decoder (CODEC).	3-5

3.3.1.2	Audio Processing Module.	3-5

3.3.1.3	Project 25 Voice Module.	3-5

3.3.1.4	CVSD DES Module	3-6

3.3.1.5	Analog FM Module	3-6

3.3.1.6	Modulation Module	3-6

3.3.1.7	Transceiver Interface	3-7

3.3.2	DSP Receive Chain	3-7

3.3.2.1	Transceiver	3-7

3.3.2.2	FM Demodulator	3-7

3.3.2.3	Signal Detectors	3-7

3.3.2.4	Project 25 Voice Module	3-9

3.3.2.5	Analog FM Module	3-9

3.3.2.6	CVSD DES Module	3-9

3.3.2.7	Audio Processing Module	3-9

3.3.2.8	Audio CODEC	3-9

3.3.3	DSP Software	3-9

3.4	Keypad MPU Software.	3-9

3.4.1	Overview.	3-9

3.4.2	General.	3-9

3.4.3	Keypad Scanning.	3-10

3.4.4	PTT Input.	3-10

3.4.5	Switch Input.	3-10

3.4.6	LED Output.	3-10

3.4.7	Backlight Control.	3-10

3.4.8	Serial Interface.	3-10

3.5	Data Interface.	3-10

3.5.1	CAI Data Interface.	3-10

3.5.2	Synchronous Serial Data Interface.	3-10

3.5.2.1	Receiver Synchronous Serial Data Buffering.	3-10

3.5.3	CAI Data Link Layer.	3-11

3.5.3.1	CAI Transmit Voice Mode.	3-11

3.5.3.2	CAI Transmit Voice Test Modes.	3-11

3.5.3.3	CAI Receive Voice Mode.	3-11

3.5.3.4	CAI Receive Voice Test Mode.	3-11

3.5.3.5	CAI Transmit Data Mode.	3-11

3.5.3.6	CAI Receive Data Mode.	3-11

3.5.4	Transmit Physical Link Layer.	3-11

3.5.4.1	Transmit Modulation Interface.	3-11

3.5.4.2	Transmit CAI Physical Link Layer.	3-11

3.5.4.3	Transmit Analog FM Physical Link Layer.	3-11

3.5.4.4	Transmit CVSD Physical Link Layer.	3-11

3.5.5	Receive Physical Link Layer.	3-11

3.5.5.1	Receive ADC and DAC Interface.	3-12

3.5.5.2	FM Demodulator.	3-12

3.5.5.3	Receive CAI Physical Link Layer.	3-12

3.5.5.4	Receive Analog FM Physical Link Layer.	3-12

3.5.5.5	Receive CVSD Physical Link Layer.	3-12

3.5.6	DES Encryption.	3-12

3.5.6.1	DES Kernel.	3-12

3.5.6.2	CAI Encryption.	3-12

3.5.6.3	DES Data Link Layer.	3-12

3.5.6.4	Key Interface.	3-12

3.5.6.5	Key Bank.	3-13

3.5.7	Host Interface.	3-13

3.5.8	Flash Interface.	3-13

3.5.9	Paging.	3-13

3.5.10	Hardware Control.	3-13

3.6	Controller Software.	3-13

3.6.1	Overview.	3-13

3.6.2	Environment.	3-13

3.6.2.1	General.	3-13

3.6.2.2	Scheduler.	3-13

3.6.2.3	Start-Up Software.	3-15

3.6.2.4	Shut Down Software.	3-15

3.6.2.5	Debug Driver Software.	3-15

3.6.3	Radio Store.	3-15

3.6.3.1	Physical Data Storage.	3-15

3.6.3.2	Data Types.	3-15

3.6.4	Program/Fill/Control Interface.	3-16

3.6.4.1	Radio Programming.	3-16

3.6.4.2	Radio Fill.	3-16

3.6.4.3	Radio Control Port.	3-16

3.6.4.4	PC Serial Port Driver.	3-16

3.7	MMI.	3-17

3.7.1	Display Description.	3-17

3.7.2	Keypad	3-17

CHAPTER 4.0.  HARDWARE THEORY OF OPERATION	4-1

4.1	Introduction.	4-1

4.2	Control Board.	4-1

4.2.1	Power Supply Unit.	4-3

4.2.1.1	Battery/External Power Switching.	4-3

4.2.1.2	On/Off Switching.	4-3

4.2.1.3	Transmit Power Switching.	4-3

4.2.1.4	4.5V Switch Mode Power Supply.	4-3

4.2.1.5	3.3V Linear Logic Supply.	4-3

4.2.1.6	Reset Generator.	4-3

4.2.1.7	5V Linear Logic Supply.	4-3

4.2.1.8	3V Backup Supply.	4-3

4.2.1.9	Low Voltage Detector.	4-3

4.2.1.10	Audio Supplies.	4-3

4.2.1.11	H8 Microcontroller.	4-5

4.2.1.12	H8 Input Requirements.	4-5

4.2.1.13	H8 Output Requirements	4-5

4.2.1.14	H8 Input/Output Requirements.	4-5

4.2.1.15	H8 Input Interrupt Requirements.	4-5

4.2.2	DSP.	4-5

4.2.3	Flash ROM.	4-6

4.2.4	RAM.	4-6

4.2.5	TCXO.	4-6

4.2.6	FPGA.	4-6

4.2.6.1	FPGA Configuration.	4-6

4.2.7	IF ADC.	4-6

4.2.8	Clock Generation.	4-6

4.2.9	Logic Audio CODEC.	4-6

4.2.10	Side Connector Interface & Filtering.	4-6

4.2.11	Transceiver Interface & Filtering.	4-6

4.2.12	Battery Interface & Filtering.	4-7

4.2.13	Keypad Interface.	4-7

4.2.14	Audio Interface.	4-7

4.2.15	Test Interface.	4-7

4.2.16	LED	4-7

4.2.17	Power Consumption.	4-7

4.3	Keypad Board.	4-7

4.3.1	Keypad Microcontroller.	4-7

4.3.2	LCD Power Supply.	4-7

4.3.3	Display	4-7

4.3.4	Keypad.	4-8

4.3.5	Backlighting.	4-8

4.3.6	Switch Interface.	4-8

4.3.7	Control Board Interface.	4-9

4.3.8	Power Consumption.	4-9

4.4	Transceiver.	4-9

4.4.1	Interface Section.	4-9

4.4.2	Transmitter Section.	4-9

4.4.2.1	Transmit Chain.	4-9

4.4.2.2	Power Amplifier Control.	4-10

4.4.2.3	PIN Diode Switch.	4-10

4.4.3	Receiver Section.	4-10

4.4.4	Synthesizer Section.	4-12

4.4.4.1	Synthesizer and Reference Oscillator.	4-12

4.4.4.2	Receive/Transmit VCOs and Buffer Amplifiers.	4-12

4.4.5	Digital/Analog Control.	4-13

4.5	External Interfaces.	4-14

4.5.1	Side Connector.	4-14

4.5.2	Battery Connector.	4-14

4.5.3	Antenna Connector	4-14

CHAPTER 5.0.  ASSEMBLY/DISASSEMBLY	5-1

5.1	General.	5-1

5.2	Power-On Self Test.	5-1

5.3	CCA Removal and Replacement	5-1

5.3.1	Disassemble the Radio	5-1

5.3.2	Reassemble the Radio.	5-2

CHAPTER 6.0.  TROUBLESHOOTING	6-1

6.1	Introduction.	6-1

6.2	How to Use This Section.	6-1

6.3	Radio Functional Tests.	6-1

6.3.1	Overall Radio Test.	6-1

6.3.1.1	Power-On Test.	6-1

6.3.1.2	Buttons and Switches Test.	6-1

6.3.1.3	Transmit Test	6-1

6.3.1.4	Receive Test	6-1

6.3.1.5	Audio Test	6-1

6.3.1.6	Accessories Test	6-2

CHAPTER 7.0.  SPECIFICATIONS AND PARTS LISTS	7-1

Section I - INTRODUCTION	7-1

7.1	Parts List.	7-1

7.2	Electrostatic Discharge (ESD) Sensitive Devices.	7-1

7.3	Specifications.	7-1

7.3.1	Physical Characteristics.	7-1

7.3.2	Technical Specifications	7-2

7.3.2.1	System	7-2

7.3.2.2	Transmit	7-2

7.3.2.3	Receive	7-2

7.3.3	Accessory Equipment	7-3

APPENDIX A.  MENU STRUCTURE	A-1

APPENDIX B.  DEFINITIONS	B-1

APPENDIX C.  SIGNAL TONES	C-1

APPENDIX D.  INTERFACES	D-1

APPENDIX E.  SOFTWARE UPDATE	E-1

APPENDIX F.  GLOSSARY	F-1

SCHEMATICS

FIGURES

Figure 2-1.  Controls and Connectors.	2-3

Figure 2-2.  Sixteen-Button Keypad	2-4

Figure 3-1.  Racal 25 Block Diagram	3-1

Figure 3-2.  DSP Transmit Chain Block Diagram	3-6

Figure 3-3.  DSP Receive Chain Block Diagram	3-8

Figure 3-4.  Controller Software Block Diagram	3-14

Figure 4-1.  Racal 25 Interconnection Diagram	4-1

Figure 4-2.  Control Board Block Diagram	4-2

Figure 4-3.  PSU Block Diagram	4-4

Figure 4-4.  Keypad Board Block Diagram	4-8

Figure 5-1.  Transceiver CCA (shown in Transceiver Module)	5-1

Figure 5-2.  Control CCA (shown in Control Module)	5-2

Figure 5-3.  Keypad CCA (shown in Control Module)	5-2

Figure 6-1.  Overall Radio Test	6-2

Figure 6-2.  Power-On Troubleshooting	6-3

Figure 6-3.  Controls Troubleshooting (Sheet 1 of 2)	6-4

Figure 6-4.  Transmit Troubleshooting	6-6

Figure 6-5.  Receive Troubleshooting.	6-7

Figure 6-6.  Accessories Troubleshooting Flowchart	6-8

Figure 7-1.  Racal 25 Portable Radio	7-4

Figure 7-2.  Racal 25 Portable Transceiver	7-5

Figure 7-3.  Racal 25 Control/Crypto/Display Module Assembly (4101254-501), Exploded View	7-6

Figure 7-4.  Racal 25 Transceiver Assembly (4101255-501)	7-8

Figure 7-5.  Racal 25 Keypad CCA (3100618-501)	7-10

Figure 7-6.  Racal 25 Control CCA (4101258-501)	7-11

Figure 7-7.  Racal 25 Transceiver Circuit Card (4101257-501), PRIMARY SIDE	7-13

Figure 7-8.  Racal 25 Transceiver Circuit Card (4101257-501), SECONDARY SIDE	7-15

Figure 7-9. Control Circuit Card (4101258-501), PRIMARY SIDE	7-31

Figure 7-10.  Control Circuit Card (4101258-501), SECONDARY SIDE	7-33

Figure 7-11.  Keypad Circuit Card (3100618-501) - PRIMARY SIDE	7-47

Figure 7-12.  Keypad Circuit Card (3100618-501) - SECONDARY SIDE	7-48

TABLES

Table 7-1.  Physical Characteristics	7-1

Table 7-2.  Accessory Equipment	7-3

Table 7-3.  Racal 25 Portable Radio (PRC6994HB-SYS)	7-4

Table 7-4.  Racal 25 Portable Transceiver (4101126-501)	7-5

Table 7-5.  Racal 25 Control/Crypto/Display Module Assembly (4101254-501), Exploded View	7-7

Table 7-6.  Racal 25 Transceiver Assembly (4101255-501)	7-9

Table 7-7.  Transceiver CCA (4101257-501) Bill of Materials	7-17

Table 7-8.  Control CCA (4101258-501) Bill of Materials	7-35

Table 7-9.  Keypad CCA (3100618-501) Bill of Materials	7-49

RACAL 25 PORTABLE RADIO (WITH BATTERY AND ANTENNA)

INTRODUCTION

Racal Communications, Inc. (RCI),which is headquartered in Rockville, MD, is a division of Racal Electronics, PLC, a multi-billion dollar international electronics company.  Our goal and the focus of our business is to be the recognized global leader in secure, tactical handheld radios:  We have received eleven NSA Type 1 encryption endorsements with two more in process.  We have built the world’s smallest and lightest full-featured tactical radio and we are using the latest commercial digital technology in our radios.

The Racal 25 radio is the smallest, lightest, and most rugged radio available that meets the requirements of the Association of Public Safety Communications Officials (APCO) Project 25 Common Air Interface (CAI) Standard.  It supports full multi-mode operation over a frequency range of 136 to 174 MHz and features high quality, error-corrected, digital voice and DES Encryption.  Programming is done via the radio front-panel keypad (password protected), by direct unit-to-unit cloning, or with a PC using the Windows 95/98/NT-based PC Programmer.  Software upgrades can be distributed via the Internet or disk without the requirement for special interface boxes or software.  The Racal 25 has exceptional scanning capabilities and an advanced Lithium-Ion Battery with fuel gauge display.  The Racal 25 radio has received FCC and Canadian approval.

Project 25 is a narrow band digital radio standard adopted by APCO.  It provides the guidance that allows US Government public safety agencies to move to 12.5 kHz channels.  Project 25 is the only Frequency Division Multiple-Access (FDMA) “standard” existing.

The standard requires:

- 12.5 kHz channels.

- Compatible 4-level Frequency Modulation (C4FM) or Compatible Quadrature Phase Shift Keying (CQPSK) modulation.

- Improved Multi-Band Excitation (IMBE) Voice Coder/Decoder (VOCODER).

- Common Air Interface (CAI).

The Project 25 standard will cover all the parts of a system for public safety land mobile radio communications.  The systems have subscriber units (which include portable radios for handheld operation and mobile radios for vehicular operation), base stations (for fixed installations), and other fixed equipment (for wide-area operation and console operator positions), as well as computer equipment (for data communications).  The CAI allows these radios to send and receive digital information over a radio channel.

Racal assumes that the user of this manual is familiar with radio and PC Programmer operations.  However, if required, Racal Communications has a handheld radio User's Guide (p/n 84326) and a PC Programmer User's Manual (p/n 84330) for sale.  Call 1-800-914-0303 for more details.

This manual provides required technical information necessary for the Racal 25 handheld radio.

This page left blank intentionally

# .  INSTALLATION, ADJUSTMENT AND OPERATION

DETAILED OPERATING INSTRUCTIONS ARE CONTAINED IN THE RACAL 25 USER'S GUIDE, RACAL PART NUMBER 84326.

## Radio Configuration.

### Channels.

A channel consists of a receive and transmit frequency band.  The radio has a total of 256 selectable channels.  Each channel may be programmed for different receive and transmit frequencies, squelch, modulation, encryption, and power.  Each channel can have one (of a possible sixteen (16)) keys assigned on a channel-by-channel basis.  Each channel is identified by an eight-character alpha-numeric label or by it's channel number if no text label is programmed.  Up to seven shadow channels may be added to each channel.  They enable the radio to be used in several squelch/encryption modes on each physical channel.

### Zones.

A zone is a group of channels.  Each zone can be assigned up to 16 channels.  The radio can store up to 16 zones, or groups of channels.  The zones may be assigned names of up to eight alphanumeric characters and can be assigned to Banks during programming.  Three zones can be selected by the toggle switch or 16 zones by the front panel keyboard.  Channels can be mapped to the channel select switch positions using the MA6941C PC Programmer.  When a zone is active (has been selected), the channels within the zone can be selected using the 16-position channel select switch on top of the radio.

### Banks.

A bank is a group of zones.  Zones can be assigned to banks during programming.  The radio can store up to four banks of 16 zones each.  Banks may be assigned names of up to eight characters.

## Installation and Adjustment

### Hardware

No installation or adjustments are required.

All functional parameters are set at the factory and do not require updating in the field.

### Software

Software updates will be periodically available. See Appendix E for detailed update procedures.

## Operating Procedures.

### Connect the Battery

The battery is connected to the radio by a simple twist-lock mechanism that mates the top of the battery to the bottom of the radio.

### Connect the Antenna

The antenna is connected to the radio through a Sub-Miniature series A (SMA) connector on top of the radio. Screw the antenna clockwise onto the connector.

### Radio Programming

Prior to operation for the first time, the radio should be programmed via the PC Programmer.  For help in programming a radio using the PC Programmer see the MA6941C PC Programmer's User Manual, Racal part number 84330.

### Radio Initialization.

Turn on the radio using the On/Off/Volume knob.  The radio performs a Power-On Self Test (POST) and gives a short medium-pitched tone to indicate PASS.  Use the switch to set the volume to a comfortable level.  Select the desired channel using the Channel Select Switch.  The radio is now ready for operation.

### Choose a Channel.

The default Display Screen shows the current zone and channel.  Use the Channel Select Switch to select a different channel in the zone.  To change zones either program one of the side keys or the three-position toggle switch to Zone Select, or program through the display using the "Select" menu.

### Transmit a Voice Message.

Press the PTT switch, hold the radio two to six inches from your mouth, and speak in a clear voice.

### Receive a Voice Message.

To enable the radio to receive a voice message, release the PTT.  Use the PC Programmer or the Radio "Program" menu (see Appendix A) to set/adjust the squelch level, Continuous Tone-Controlled Squelch System (CTCSS) tones, Digital Coded Squelch (DCS) variables, Network Access Codes (NACs), and/or Talk- Group Identifiers (TGIDs) as required.

# .  RADIO SET AND ACCESSORIES

## System Description.

The Racal 25 Radio set consists of the following components:

- Transceiver Module, which includes:

- Transceiver Circuit Card Assembly (CCA)

- Rear Cover

- Control/Crypto/Display Module which. includes:

| - Display | - Chassis, |
| --- | --- |
| - Control CCA | - Speaker/Mic |
| - Keypad CCA | - Cables |
| - Keypad | - Switches/Buttons |

- Antenna

- Battery

Available accessories include:

- Standard Antenna, 136-174 MegaHertz (MHz),

- Low-Band Narrowband Antenna, 136-152 MHz,

- Mid-Band Narrowband Antenna, 147-163 MHz,

- High-Band Narrowband Antenna, 158-174 MHz,

- Lithium-Ion (Li-ion) Rechargeable Battery,

- Alkaline Clamshell Battery Pack,

- Six-Unit Battery Charger, Alternating Current/ Direct Current (AC/DC),

- Single-Unit Battery Charger (AC),

- Six-Unit Battery Charger (AC),

- Surveillance Harness (wireless or wired options),

- MA6941C Personal Computer (PC) Programmer,

- Cloning Cable,

- Belt Clip,

- Nylon Carrying Case,

- Li-ion Leather Carrying Case,

- Alkaline Pack Leather Carrying case

- Side Connector Assembly

- Speaker/Microphone

- Digital Encryption Standard (DES) Keyfill Cable.

### Portable Radio.

The Racal 25 Portable is a handheld, battery operated transceiver capable of providing secure and non-secure communications over a 136-174 MHz Radio Frequency (RF) range.  The portable radio (Figure 1-1) includes a Liquid Crystal Display (LCD), Dual-Tone Multiple-Frequency (DTMF) keypad, emergency push button, speaker/microphone, multi-function side connector, three programmable function side keys, 16-position channel select rotary knob, On/Off/Volume rotary knob, three-position programmable toggle switch, antenna connector, battery connector and a Light Emitting Diode (LED) Indicator.  The Racal 25 Portable has an adjustable power output ranging from 0.1 up to 5 watts.  The Racal 25 portable radio operational modes include:

- Clear Analog Voice FM, 12.5 and 25 kilo-hertz (kHz)

- DES Continuously Variable Slope Delta (CVSD) Modulation Voice, 25 kHz, 12 kilobits per second (kbps)

- Project 25 Clear Digital Voice 12.5 kHz

- Project 25 Output Feedback (OFB) DES Digital Voice, 12.5 kHz

- DTMF Overdial (future incorporation)

- Project 25 Wide-pulse Operation (future incorporation)

### Antennas.

There are four antennas that can be used with the Racal 25 Radio.  The antenna are connected to the radio by a Sub-Miniature series A (SMA) connector.

#### Standard Broadband Antenna.

The Racal 25 standard antenna is a helical antenna designed to cover the entire 136-174 MHz frequency range, with a nominal impedance of 50 ohms.

#### Low-Band Narrowband Antenna.

A helical antenna designed for the 136-152 MHz frequency range, with a nominal impedance of 50 ohms.

#### Mid-Band Narrowband Antenna.

A helical antenna designed for the 147-163 MHz frequency range, with a nominal impedance of 50 ohms.

#### High-Band Narrowband Antenna.

A helical antenna designed for the 158-174 MHz frequency range, with a nominal impedance of 50 ohms.

### Battery.

#### Li-ion Rechargeable Battery

A rechargeable LiIon battery is the standard power source for the radio.  The battery is a self-contained unit connected to the radio by a reliable twist-lock mechanism.  The battery can hold a charge of 1,500 milliAmpere Hours (mAH) and at room temperature can provide over ten (10) hours of battery life at the five (5) Watt (W) transmit power level with an 90:5:5 Standby:Receive:Transmit (Stby:Rx:Tx) duty cycle.

#### Alkaline Clamshell Battery Pack

A reusable battery cassette that holds ten (10) non-rechargeable AA-size alkaline batteries is also available. At room temperature, the battery pack can provide over eight (8) hours of battery life at the 2W power level with an 80:10:10 duty cycle.  When the alkaline battery cassette is used, the radio limits the maximum RF power to the nominal power of the 2W setting.  This limit is indicated by a "2W" on the display.

### Battery Chargers.

There are three available chargers: a single-unit charger and a six-unit charger that operate from 90 to 265 Volts, Alternating Current (VAC), 50 to 400 Hertz (Hz); and a six-unit charger that operates on 10 to 32 Volts, Direct Current (VDC) power, in addition to the AC supply.  All of the battery chargers are capable of recharging the lithium ion battery to 95% charge in 1.3 hours and full charge in three (3) hours.  The chargers monitor charge current, temperature, and voltage to prevent improper charging.  Indicator LEDs on the chargers provide status as follows:

LED	Indication

Red	High temperature exceeded

Flashing Red	Over-discharged battery

Flashing Yellow	Under-temperature battery

Yellow	Normal charge

Green	Charge successfully completed

Flashing Red/Green	Charge error

Note:  The battery "fuel gauge" is not operable while the battery is charging.  It will refresh approximately 30 seconds after the radio is removed from the charger.

### Surveillance Harness.

The surveillance harness allows covert operation of the radio.  The harness operates via an inductively-coupled wireless earpiece, a microphone capable of being hidden in the wearer’s shirt or jacket, and a single button, Push-To-Talk (PTT) switch that can be strung through a shirt sleeve or coiled in a pocket.  When the radio is being used with a surveillance harness, the internal speaker is automatically disabled.

### MA6941C PC Programmer

The MA6941C Personal Computer (PC) Programmer is a Windows 95/98/NT-compatible software program capable of loading or modifying programming information into the radio from a PC.  It includes software, a detailed user's guide, and an RS-232 compatible cable.  The cable connects the PC serial port to the radio side connector.  See the MA6941C PC Programmer User's Manual (Racal p/n 84330) for a complete description of PC Programmer operation.  The PC Programmer is capable of programming the following parameters:

BANK

- Bank Tag

- Special Channels

- Priority Channels 1 & 2

- Emergency Channel

- Home Channel

- Zones/Available Zones

ZONE

- Zone Tag

- Scan List

- Channels/Available Channels

CHANNEL

- Channel Tag

- Channel Type

- Bandwidth

- Receive Only Option

- Options

- Scanlist

- Talkaround

- Locked

- Encryption

- Enable

- Key

- Transmit Power

- High and Low RF Power Levels

- Receive and Transmit Parameters

- Operating Frequencies

- P25 NAC (Digital)

- Talkgroup (Digital)

- Squelch Mode/Value (Analog)

- Shadow Channels

GLOBAL

- User (Configuration Name)

- User ID P25

- Keys and Switches

- Auxiliary Switch (1-3) Function

- Toggle Switch Function

- Emergency Button Function

- Programming Access

- Programming Enable

- Programming Password

- Scan

- Revert Mode

- Scan Delay

- Scan Reply

- Monitor Time

- Transmit

- Transmit Inhibit/Override

- Transmit Timeout

- Emergency

- Alert Mode

- Duration Timer

- Repeat Timer

KEY

- Key Tag

- Key ID

- Key Data

### Cloning Cable

Used to transfer programming information (excluding crypto keys and global parameters) from one radio to another radio. The cable connects to the radio side connector on both the sending (source) and receiving (target) radios.  Each cable end is labeled accordingly ("Source" and "Target"), for ease of use.

### Belt Clip

A mechanical means of attaching the radio to a belt.

### Nylon and Leather Bags

Harness type supports for the radio.

### Side Connector Assembly

An audio adapter assembly that allows several different audio accessories to be connected to the side connector.

## Controls, Indicators, and Connectors.

### Controls.

The radio controls (Figure 2-2), consist of a 16 channel rotary knob, an On/Off/Volume rotary knob, a three-position toggle switch, three programmable function side keys, an emergency push button, a PTT switch, and a 16 button keypad.

Figure 2-2.  Controls and Connectors.

#### On/Off/Volume Rotary Knob

The On/Off/Volume rotary knob located on the top of the radio, is a 16-position mechanical switch.  The first position is "Off", the second position is "On With Speaker Off" (mute), the remaining positions are used for increasing volume levels.

#### 16 Channel Rotary Knob.

The Channel Select knob, located on top of the radio, is used to rapidly switch between sixteen (16) pre-selected channels.

#### Three Position Toggle Switch.

The three-position toggle switch, located on top of the radio can be programmed using the PC Programmer for Zone Select, transmit Encryption enabled/disabled, Scan on/priority/off, High/Low Power, Talkaround on/ off, Monitor (squelch adjust) on/off, and Disabled.

#### Push-To-Talk (PTT) Switch.

The PTT switch is located on the left side of the radio.

#### Three Programmable-Function Side Keys.

Three programmable side keys are located on the left side of the radio, two above and one below the PTT switch.  These keys can be programmed using the PC Programmer for the following functions:

- Backlight dim/bright/off, (future use)

- Encryption on/off,

- Scan List add/delete,

- Keypad Disable, (future use)

- Monitor on/off,

- Scan on/priority/off,

- Signal Strength Meter on/ off,

- Talkaround on/off,

- Home Channel,

- Audible Tones on/off, (future use)

- Next Zone, (future use)

- Open Microphone, (future use)

- Previous Channel, (future use) and

- Disabled.

#### Emergency Push Button.

The Emergency push button can be programmed for Emergency operation or Key Zeroize operation.  When programmed for emergency operation, the Emergency push button is activated by the operator for emergency calling.  Once activated, the emergency condition remains active until it is cleared by a different means, such as turning off the radio.  When the emergency mode is activated, an emergency message is broadcast over the Emergency channel.  There are two programmable audio (full alert and silent) and display (alert and silent) modes.  When in Alert mode, EMERGENCY flashes on the radio display and an audio tone is sounded.  When in the Silent mode, there is no audio tone and no LED indication.  When programmed for Key Zeroize operation the emergency push-button can be used to instantly "panic zeroize" all encryption keys contained in the radio.

#### 16-Button DTMF Keypad.

The radio includes a 16-button conductive rubber keypad with positive feedback on the front panel.  The keypad provides adjustable backlighting for nighttime viewing.  The 16 keys are divided into two sections, as shown at Figure 2-3.  The function of each of the four keys on the first row is indicated on the bottom line of the display.  The remaining four rows of three keys each correspond to a standard 12 button DTMF (telephone) keypad.

Figure 2-3.  Sixteen-Button Keypad

### Indicators.

#### Liquid Crystal Display (LCD).

The radio contains a full graphics 80 x 32 pixel LCD that uses characters and graphics to provide the operator with radio operating information.  The display provides backlighting for nighttime operation.

#### LED Status Indicator.

Operating status is visually indicated by a three-color LED.  The LED is viewable from both the top and front of the radio and provides radio status as follows:

LED	Indication

RED	Transmitting.

GREEN	Receiving/Busy Channel indicator

Flashing GREEN	Receiving Encrypted transmission

ORANGE	Emergency/Low Battery

#### Audible Tones.

The radio has several audible tones that are activated by states of operation or by radio faults.  These tones are described in Appendix C.

### Connectors.

The radio connectors are shown in Figure 2-2.

#### Side Connector.

The side connector is an 18-pin connector located on the right side of the radio.  This connector is used for multiple functions, including PC Programming, keyfill, cloning, and audio accessory attachment.  The pin names and functions are delineated in Appendix D.

#### Antenna Connector.

The antennas screw into an SMA female connector located on the top of the radio.

#### Battery Connector.

The battery connects to a reliable, long-wearing, easily operated twist-lock connector mechanism located on the bottom of the radio.

## Transceiver Characteristics.

The radio frequency range is 136-174 MHz with channel spacing of 12.5 or 25 kHz, tunable in 5 kHz steps.

### Transmitter Characteristics.

#### Transmitter Output.

The transmitter output consists of a single channel Frequency Modulated (FM) carrier using either conventional 12.5 or 25 kHz FM modulation, or 12.5 kHz Compatible 4-level FM (C4FM).  The signal source is analog or digitized voice signals.

#### Transmit Squelch.

Transmit squelch parameters are required to enable selective squelch communications options.  These parameters are described below.

##### Analog Transmit Squelch.

There are three types of analog transmit squelch:

- None - No squelch will be included with the analog transmit signal.

- Continuous Tone-Controlled Squelch System (CTCSS) - Sub-audible CTCSS squelch tones will be included with the analog transmit signal.

- Digital Coded Squelch (DCS) - DCS variables will be superimposed on the analog transmit signal.

##### Digital Transmit Squelch

There are four types of digital transmit squelch:

- None - No squelch will be included with the digital transmit signal.

- Network Access Code (NAC) - A digital NAC will be transmitted with the Project 25 digital transmit signal.  This primary purpose of this code is to allow a user access to a repeater network.

- Talk Group Identifier (TGID) - A digital TGID will be transmitted with the Project 25 digital transmit signal.  This primary purpose of this selective digital calling identification is to group users into functional teams.

- Individual Call - TGID is automatically set to 0000 (hex) and the User ID of the targeted radio is activated within the Project 25 digital transmit signal.

### Receiver Characteristics.

#### Receiver Performance.

The receiver is capable of demodulating a single-channel FM carrier using either conventional 12.5 kHz FM, 25 kHz FM, C4FM, or Compatible Quadrature Phase Shift Keying (CQPSK) modulation.  The receiver demodulates analog or digital voice and data signals.  The radio circuitry can receive clear messages while operating in secure mode, and secure messages while in the clear mode if encryption is enabled.

#### Receive Squelch.

##### Analog Receive Squelch

There are three types of analog receive squelch:

- Carrier (Noise) - Squelch is opened on any intelligible analog signal.

- CTCSS – Squelch is opened on any analog signal that has the correct CTCSS tone.

- DCS - Squelch is opened on any analog signal that has the correct DCS variable.

##### Digital Receive Squelch

There are four types of digital receive squelch:

- Monitor - Squelch is opened on any intelligible digital signal.  The NAC and Talkgroup ID do not have to match.

- Normal - Squelch is opened on any digital signal that has the correct NAC.

- Selective - Squelch is opened on any digital signal that has the correct NAC and TGID.

- Individual Call - Squelch is opened on a digital signal that has a TGID of 0000 (hex) and has a User ID matching that of the receiving radio.

## Communication Security.

The radio is capable of secure communication by means of Type 3 software-based encryption, and is fully compatible with any radio using Project 25 DES encryption.  When the radio is operating in the secure mode, the transmission of all tone squelch signals is disabled.

### Algorithms.

The radio is capable of hosting two algorithms: Single-Bit Cipher Feedback (SBCF) DES (compatible with other manufacturers) 25 kHz channels and Project 25 OFB DES 12.5 kHz channels.

### Key-Fill.

Key-fill is accomplished through the radio side connector using the PC Programmer or a Motorola ® DES/DES-XL Key Variable Loader (KVL).  The PC Programming cable or DES Encryption Keyfill cable is used to load the keys.  The radio can store up to 16 cryptographic encryption keys.  The radio retains encryption keys until they are rewritten or zeroized.

### Zeroization.

The radio can immediately zeroize all encryption keys through the manual panic zeroize control.  Panic zeroization can be accomplished by pressing and holding the top key on the side panel and then pressing the emergency button.  The radio can also zeroize all encryption keys, or can selectively zeroize individual encryption keys, through the programming menu.  The emergency key can be programmed using the PC Programmer to panic zeroize all encryption keys.

# .  SOFTWARE THEORY OF OPERATION

## Functional System(s) Operation.

### Architecture.

A single Digital Signal Processor handles all signal processing functions.  An H8 microcontroller is used to control the man-machine Interface (MMI) and implement other radio control functions.  Functionality partitioning is shown at Figure 3-4.

### Module Identification.

The control module stores an electronic serial number and modification status within non-volatile storage on the module.

### Power-On Self Test (POST).

The software executes a POST automatically when the radio is switched on.  This test is capable of detecting and identifying faults that prevent the radio from operating.

Switches

LCD

Keypad

Keypad MPU

H8 Controller

DSP

Audio

Side

Conn.

Transceiver

PSU

Battery Pack

Ext. Power

RACAL 25

CONTROL MODULE

Battery Data

Power

Modulation

Control

IF Output

Control/Fill/Code

Keyfill

User Data

Figure 3-4.  Racal 25 Block Diagram

### Flash Software Upgrades.

The radio software can be updated in the radio using a PC and the PC Programming cable.  The update software (Updatexxx.exe) will either be sent via Email or in a zipped format on a 3.5" disk.  Updatexxx.exe is ready to use by double-clicking on the icon.  The zipped version must be extracted, using unzip software, prior to use.  Appendix E explains in detail how the upgrade is to be completed.  A Technical Service Bulletin with this information will be sent out with each upgrade software.

### Voice Coder/Decoder (VOCODER).

The VOCODER uses an Improved Multi-Band Excitation (IMBE) voice-coding algorithm as specified in Telecommunications Industry Association/Electronic Industries Alliance (TIA/EIA)-102.BABA.  The IMBE VOCODER compresses a high-bit-rate waveform into a low-bit-rate data stream suitable for transmission over the channel.  The VOCODER operates at a net bit rate of 4.4 kilobits per second (kbps) for voice information and a gross bit rate of 7.2 kbps after error control coding.

## Radio Control Software.

This software controls the transceiver and baseband signal processing functions.

### Audio Control.

The H8 controls the analog audio signal processing. Audio for transmission comes from either the internal microphone or an external microphone through the side connector, they are wired in parallel to the microphone amplifier which is permanently powered.

Audio output is required when a voice message is received or a tone is generated by the MMI. To allow audio output the H8 sets either SPKRON or EXTSPKRON to allow audio out to the internal speaker or the side connector respectively. If an external audio accessory is detected by Who Are You (WRU) < 0.5 VDC, the audio is routed to the side connector.  Otherwise audio is routed to the internal speaker.  The sixteen (16)-position volume control knob is decoded and sent to the DSP through the controller software to control the output audio volume.

### DSP Control Software.

The DSP implements most of the baseband signal processing in the radio.  Its function is controlled by the H8 controller through its host port.  The DSP operates in a number of basic modes, controlled by the H8 through the host port, these are:

- Idle	Current shut-down mode

released through the host port.

- Searching	Actively looking for a signal

on the IF input signal.

- Searching Paused	Search algorithm paused for

an economize cycle or frequency change.

- Active Receive	Actively receiving a

message, initiated by detecting a signal or H8 command

- Transmit	Actively transmitting voice

or data.

- Key Fill/	Keyfill operations and key

Management	management tasks.

The DSP pages in different program images from the Flash for different modes of operation.  Typically one image is used for receive and standby modes, but a new image is needed for transmit and for key management operations.  The DSP can interrupt the H8 controller, and then pass data over the host port back to H8.  Interrupts from the DSP include the following events:

- Signal detected, with type data.

- Signal lost

- DSP BIT errors

- Paging request

In all active modes the H8 software must be able to write a number of parameters to the DSP and also read back a number of parameters from the DSP. This is implemented through the host port. The parameters used include:

- Searching	Reference oscillator temperature, used

by DSP to correct frequency offsets. AGC reset control used at start of  search period.

Search modes:

Analog:  bandwidth, squelch tones, squelch code, and squelch level.

Digital:  data rate, key, and algorithm

Project 25:  NAC and TGID
CVSD:  data rate

- Receive	Reference oscillator temperature, used

by DSP to correct frequency offsets. Audio Volume.

Analog parameters: squelch controls, de-emphasis, and companding.

Digital parameters: data rate and key algorithm.

Project 25 parameters: NAC, TGID, BER, and Test mode.

CVSD:  data rate and key

Project 25 parameters read by H8: SS bits, low rate data (future), sender ID

- Transmit	Reference oscillator temperature, used

by DSP to correct frequency offsets.

Audio volume, sidetone on/off.

analog parameters: squelch controls and de-emphasis.

CVSD digital parameters: data rate and key

Project 25 parameters: NAC, TGID, key and low-rate data (future).

### Transceiver Module.

The transceiver module is controlled through a synchronous serial bus from the H8 to the transceiver that allows the H8 to control: the synthesizer; two 4-channel 8-bit DACs; and a control shift register in the transceiver module.  Some of the DAC channels are set according to data in the transceiver Electronically Erasable Read-Only Memory (EEPROM) calibration tables.

#### Transceiver Module Mode Control.

The transceiver module mode of operation: transmit, receive or standby is controlled by the transceiver shift register and the CTX output of the FPGA.  The outputs are controlled as below:

- Spare	(SR bit 1)

- 3.3VRXSynth	(SR bit 2) 	Set in active receive

mode RXVCO enable.

- 3.3VTXSynth (SR bit 3) 	Set in active transmit

mode, TXVCO enable.

- 3.3VRXEnable(SR bit 4)	Set in active receive

mode receiver enable

- Spare	(SR bit 5)

- Spare	(SR bit 6)

- TX/RX	(SR bit 7)	Set in active transmit

mode, front end Tx/Rx control

- STD/SIDE	(SR bit 8)	Set to use radio

antenna, reset to use side connector RF port

- CTX	(FPGA output)	Set in active transmit

mode to enable the RF power amplifier.

#### Transceiver Frequency Control.

The frequency of operation in both transmit and receive is controlled by the H8 setting the synthesizer through the serial bus.  To set the desired frequency: the appropriate Tx or Rx synthesizer enable S-R bit must be set; the serial data must be loaded into the synthesizer chip; and the DAC2 output A, synthesizer coarse tune,

must be set to the appropriate value for the frequency according to the EEPROM calibration table.  Synthesizer lock can be monitored by the OOL input. Once synthesizer lock is achieved the transmitter or receiver can be enabled with the appropriate control bits, 3.3VRX enable,  CTX and TX/RX.  Economizing the synthesizer function is implemented by controlling the 3.3VTXS/RXS bits and by controlling the EM main divider enable bit in the synthesizer control word. The synthesizer serial data need not be reloaded when coming out of economize if the frequency is unchanged.

#### 12-Bit DAC

DACLDA, DACADCCLK, and DACDOUT are used to control the 12-bit DAC for IFAGC, TXVCOMOD, REFOSCMOD, and VATT.

#### Transceiver Reference Oscillator Temperature Compensation.

The H8 software constantly monitors the reference oscillator crystal temperature using the XTALMON line.  The temperature data is then used to lookup a compensation factor in the transceiver EEPROM calibration table.  This compensation factor is then written into the DSP when it is added as a DC offset the reference oscillator modulation signal, and used as a DC offset to the same in receive modes.

#### Transceiver Receiver Control.

The linear receiver chain is enabled by setting 3.3VRXE.  The DSP implements a software AGC system to control the gain of the linear receiver chain.  The H8 controller can monitor the actual received signal level by reading RSSI.  At all times during receive the RXVTF DAC2 output C must be set to the value in the EEPROM calibration table corresponding to the receive frequency being used.  This sets the receiver front end tunable filter to be centered on the desired frequency.  At all times in receive modes the 2nd LO DAC2 line output B must be controlled using data from the EEPROM calibration table indexed with the reference oscillator temperature data XTALMON.  Temperature compensates the second LO in the receiver chain.

#### Transceiver Transmitter Control.

The radio uses a complex H8 software based algorithm to control the transmit power of the radio dynamically. The inputs to the power control algorithm are:

- The requested power level: 0.1, 0.5, 1, 2 or 5W

- PA calibration data in the EEPROM

- Supply voltage, BATMON - used for monitoring

- The transmit frequency

- PA current - used for monitoring

- PA temperature - used for monitoring

The power control algorithm takes these inputs and uses them to control the following outputs to provide a steady RF power output with a clean rise and fall at switch on/off.

- PWRSET	Sets the power  level in the

(DAC1 output A)	power amplifier ALC loop.

- PABIAS1	Adjusts the bias in the final

(DAC1 output B)	driver stage.

- PABIAS2	Adjusts the bias in the final

(DAC1 output C)	driver stage.

#### Transceiver Tx/Rx Switching.

The basic principle of events needed to quickly switch the transceiver module from receive to transmit and back again is to shut down the current mode, lock the synthesizer up in the new mode on the new frequency (it will always be different), then enable the transmitter or receiver as required.

#### Receiver Scanning.

In some scanning modes it is necessary for the radio to scan a number of channels looking for traffic, controlled by the H8 software.  The basic requirement is to change the synthesizer frequency, and RXVTF and Synthesizer Tune DACs, and resume searching on the new frequency.  The DSP may have to be informed of new traffic parameters to search for on each new frequency.  Scanning is interrupted when the DSP detects a signal of interest.

### DC Power Control.

The H8 controller software controls the power supply switching in the radio.  The control software algorithm uses the following inputs:

- /PWROFF	This input indicates the current

position of the radio on/off switch and the side connector off line.

- WRU	Indicates if the radio is fitted into a

harness providing external power

- /LBOUT	Indicates that the supply voltage at the

minimum that is required for correct operation.

- BATBUS	Battery serial bus indicates whether a

Smart battery is connected, and if so its charge state.

- BATMON	Indicates the voltage on the main

radio supply from battery or external power source.

These inputs are used to control the following FPGA outputs:

- PWRHOLD	Set during normal operation, used to

hold the radio on regardless of the on/off switch.  When /PWROFF indicates that a switch off is required, a clean software shut down is executed followed by release of the PWRHOLD output.

- BATOFF	This output can be reset to prevent

current flowing out into the battery from an external power source. This line is set when the battery indicates it is fully charged and an external power source is being used as the primary radio power.

Additionally the /LBOUT interrupt is used to execute a fast shutdown of the software when the supply voltage drops below that needed for normal operation or when the power source is removed without switching the radio off.

#### Power Supply Frequency Control.

The power supply control software controls the switch mode power supply frequency output according the RF frequency being used.  The frequency is checked and changed if necessary at every transceiver synthesizer frequency change.

#### Battery Interface

The battery interface software implements a single-wire bi-directional serial interface to the Benchmarq ® power gauge device in the smart battery pack.  The battery interface software provides an interface to read the battery voltage register and the compensated available capacity registers in the power gauge.  Data is read out of the battery at power up, then at least every minute, and within five seconds of every transmit exit.  The battery interface software includes timeouts to start again in case of errors, to cope with the situation where a charger may be accessing the battery at the same time as the radio, and a clash on the serial bus occurs.

### Monitoring

The H8 software monitors the following signals:

- Out-of-Lock	In all active modes, every 100 ms.

- EPTT/RTS	In all modes, every 20 ms.

- RSSI	In receive modes, every 100 ms.

- WRU	In all modes, every 1 second.

- Battery Volts	In all modes, every 5 seconds.

- PA Temp	In transmit modes, every 1 second.

- REF Temp	In all modes, every 5 seconds.

- PA Current	In transmit modes, every 1 second.

### Radio Control Drivers.

A number of low level software drivers are used by the H8 to interface to the transceiver hardware.

#### Audio/Power Supply Unit (PSU) Driver.

A serial interface driver controls the output bits of a serial to parallel output shift register in the FPGA.  Clock and data source for this shift register is the same serial port used for the MMI serial bus, but data is directed to the shift register using high-order H8 address lines.

#### Transceiver Serial Bus Driver.

A serial interface driver is used to control the transceiver shift register, DAC, and synthesizer.  This interface uses a common clock and data line, and three separate strobe lines for each device.

#### Digital Signal Processing (DSP) Host Driver.

The H8 software includes a DSP host driver for controlling the DSP mode of operation, and initial start-up code download.

#### BATBUS Driver.

The H8 software includes a driver to implement the smart battery serial bus.

#### IIC Bus Driver.

The H8 software includes a driver to allow the controller software to read and write to the transceiver EEPROM using IIC protocols.  The two lines are general purpose I/O lines controlled on a bit-by-bit basis by the software

## Digital Signal Processing.

The DSP software implements all baseband signal processing in the radio.  It processes signals between the user audio and data interface, and the transceiver modulation and Intermediate Frequency (IF) interfaces.  The signal processing provides compatible analog FM modes, Common Air Interface (CAI) compatible modes, and 12 kbps secure CVSD modes.

### DSP Transmit Chain.

Signal processing while the radio is transmitting depends on the radio's operational mode.  The possible modes are Clear Analog Voice FM, CVSD DES Voice, Project 25 Clear Digital Voice, and Project 25 DES Digital Voice.  The DSP Transmit Chain Block Diagram is shown at Figure 3-2.  The major signal processing functions of the DSP transmit chain are described in the following paragraphs.

#### Audio Coder/Decoder (CODEC).

The Racal 25 uses a Texas Instruments ® TLV320-AC36 Audio CODEC.  Data is transferred to and from the CODEC using the DSP Enhanced Synchronous Serial Interface (ESSI) 0 port.  The data word is sixteen (16) bits long.  The first thirteen bits are the two's compliment audio sample, the last three are the volume control word in receive direction (DIN) and are zero padded in the transmit direction (DOUT).  The DSP currently sets volume control bits for no attenuation and the volume is controlled by scaling the signal prior to sending it to the CODEC.  The sample rate from the CODEC is 8 kilo-samples per second (ksps).

#### Audio Processing Module.

The Audio Processing Module receives audio input from the Audio CODEC, applies filtering and Automatic Gain Control (AGC), then transmits it to the mode-specific formatting module.  The audio filter has a pass band from 300 Hz to 3 kHz.  This module also transmits DTMF tones to the Audio CODEC.  DTMF over-dial is supported to allow redirection through the phone network via a base station.  Data is transferred to and from the CODEC under Interrupt Service Routine (ISR) control.

#### Project 25 Voice Module.

The Project 25 Voice Module performs framing and conversion tasks.  The framing Function uses it's own task table to build a CAI TDMA frame.  This includes compression of the voice signal using the IMBE VOCODER, forward error correction, and encryption.  The physical layer task converts a 4.8 ksps dibit data stream into a 48 ksps real sampled waveform, which is then fed to the Modulation Module.  The physical layer scales each dibit symbol so that the proper frequency deviation is attained.  It also applies raised cosine filtering for control of inter-symbol interference.

CVSD DES Module

Audio

CODEC

Transceiver

Audio

Processing

Module

Modulation Module

Project 25 Voice Module

Analog FM Module

VOICE

DTMF

Tones

Figure 3-5.  DSP Transmit Chain Block Diagram

#### CVSD DES Module

Audio data from the Audio Processing Module is sent to the audio circular buffer.  The sample rate is then increased from 8 ksps to 12 ksps.  The CVSD encoder encodes the data and sends it to the transmit CVSD audio circular buffer.  The data is then DES-encrypted and differentially encoded before being sent to the physical interface buffer.  The CVSD physical layer converts the CVSD encoded, DES encrypted 12 ksps data stream into a 48 ksps waveform suitable for processing by the Modulation Module.  The module contains a Finite Impulse Response (FIR) raised cosine filter that acts as an interpolation filter.  The end of a transmission is signaled by transmitting an End of Message (EOM) indicator, which consists of 160 ms of alternating ones and zeros.  This allows the receiving radio to squelch the audio output before the radio stops transmitting.

#### Analog FM Module

Audio data entering the Analog FM Module is sent through a linear-phase, FIR, audio-shaping filter.  Interpolation from 8 ksps to 48 ksps is accomplished using a linear-phase, FIR filter.  A single-quadrant sine Look-Up Table (LUT), using fractional addressing and quadrant folding, is used to generate CTCSS tones.  If the DCS audio turn-off code is to be transmitted, the tone is fixed at 134.4 Hz and the codes are transmitted at a rate of 134.4 bits per second (bps), which is derived using the CTCSS tone generator.  The DCS data stream is passed through a raised cosine filter before being added to the speech.  The 8 ksps audio stream, with CTCSS/DCS controls, is interpolated to 48 ksps before being sent to the Modulation Module.

#### Modulation Module

Modulation Module prepares the signal for transmission.  The signal is split into a reference oscillator signal and a Voltage Controlled Oscillator (VCO) signal.  This allows independent scale and offset values for each signal.  A modulation balance variable scales the reference oscillator voltage, so that the maximum frequency deviation is constant for all RF channels.  A transmit modulation variable does the same for the VCO signal.

#### Transceiver Interface

The transceiver Digital to Analog Converter (DAC) has four output ports, two of which are used to modulate the carrier.  One of the two channels is used to maintain carrier frequency accuracy.  On transmit channel changes, the controller provides the DSP with two fractional values used to scale the two signals output from the DAC.  The controller provides the DSP with an additional integer value at one second intervals, that is added to one of the DAC output signals to control carrier frequency accuracy.  The modulation interface receives modulation data samples at 48 ksps, independent of transmit mode.  When the radio is operating as a transmitter, the transceiver interface controls the operation of the DAC via ESSI 1 on the DSP.  Data is written to the DAC at 96 ksps.

### DSP Receive Chain

The radio receive chain hardware consists of an RF transceiver board, Analog to Digital Converter (ADC), a Motorola ® DSP 56302 or DSP 56309, and an Audio CODEC.  The DSP Receive Chain Block Diagram is shown at Figure 3-3.  The major signal receive functions of the DSP receive chain are described in the following paragraphs

#### Transceiver

The RF transceiver board performs mixing and filtering of the received signal to produce a 455kHz, 25 kHz Bandwidth (BW), IF signal.  The output signal from the transceiver is digitized by the ADC and fed to the DSP.  The bulk of signal processing is performed by the DSP.  An ISR that implements the Transceiver/ ADC/DAC interface is called at a rate of 96 kHz in receive modes.  The ISR reads ADC output data and stores the values in a circular buffer and controls data transmission to the DAC.

#### FM Demodulator

The FM Demodulator converts the FM output of the transceiver to a real-valued, baseband signal.  The FM demodulation is implemented by a discriminator task.  Sub-sampling the 455 kHz IF at 96 kHz folds down the signal to 25 kHz.  A mixing function mixes the sampled IF data in the input buffer before filtering,  For 12.5 kHz channels, a second filter is applied to the IQ data stream.  Calculating the angular difference between consecutive IQ pairs demodulates the received signal.

##### Analog to Digital Converter.

The ADC sub-samples the 455 kHz first IF producing a frequency translation as part of the sampling process, since the signal BW is much less than the 455kHz carrier frequency.  The ADC sampling rate is 96 ksps.

##### 25 kHz Frequency Translation.

The 25 kHz frequency translation converts the signal image into a baseband signal, centered at 0 Hz.  DSP implements the digital equivalent of a mixer to perform frequency translation.

##### IF Filtering

The complex baseband signal is sent through two linear phase, FIR filters.  The first IF filter is implemented as a decimate by two, polyphase, FIR filter and is applied to the 96 ksps, complex, baseband output of the 25 kHz mixer.  This filter removes some of the out-of-band noise produced by the nonlinear analog components of the transceiver.  CVSD DES and Analog Wide modes have a 25 kHz BW and the first IF filter is the only filtering performed for these modes.  Project 25 and Analog Narrow modes have a 12.5 kHz BW, and the second IF filter provides the filtering required for these modes.  The second filter is applied to the 48 ksps output of the first IF filter.

#### Signal Detectors

The radio uses three signal detectors to detect the presence or absence of a modulated signal in the tuned channel.  These signal detectors search for Project 25, Analog FM (including noise, CTCSS, and DCS), and CVSD DES signals.

##### Project 25 Detector.

The Project 25 frame detector detects a Project 25 signal by searching for the Frame Synchronization (FS) signal and Network Identifier (NID)that are embedded in the preamble of every project 25 data unit.  The detector uses this information to perform bit recovery and packet identification.  It processes and buffers the binary data for use by the Project 25 Voice Module.  Once a target signal is detected, the radio disables squelch using an enable transmit function, so the signal can be monitored by the user.  If the detected signal is lost, squelch is enabled by the shutdown active receive function.

##### Analog FM Detector

The Analog FM Detector uses a function to decimate the incoming data stream by six, and run the noise detector, CTCSS single-tone detector, CTCSS multi-tone detector, and DCS multi-code detector.  A noise squelch detect function detects the appearance of a carrier by searching for a drop in power in a frequency band just above the audio band.  In the analog noise detector, the input data is scaled and high-pass filtered, then rectified and scaled again. Then the data is low-pass filtered.  The output of the low-pass filter is used to determine whether or not a signal is present.

CVSD DES

Module

Audio

CODEC

Transceiver

Audio Processing

Module

FM Demodulator

Project 25 Voice Module

Analog FM

Module

Signal

Detectors

Figure 3-6.  DSP Receive Chain Block Diagram

The detector has two states: SEARCHING and LOCKED.  If the detector state is SEARCHING and the detected power drops below the lower squelch threshold, the detector state transitions to LOCKED.  Conversely, if the detector state is LOCKED and the detected power rises above the upper squelch threshold, the detector state transitions to SEARCHING.  Upper and lower threshold values are BW dependant and can be adjusted at run time.

The multiple-value DCS detector searches for a 134.4 bps bit stream in the sub-audible frequency band used for DCS codes.  If found, the code is extracted and appropriate state variables are updated.  Code extraction is performed in two steps: first the input data is converted to a binary bit stream and second, data extraction, and code comparisons are performed.

The conversion of the input data to a binary bit stream starts with the 8 ksps input data being sent through a decimate by six, FIR filter to produce a 1.33 ksps, real valued data stream.  This filter removes any signal energy outside of the sub-audible frequency band.  The data is then split into two paths.  The lower path estimates the Direct Current (DC) content of the signal with a very narrow low pass Infinite Impulse Response (IIR) filter.  Subtracting the lower path signal from the upper path signal removes the DC component from the upper path signal.  Following this, the resulting signal passes through a single-bit quantizer and the output is buffered for use by the code removal step.  Data extraction, and DCS code comparisons are then accomplished.

##### CVSD DES Detection

Detection of CVSD DES waveforms is performed by a secure detection function.  This function also recovers the 12 kbps bit stream from the 48 ksps input signal.  The detector looks for a 12 kbps data stream to determine if a CVSD signal is being received.

#### Project 25 Voice Module

The physical layer task extracts FS, NID, SS symbols, and data.  All other dibits are passed to the receive framing task.  The module performs recovery and symbol extraction based on frame synchronization using a correlation detector.  Carrier frequency offset compensation is followed by symbol extraction and error-correction decoding.  VOCODER data and Project 25 framing data is removed and secure mode decryption is performed.  The VOCODER converts the compressed voice data stream to a 8 ksps audio data stream that is sent to the Modulation Module.

#### Analog FM Module

The analog FM module performs two tasks.  The Detection Task uses a FIR filter to down-sample the FM demodulated bit stream from 48 ksps to 8 ksps.  It then uses four detectors (noise, single-tone CTCSS, multi-tone CTCSS, and multi-code DCS) to determine signal squelch.  The Post-Detection Audio Shaping Task applies de-emphasis on/off filtering to a received clear analog signal.

#### CVSD DES Module

The CVSD DES Module consists of a 12 kbps clock detection/recovery task, a 12 kbps symbol resolver, a differential decoder, a DES decoder, a 12 kbps CVSD decoder, and a 12 ksps to 8 ksps sample rate converter.  The input to the clock detection algorithm is a 48 ksps data stream, representing the sampled FM demodulated carrier.  To allow for variation in carrier frequency, the DC component of the demodulated carrier is removed before zero-crossing detection.  From a zero-crossing phase profile, a decision can be made whether a 12 ksps data stream is present on the demodulated carrier and a 12 kHz clock can be recovered for usage within the 12 kbps symbol resolver.  Differential encoding of the binary FSK modulation is used to ensure compatibility between manufacturers, in that either a positive or negative frequency shift can be used to represent a "1" symbol.  In the CVSD receive chain, differential decoding precedes one-bit cipher feedback DES decoding.  To conserve memory and aid processing efficiency, all symbols (encoded and decoded) are packed in memory.  The 12 kbps CVSD decoder is modeled after FED-STD-1023.

The decoder consists of a Modulation Level Analyzer (MLA), a Syllabic Filter, a Pulse Modulator, a Principal Integrator and a Comparator.  The output of the CVSD decoder is at 12 kbps that must be changed to 8 ksps for output by the CODEC.

#### Audio Processing Module

A audio receive task function and a audio filter is used to output Project 25, clear analog, or secure analog speech samples to the CODEC.  When in active receive modes, the ISR is enabled and the task outputs data to the CODEC circular buffer as data is being written to it's audio input circular buffer.

#### Audio CODEC

The Audio CODEC and the DSP interface using the DSP ESSI 0 port.  The serial clock to the CODEC operates at 2.048 MHz, and is derived from the DSP internal clock.  ESSI 0 is configured to operate using a frame rate divider of sixteen (16) and a word length of sixteen (16) bits, transmitting packets of encoded audio to the CODEC at 8 kHz.  The CODEC is used in a linear decode mode, where thirteen (13) bits are used to represent the full audio range.  The post-processed DSP signal is fed to the Audio CODEC, which converts the signal to an analog waveform, applies gain, and routes it to the appropriate output device.

### DSP Software

The program data for the DSP is stored in 64K Flash program blocks.  The data is stored as unpacked bytes.  The blocks used for the DSP software are dedicated so that selective upgrades of this code only are possible.

## Keypad MPU Software.

### Overview.

The keypad Micro-Processor Unit (MPU) provides the direct interfaces to the radio keypad and switches. It communicates with the main controller via a synchronous bi-directional serial link.

### General.

The software is designed so that processor activity and hence current consumption is minimized.  The only continuous operation required is the keypad scanning and switch reading.  The keypad MPU is clocked by an external clock at 1.5 MHz, and the hardware reset supplied from the main controller.

### Keypad Scanning.

The keypad scan software continuously scans the keypad at a rate of a row every 10ms.  A debounce period of 40ms is used on key presses and key releases. The key scan software deals with simultaneous key presses and key rollovers, such that only single key presses are validated. Debounced and validated key presses are passed to the serial data output buffer.

### PTT Input.

The inputs from the momentary input switches, PTT and the Auxiliary keys are read every 10ms. and are debounced for 40ms.  The validation software filters out simultaneous presses of the auxiliary keys and key rollover between them.  Simultaneous presses of the PTT switch and one auxiliary key are allowed.  Debounced and validated auxiliary key presses and PTT press and releases are passed to the serial data output buffer.

### Switch Input.

The inputs from the rotary switches and toggle switch are read at least every 40ms.  Changes in state are debounced for 100ms.  Debounced new switch positions are then passed to the serial output buffer.

### LED Output.

The outputs to the LED are controlled under instruction from the serial port.  It is possible to set both outputs off, set the red LED on, set the green LED on, set both LEDs on (orange), and to flash either or both on a 50% duty cycle at a controlled rate of approximately 1 Hz.

### Backlight Control.

The two LCD backlight controls to set bright and dim operation are controlled under instruction from the serial port.  The keypad uses a fixed level backlight operation.  A timeout facility, to switch off the backlight after 30 seconds if not requested by the main controller, is provided.

### Serial Interface.

The keypad controller implements a synchronous bi-directional serial interface using its serial port that allows it to interface to the main controller.  The serial data clock is always sourced by the main controller.  To allow autonomous transfers from the keypad controller a separate Keypad interrupt line is provided with the interface.  The interrupt line is used to request 8 clocks from the main controller to transfer data from the keypad.  The LCD Chip Select (CS) input line is used to distinguish between serial data for the keypad controller and LCD driver.  The serial interface supports the following transfers:

- Keypad to Controller

- Keypad power up OK

- Keypad error 1-n

- Key press 1-16

- Key release 1-16

- PTT press

- PTT release

- Volume Switch 1-16

- Channel Switch 1-16

- Toggle switch 1-3

- Auxiliary key press 1-3

- Emergency key press

- Controller to Keypad

- Request current switch status

- Reset, execute BIT test

- Backlight off/bright/dim

- LED off/red/green/yellow/flash/flash rate

- Keypress request, interrupt acknowledge

LCD data transfers are in blocks of 80 bytes maximum, to allow a pause on the serial interface at least every 100ms for the keypad MPU to assert the interrupt and transfer keypress or switch change data. During the LCD data transfers, the key data are buffered in the keypad MPU.

## Data Interface.

The DSP incorporates a user data interface through its SCI port

### CAI Data Interface.

The DSP supports an asynchronous data interface for CAI modes using its SCI port.  This interface conforms to the CAI Data Peripheral Interface.  It uses standard V24, and RS232 baud rates up to 9600 baud.  The software also controls the associated flow control signals Data Terminal Ready (DTR) input to DSP and Clear to Send (CTS) output from DSP.  The Request To Send (RTS) input to the radio for this interface is processed by the H8.

### Synchronous Serial Data Interface.

The DSP supports a 12 kbps synchronous serial port using its SCI port. The interface is half duplex, uses a DSP generated clock, and includes minimum data buffering within the DSP and RTS/CTS flow control on the transmit function.

#### Receiver Synchronous Serial Data Buffering.

In receive synchronous serial data modes the DSP software uses a variable length First In First Out (FIFO) buffer to cope with differences in clock rates between the transmitter and receiver.

### CAI Data Link Layer.

This software provides the link between the raw voice or data bit streams and the data formats required to implement a 9.6 kbps CAI compatible interface.

#### CAI Transmit Voice Mode.

The DSP software takes the 144 bit voice code words (encrypted or not) and a number of link control fields set by the host H8 or from the DES system, and formats CAI-compatible logic link data units.

#### CAI Transmit Voice Test Modes.

The DSP software is capable of transmitting the CAI voice silence test pattern, the CAI 1 kHz test pattern, and a 9x144 bit (1296) PRBS test pattern used for error rate tests.  These test modes are controlled by the H8 controller.

#### CAI Receive Voice Mode.

The DSP takes the 9.6 kbps CAI-compatible data stream and framing, split out the voice data for passing to the VOCODER.  The DSP decodes the link control words so that the link control fields can be read by the host H8, and the encryption synchronization information is available to the encryption process.  The CAI receive processing is initiated by the frame synchronization correlator trigger.  When this is asserted the next 64 bits of Network Identifier (NID) data are decoded and checked.  If the NAC code matches the one selected for the channel, voice or data processing proceeds, otherwise the physical layer will be forced back into search mode.

#### CAI Receive Voice Test Mode.

The DSP software is capable of testing the 9 voice code words received in a CAI frame against a known 144 x 9 PRBS segment.  The total number of errors in that frame is then output to the H8 controller.  This test mode is controlled by the H8 controller.

#### CAI Transmit Data Mode.

The DSP selects the  user data (encrypted or clear) and a number of link control fields set by the host H8 or from the DES system, and formats logic link data units, compatible with the CAI at 9600 bits/second.

#### CAI Receive Data Mode.

The DSP selects the 9.6 kbps CAI-compatible data stream and framing, splits out and decodes the data for passing to the user data port.  The DSP decodes the link control words so that the link control fields can be read by the host H8, and encryption synchronization information is available to the encryption process.

### Transmit Physical Link Layer.

This software uses common modulator interface software and a number of mode dependent physical link layer software modules.

#### Transmit Modulation Interface.

The software provides a common interface to the dual modulation Digital to Analog Converters (DACs) in the transceiver through its SSI serial port 1 for all transmit modes.  In transmit modes, the SSI uses an externally sourced clock at 3072 kHz.  This interface takes frequency deviation samples at 48 kHz, and writes each value scaled by a fixed number set by the host to both the reference oscillator DAC and the VCO DAC.  Additionally a host-controlled DC offset is added to the reference oscillator DAC value.

#### Transmit CAI Physical Link Layer.

This software takes the 9.6 kbps CAI-compatible data stream and converts it to 48 k samples of frequency deviation data.  To achieve this the software implements the dibit to symbol mapping and Nyquist and shaping filters as described in the CAI.

#### Transmit Analog FM Physical Link Layer.

This software takes the 8 ksps filtered audio and converts it to 48 ksps of frequency deviation data compatible with TIA/EIA-603 in 12.5kHz and 25kHz modes. The signal processing used is additional high pass filtering to reduce the energy in the DCS tone band, audio band pre-emphasis if required, DCS tone addition, deviation limiting and smoothing.  Software is capable of operation in 12.5 kHz, and 25 kHz channel spacing with appropriate deviation scaling. The software is capable of: appending a phase reversed tone burst of 180 ms as defined in EIA-603, generating DCS and audio turn-off codes, and companding the voice signal in 12.5kHz mode.

#### Transmit CVSD Physical Link Layer.

The transmit CVSD physical link layer converts the 12/16 kbps CVSD data stream and converts it to 48ksps of frequency deviation data. The software implements a pre-modulation filter with raised cosine time response and 100% eye height.

### Receive Physical Link Layer.

This software uses common FM demodulation software, and mode dependent receiver physical layer software modules.

#### Receive ADC and DAC Interface.

In receive modes the SSI port 1 is used in a duplex manner to allow the ADC to be read continuously at 96 kHz and the DAC to be written to at up to 48 ksps for AGC and reference oscillator adjustment.  In receive modes the SSI port is clocked at 1536 kHz from an external clock source, using a sixteen bit cycle.  Every cycle a value is read out of the ADC.  Every other cycle a value may be written to one of the DAC channels to control the AGC and reference oscillator.

#### FM Demodulator.

This software provides a common interface to the 96 ksps IF signal at the SSI port 1 and produce 48 ksps of frequency deviation data for use in all modes.  The SSI port uses an external clock at 1536 kHz.  The software implements an FM demodulator function using a quadrature mix with a 24 kHz (Fs/4) local oscillator, dual I and Q channel filters and a frequency estimator.  The channel filtering function is programmable dependent on the channel spacing being used.  The channel filtering provides the adjacent channel filtering additional to that provided by the hardware needed to achieve the radio adjacent channel rejection performance.

#### Receive CAI Physical Link Layer.

This software takes the 48 ksps of frequency deviation data and outputs a 9.6 kbps data stream. The software implements an integrate and dump filter and data slicer as described in the CAI.  The integrate and dump filter is controlled by a clock recovery function which selects one of ten possible phases for output to the slicer.  The slicer incorporates an averager with a time constant of at least 100 bits to correct for DC offsets in the received signal.

In parallel with the above, a FIR correlator searching for the CAI fixed framing sequence of 24 symbols operates on the filtered 48 ksps of frequency deviation data.  The correlator therefore operates at 10 samples per symbol.  The correlator phase that has the highest correlation peak is used to select the clock phase for use in the integrate and dump filter, and slicer.  This correlator operates continuously when searching for CAI traffic and occasionally when tracking an CAI signal when subsequent frame syncs are expected.  The correlator trigger is used to provide a framing signal for the subsequent CAI data link layer processing.

#### Receive Analog FM Physical Link Layer.

This software takes the 48 ksps frequency deviation data and outputs 8 ksps of audio to the receive audio processing.  The software signal processing implements a high pass filter to remove CTCSS tones and de-emphasis if required.  The gain of the signal path is adjusted to cope with the different deviations used on different channel bandwidths.  The signal processing signal path is controlled by squelch signals.  The software includes audio expanding to reverse the transmit companding.

#### Receive CVSD Physical Link Layer.

This software takes the 48 ksps frequency deviation data and output 12/16 kbps serial data. The software implements a data filter, a slicer and a clock recovery function.

### DES Encryption.

The DSP software implements DES encryption of traffic in CAI modes and CVSD modes.

#### DES Kernel.

The DSP software implements the DES encryption kernel as described in FIPS 46-2, encrypting 64 data bits using a 56 bit key using output feedback operation or single bit cipher feedback operation.

#### CAI Encryption.

The DSP software uses the DES kernel software to implement the CAI encryption of voice traffic as described in TIA/EIA/IS-102.AAAA.  The encryption key is supplied by the key manager.  In transmit, the Message Indicator (MI) vector is passed to the data link processing for encoding and transmission.  In receive, the MI vector is decoded by the data link layer, and flywheeled if decoding fails for up to n frames TBD.

#### DES Data Link Layer.

This software encrypts and decrypts the 12 kilobits of CVSD data using the DES kernel.  In transmit, framing synchronization data and the MI vector are inserted into the data stream.  In receive, the software searches for and extracts the framing data and MI data.  The bit definitions and formats used are defined in the DES protocol.

#### Key Interface.

This software provides an interface for inputting DES encryption keys from the DSP SCI port using synchronous data transfers with an external clock, conforming to the Motorola KVL data transfer mechanism and the CAI DES keyfill protocol.

#### Key Bank.

The radio maintains a bank of up to 16 encryption keys stored in the flash memory.  Associated with each key are a Key ID, Key Data, and an eight character alpha-numeric tag.  Each encrypted channel is assigned one of the 16 keys for both secure transmit and secure receive modes.  Channel key assignment is accomplished via selecting the corresponding key tag.  Upon entering secure transmit or secure receive mode, the H8 transfers the appropriate encryption key to the DSP through the SCI port.

### Host Interface.

The DSP is controlled through its host interface by the H8.  It initially boots up through this interface.  The host interface is used for DSP mode control, encryption key transfer, link control data transfer, low rate data transfer, frequency variable data transfer, CTCSS mode control, and initial software download.

### Flash Interface.

The DSP also has direct access to the main radio Flash memory through the H8 bus arbitration logic.  This interface is used for software downloads using byte-wide Direct Memory Access (DMA) transfers under host control for mode changes.  The DSP software does not write to the Flash memory.

### Paging.

The DSP software is designed such that normal operation does not involve off-chip bus accesses.  This means that the program code size must be limited to 24k words and the data memory to 10k words.  A number of program images corresponding to different modes are allowed, with paging of images out of the Flash by DMA at mode changes allowed.  The minimum subdivision of images used corresponds to the following modes plus a continuously resident core host interface function.

- Initialization/POST

- Receive 12.5 kHz

- Receive 25 kHz

- Transmit 12.5 kHz

- Transmit 25 kHz

- Keyfill

The paging DMA mechanism is controlled by the host H8 and allows the transfer of a program image within 50ms.

### Hardware Control.

The DSP software controls the DSP clock rate through the Phased Locked Loop (PLL) output divider such that the DSP clock rate is dynamically matched to the mode of operation, in coarse steps for example, between searching and tracking receive modes.  The DSP software is designed to use low current wait modes in pauses between processing to minimize current consumption.  The host is also able to request a very low current idle mode in the DSP, this mode is released by the Host.

## Controller Software.

### Overview.

The controller software has overall control of the radio, including Man-Machine Interface (MMI) operations, and control of the DSP and transceiver.  Figure 3-4 shows a block diagram of the controller software.

### Environment.

#### General.

The H8 controller system is designed for minimum external bus activity and minimum current consumption.  These features are provided by the maximum use of low current standby modes in the H8, and an interrupt driven architecture, with the minimum of input polling.  In radio standby and receive modes the only H8 tasks are the control of the transceiver frequency and DSP mode as the radio scans, and economizes. The most H8 intensive activities are associated with MMI interactions with the user, and operations on the fill/program/control port.

#### Scheduler.

The H8 operates with a simple scheduler that launches tasks after interrupt events.  A timebase interrupt of 10ms is used to keep track of time and poll inputs at regular intervals.

##### Interrupt Sources.

The following interrupt sources are used in the H8:

- MMI Interrupt	External interrupt from keypad

- DSP Interrupt	External interrupt from the

DSP, signal detected etc.

- Low Battery	External NMI from power

supply indicates power supply fail

- Timer Interrupt	Internal timebase tick interrupt

every 10ms

- PC Serial Port 	Internal interrupt from PC

Interrupt serial port

Figure 3-7.  Controller Software Block Diagram

Radio Store

MMI Software

MMI Drivers (AVR)

Alarms

Switches &  Keys

LCD

Keys

Fill Data

Radio State

Radio Control Software

Audio

BIT Monitoring

EEPROM

DSP

PSU Control

Battery

Power/Mode Control

Frequency Control

Transceiver Driver

Audio/PSU Driver

DSP Host Driver

IIC Bus Driver

BATBUS Driver

Fill/Program/Control Software

PC Serial Port Driver

Environment Software

Main Controller/ Scheduler

Debug Serial Driver

FPGA Configure Driver

Software Update Driver

MMI serial port	Internal interrupt from MMI

Interrupt serial port, keypress data and LCD data

- Debug serial port	Internal interrupt from DMA/

Timer

- Alarm generator	Internal interrupt from DMA/

Timer

##### Polled Inputs.

The following inputs are polled by the H8 software at regular intervals.

- EXTPTT/RTS	External PTT and data RTS

- OOL	Synthesizer Out of Lock

- PWROFF	The on/off switch position

- SENSE	External keyfill device detect

ADC as regularly as appropriate to the radio mode of The following analog inputs are measured with the operation.

- RSSI	Receiver signal strength

- WRU	External Device Detect

- BATT	Main radio 10V supply monitor

- PA TEMP	Transmitter temperature

- XTAL TEMP	Reference crystal temperature

- PA CURRENT	Transmitter Current

##### Watchdog.

A regular watchdog service task is scheduled to prevent the H8 watchdog controller overrunning and a hardware reset occurring. The target watchdog timeout is 100/ 200ms.

#### Start-Up Software.

##### Boot Block Start-Up Software.

The minimum simplest start-up software is provided in the boot block of the Flash.  This software holds the keypad, DSP, MMI and Field Programmable Gate Array (FPGA) in reset, then establish if a valid H8 program image exists in the program blocks of the Flash.  If no valid program exists a simple alarm is sounded. T he boot block software includes a minimum basic BIT facility to check the code itself and internal and external Random Access Memory (RAM).  The boot block code also includes the software to allow programming of the program blocks of the Flash through the PC serial port.

##### Full Start-Up Software.

If a valid H8 program image exists the full start-up code is executed.  This involves: initializing RAM, DSP, keypad, MMI, FPGA, transceiver etc.; executing the start-up BIT; and then transferring control to the main scheduler.

##### FPGA Configure Software.

At start-up the H8 configures the FPGA using data from the main Flash memory and transferring it via a synchronous serial bus to the FPGA.

#### Shut Down Software.

At normal shut down when the on/off switch is switched off the H8 software executes a clean shut down to shut down the transceiver, DSP and MMI, save any usage data to the Flash, then release the main power supply.  In cases where the battery is removed the Low battery interrupt is used to execute a minimum fast shut down, just saving RAM data as required, with no Flash update.

#### Debug Driver Software.

The H8 controller includes software to implement an asynchronous serial port on two Input/Output (I/O) pins of the H8. Facilities provided include the ability to monitor particular radio variables, and to control specific variables. This port is used to allow PC serial access to the board at board level factory test, through the test connector.

### Radio Store.

The radio data store is accessed by all functional areas of the controller software, including the MMI, fill control and radio software.

#### Physical Data Storage.

The radio data is physically stored in 4 devices all accessible by the controller software.

##### H8 Internal RAM.

The H8 controller has 2k x 8 of internal RAM. This data is not retained when the radio is switched off or the battery is removed. It is used for short term storage of  frequently accessed variables, stack workspace etc. to minimize the bus activity when the H8 controller is running.  The internal RAM is used as program space to execute from during some Flash update operations.

##### External RAM.

The H8 is provided with an external 128k x 8 bit RAM that is backed up for at least 30 sec when the radio battery is removed, and also at all times when a battery is fitted even when the radio is switched off.  This device is used as a variable data expansion area, and to store specific user-entered data which must be retained over battery changes (the unlock password etc.)

##### Flash ROM.

The H8 is provided with 512k x 16 bit Flash Read-Only Memory (ROM) that is primarily used for program storage. The data in the Flash is retained permanently.

Different areas of the Flash have different characteristics:  The boot sector is a 16k block that is used for the reprogramming software and the radio serial number etc. The data in this sector is designed to be programmed or block erased only in the factory.

The Flash has fifteen 64k and six 8k program blocks that can be block erased and programmed by the boot block code during normal reprogramming operations without special equipment and without opening the radio. These blocks are use for H8 and DSP operating software, radio fill data, and FPGA programming data.  The Flash has two small 8k parameter blocks that can be used for changing data that must be stored indefinitely, examples are user specific settings and usage data.

##### Transceiver EEPROM.

The radio transceiver has an 8k x 8 serial EEPROM that is used to store transceiver calibration data. This data is set during production test for the specific transceiver. The data in this device is essentially constant and it is never written to by the main controller. At switch-on the contents of the EEPROM are copied into the external RAM, and the EEPROM is not accessed during normal operation.

#### Data Types.

A number of different data types as below are used by the controller software.

##### Volatile Variables.

Variable data used by the H8 controller that is not retained when the radio is switched off is stored in the H8 internal RAM and the external RAM.

##### Short Term Stored Variables.

Variable data that is retained while the radio is switched off or the battery is removed is stored in the external RAM.

##### Permanent Stored Variables.

Variable data that is retained indefinitely is stored in the parameter blocks of the Flash. Every time this data changes a parameter block must be erased and the new data written into the now blank parameter block.

##### Radio Fill Data.

The frequencies, modes, power levels, etc. associated with different channels programmed into the radio.  This data is programmed into the radio through the fill port

### Program/Fill/Control Interface.

The controller provides a serial port for PC access to allow the following functions:

- Programming – updating radio software.

- Filling – Modifying the radio data store of modes and frequencies etc.

- Control – Controlling the radio operating mode.

#### Radio Programming.

The controller software allows reprogramming of the Flash memory program blocks, on a block by block basis.  During these operations, the controller executes from the boot sector of the Flash and no radio or MMI operations are possible.  After a programming operation, the radio must be restarted by cycling the power. Programming operations are initiated on receipt of a specific serial message on the PC serial port.

#### Radio Fill.

The controller provides the facilities to modify the system, group, bank, and radio global data through the PC serial port.  This interface provides the following facilities:

- Radio erase - delete all fill data in the radio

- Selective erase - delete (mark as deleted) specific systems, groups , banks (future)

- Radio Fill - add specific systems, groups, banks and global data to the radio store.

- Radio read - export the fill data contents of the radio store to the PC.

All transfers and operations on the fill port are CRC checked and acknowledged.

##### Cloning.

The radio can export channel data to other radios.  The data export is initiated by a MMI operation at the exporting radio.  The exporting radio emulates a PC programmer during the data transfer.  Cloning of keyfill data is not allowed.

##### Radio Keyfill.

The radio keyfill protocols are managed by the DSP software.

#### Radio Control Port.

It is possible for an external PC to control the functioning of the radio.  This provides the following facilities:

- Radio status read - export the radio serial number, revision status, history, usage etc.

- Set external control mode.

- Set radio transmit frequency, power level, and mode.

- Set radio receive frequency and mode.

The transceiver mode control also allows transmission of 1kHz test tones in analog modes, and BER test patterns in digital modes.  The receiver mode control allows the continuous (every 0.5 second) output of the bit error count per frame in digital modes.

#### PC Serial Port Driver.

The controller software controls the H8 serial port to implement the asynchronous data formats and baud rates (9,600; 19,200; and 38,400) for the PC serial port.  The serial port driver software also controls the 232OFF signal to maintain the RS232 in its low current standby state, except when data is being driven out and while RTS is asserted.

## MMI.

The radio MMI is described in the following paragraphs.

### Display Description.

The radio has a 80 x 32 dot matrix LCD radio display.  Some of the features of the display are:

- Phone mode is indicated by a icon in the top right. (future)

- Scan occurring is indicated by ‘SCAN****’, ‘SRCH****’, or ‘ZONE****’ flashing on and off in the top row of the display.

- Encryption is indicated by a key icon.

- Power level is indicated by ‘HI/LO/2W’ in the bottom row of the display.

- Priority scan is indicated by ‘SCANP1(P2), ‘SRCHP1(P2)’, or ‘ZONEP1(P2)’ flashing on and off in the top row of the display.

- Receive Only Channel is indicated by an ‘RX’ in the bottom left corner of the display.

- Talkaround is indicated by an ‘TA’ in the bottom left corner of the display.

- Repeater Mode is indicated by a receiver icon in the bottom left corner of the display.

- An ‘EMG’ appears in the bottom left corner when the radio is transmitting an emergency message.

- A battery level bargraph display is permanently displayed on the far right of the display.

### Keypad

The radio can be programmed using the keypad and display.  Pressing the ENTER key from an operational screen will allow the user to enter the programming menu screens.  These screens are shown at Appendix A.  For detailed instructions on programming the radio through the display and keypad see the Racal 25 User's Guide (p/n 84326).

# .  HARDWARE THEORY OF OPERATION

## Introduction.

The radio contains three Circuit Card Assemblies (CCAs).  The Control CCA and Keypad CCA are contained in the Chassis Assembly, and the Transceiver CCA which is attached to the rear panel.  The CCAs are attached to each other and to other radio components through 32-pin, wire, and flexi connectors.  Figure 4-8.  Racal 25 Interconnection Diagram contains the Racal 25 Interconnection diagram.

Control Board.

Top Antenna

2100388-501

3500384-501

MIC

18-Pin

Side

Conn.

Volm

SW

Emg

SW

Togg

SW

Battery

Transceiver

CCA

4101257-501

Control

CCA

4101258-501

LCD CCA

1600482-1

Aux3

SW

PTT

SW

Aux2

SW

Aux1

SW

4400416-1

Keypad CCA

3100618-501

P

T

T

SPKR

Side

Antenna

2100388-501

2600133-1

3100628-1

4400384-1

J3

J2

P1

J3

J1

J1

P3

P1

P1

P2

3300153-1

3300151-1

3600178-1

3400516-1

3600178-1

BAT-

DATA

BAT+

P2

4101160-501

3300155-1

2600133-2

J1

The control board fits interfaces to the keypad board, transceiver board, internal audio, and side connector.  The board implements the main radio control function and all the baseband signal processing. Figure 4-9 shows a block diagram of the Control Printed Circuit Board (PCB).  The Control CCA schematic (drawing number 4200676) is contained in the schematic section at the back of this document

Figure 4-8.  Racal 25 Interconnection Diagram

ADC

RS232

RS232

FPGA

Clock

Generator

3V Audio

5V Audio

3.3V Logic

3.3V Cont

Reset

10VTx

10VCont

10VSW

5V Logic

LB Out

Power Off

LBOut

Ext PWR

AUD On

ON Sw

CTX

Rad Off

HOLD

Batt Off

PSU CLK

Watchdog

BATTERY INTERFACE

TCVR

KEYPAD BOARD INTERFACE

SIDE CONN.

INTERFACE

CODEC

SQU

EXT PWR

EPTT/RTS

ADC/DAC CLK

Keyfill/User Data

ADC/DAC CLK

12.288 MHz

TCXO

3V Cont

Batt Bus                      BATT    GND

MOD

OOL

IF

WRU     10V Cont

Current, Temp, RSSI

Syn/SR/DAC Bus

IIC

10V Cont

10V SW

ON SW

CS

Vpp

PSU CLK

Keypad CLK

PSU

512kx16

FLASH

128kx8

RAM

INT                       CLK

CS

SCI

SCI

DATA

ADDRESS

H8

Debug

Watchdog

DAC

ADC

INT

Host        Data  Add

CLK

SCI           RESET

DSP

SSI                           SSI

OnCE/JTAG

FPGA

Output Expand

Sense

PC Control

Figure 4-9.  Control Board Block Diagram

### Power Supply Unit.

This block of circuitry takes the raw battery supply voltage and the external power source from the side connector together with a number of control signals to generate a number of power supply outputs.  Figure 4-10 shows a block diagram of the Power Supply Unit (PSU).

#### Battery/External Power Switching.

This block of circuitry generates the 10V CONT supply from either the battery or external power source.  In normal operation with a battery, the FPGA sets /BATOFF to connect the battery through to 10V CONT (at power-up the reverse path diode operates before BATOFF is set).  In this mode of operation, power from the battery for external ancillaries is available when the radio is switched on through 500mA current limiter.  In operation without a battery but with an external power source, power is routed from the external power source through to the 10V CONT line. This input is protected by an overvoltage detector, which switches off this path if the external voltage exceeds 16V.  In the situation where there is an external power source and a battery fitted, the H8 software detects the situation by sensing a battery through the battery serial bus and an external power source through the WRU line. In this situation the software then controls /BATOFF to prevent the external power source damaging the battery.

#### On/Off Switching.

The main continuous supply 10V CONT is passed through an on-off switch to generate 10V SW, the main radio supply.  In normal operation with a battery the on/off switching is controlled by the radio on/off rotary switch by the control /RADON.  Once switched on the main controller can hold the radio on by setting PWRHOLD.  In addition to the radio rotary on/off switch, the on/off switching can be controlled by the external line /RADOFF.  This line overrides the /RADON line and can be used to force the radio off regardless of the rotary switch setting.  However, the PWRHOLD and PWROFF lines can be used by the H8 controller to implement a clean controlled switch off.

#### Transmit Power Switching.

A single FET switch controlled by CTX is used to provide a switched 10VTX high current (1.5 Ampere) supply for the transmitter.

#### 4.5V Switch Mode Power Supply.

This circuit uses a switch mode power supply device to generate a 4.5 Volt (V) supply at 450 milliAmperes (mA) maximum for the radio logic.  The switch mode device is synchronous and uses an externally provided power supply clock at 384kHz.  The power supply circuit includes input and output filters to limit the conduction of the fundamental switching components and their harmonics in the VHF band both onwards into the logic and back into the battery.

#### 3.3V Linear Logic Supply.

The output from the 4.5V switch mode power supply is passed through 3.3V linear power supplies to remove any remaining power supply switching noise on the main logic supply.  One 3.3V supply is used for control board logic, the other 3.3V, 50 mA supply is used for control board analog circuitry.

#### Reset Generator.

This circuit uses a MPU supervisory device (MAX825) to generate a reset pulse of at least 140ms whenever the 3.3V logic supply drops below 3.08 volts.  The circuit also generates a reset signal when the Watchdog input is asserted by the H8 or DSP.

#### 5V Linear Logic Supply.

This circuit generates a 5V logic supply at 50mA maximum for use on the control board.

#### 3V Backup Supply.

This circuit generates a 3.3V logic supply at 50mA maximum to power the RAM at all times when a power source is fitted regardless of the on/off switch setting.

#### Low Voltage Detector.

This circuit controls the LBOUT output so that it changes state when the main supply drops below about 6V. This signal is used by the H8 controller to implement a clean shut down when the radio battery (or external power) is removed without switching off the radio.

#### Audio Supplies.

Two linear regulators providing clean filtered supplies for the audio at 5V are provided.  The 5V 800mA audio supply is used by the audio power amplifiers. The audio supply is used for the low current microphone amplifier.  A linear 5V bias supply to the internal microphone is also provided.

TRANSCEIVER

CTX

Battoff

EXT PWR

10V CONT

5V LOGIC

3.3V ANALOG

5V AUDIO

LB OUT

10V SW

10V TX

BATT

Over-Voltage

Detect

Current

Limit

On/Off

Logic

On/Off

On/Off

Power Off

Watchdog

PSU Clock

PwrHold

/RADOFF

/RADON

5V

Linear

PSU

5V

Linear

PSU

5V

Linear

PSU

Low

Voltage

Detector

4.5V

SMPSU

3V3 CONT

3.6V

Linear

PSU

RESET

3.3V LOGIC

Reset

Gen

3.3V

Linear

PSU

3.3 V

Linear

PSU

5V MICBIAS

Figure 4-10.  PSU Block Diagram

#### H8 Microcontroller.

This is the main controller for the radio, the device used is a HD6433044 ROM-less microcontroller.  The H8 is configured with an expanded bus connected to the Flash, RAM, and DSP. The H8 integral bus arbitration logic is used to allow the H8 and DSP to both have access to the Flash and RAM.  The H8 is clocked by the external TCXO.

The H8 is powered from the 3.3V logic supply, and reset by the hardware /RES line. A watchdog output to the hardware reset circuitry is provided.  The H8 generates chip select outputs to allow the Flash, RAM, FPGA and DSP host port to be separately addressed.

One serial port of the H8 is used to implement a bi-directional synchronous serial interface to the keypad board. This interface is used to communicate with the keypad MPU and directly load the LCD controller. The clock on this interface is sourced by the H8 at c. 100kHz. An associated interrupt input to the H8 is used to initiate transfers from the keypad to the H8. This serial interface is also used to load the output expander in the FPGA, and also to configure the FPGA.  One serial port of the H8 is used to implement a bi-directional asynchronous serial interface to an external PC used for programming, fill and radio control. This interface uses programmable standard baud rates (default 9600 baud) and standard data formats. There are no handshaking parallel lines associated with this interface.  Two parallel I/O lines on the H8 are used to generate an I2C interface to allow the EEPROM on the transceiver board to be accessed.  Four parallel output lines on the H8 are used to generate a synchronous serial output bus with clock and data and separate strobe lines for the DAC, control shift register and synthesizer on the transceiver board.  One parallel I/O line on the H8 is used to generate the bi-directional Pulse Width Modulation (PWM) serial bus used to interface to the smart battery.  One H8 DAC output is used to generate simple audio tones of varying volume for use as audio alerts. The second H8 DAC output is available for VCTCXOP control.  The six channel ADC is used to measure: the raw supply voltage; WRU radio input; reference crystal temperature; PA temperature; PA current; and RSSI.  Two I/O lines are used to implement a software UART used for debug outputs in the development environment.

#### H8 Input Requirements.

The total requirement for parallel input signals to the H8 which need to be polled on a regular basis are:

- OOL	Out-of-Lock (OOL) signal from

the transceiver synthesizer

- EXT PTT	External PTT

- PWROFF	On/off switch position

- CONFDONE	Configuration status of FPGA

#### H8 Output Requirements

The total requirement for parallel output signals from the H8 which need to be controlled are as below, the output capabilities of the H8 are expanded by a serial load output latch in the FPGA.

- LCDA0	LCD controller A0 command/

data select

- LCDCS	LCD chip select

- SCL	I2C & synthesizer clock

- DACSDA	Transceiver serial data,

synthesizer, DAC, S-R.

- SYNTHENA	Synthesizer Framing pulse

- DACENA	DAC Framing pulse

- SRENA	S-R Framing pulse

- /DINT	Interrupt to DSP from H8

- /RESO	Watchdog Output from H8

#### H8 Input/Output Requirements.

The total requirement for parallel input/output signals on the H8 which need to be read and controlled are:

- BATBUS	PWM input/out to smart battery

- SDA	I2C data

#### H8 Input Interrupt Requirements.

The total requirement for parallel input interrupt signals on the H8 are:

- LBOUT	Low battery output

- DSPINT	DSP Interrupt

### DSP.

The DSP56309 (or DSP56302) processor implements all baseband signal processing functions in the radio.  It interfaces with the transceiver through one ESSI port, to the user for voice through the second ESSI port.  The DSP function is controlled by the H8 through the DSP host port.  The DSP has direct access to the main Flash memory through the bus arbitration logic in the H8, this allows it to download program images.  The initial power-on code download is through the host port.  The DSP is reset by the hardware reset line.  The DSP is clocked by the TCXO clock output line at 12.288 MHz.  The DSP ESSI 1 port is used to provide a synchronous interface to the IF ADC and the transceiver 12-bit DAC.  In receive modes that interface is capable of writing to the DAC at 48 ksps while still reading the ADC at 96 ksps.  The DSP ESSI 0 port is used to provide a full duplex synchronous interface to the audio CODEC using 8 kHz sampling rate and 13 bit samples.  The data transfer is at 2.048 MHz using a DSP sourced clock and framing pulse.

### Flash ROM.

A 512k x16 Flash ROM is used as the main program store for the H8 controller and DSP.  The Flash ROM uses a protected boot sector that is factory programmed via the DSP JTAG port.  Normal reprogramming is implemented by running the H8 from the boot sector and using 3V, programming the bulk of the device.  The Flash is used to provide a parameter storage area for non-volatile data storage of frequencies and keys etc.  This storage area is capable of in excess of 100k write cycles.

### RAM.

A 128k x 8 static RAM is used for temporary storage of data by the H8 controller.  This RAM is powered by a continuous supply that maintains its contents as long as a power source is present.  Additionally the RAM has a backup capacitor to retain its contents over battery changes.

### TCXO.

This oscillator serves as the reference for all logic and power supply clocks within the control and keypad module.  It provides the data rate clocks for radio operation, and is the source of the ADC/DAC/CODEC conversion clocks. The TCXO is at 12.288 MHz, with a temperature tolerance of +2.5 parts per million (ppm).  Additional calibration is performed to provide a typical temperature tolerance of ± 1.0 ppm  A trimmer to set the initial frequency is provided.  The TCXO sine wave output is squared up by a Schmitt trigger buffer before being output to the H8 and DSP.

### FPGA.

The control board uses an Altera 8282 FPGA device to provide a flexible serial data routing function, I/O expansion for the H8 and DSP, clock generation, data multiplexing and to absorb discrete logic functions.

The synchronous serial bus routing function involves routing the synchronous serial port of the H8 either to the keypad and LCD or to the I/O expansion in the FPGA.  This routing is controlled by high order address pins from the H8 and a FPGA dummy write with dedicated FPGA chip select from the H8. The FPGA includes a serial load parallel output shift register that is used for parallel output expansion for the H8.

The discrete logic functions of the FPGA, includes logic to control an inverter for one of the LCD control outputs.  The data multiplexing function involves re-routing serial pins between the side connector and the DSP SCI port to allow data transmission and keyfill operations.

#### FPGA Configuration.

The FPGA is configured at start-up from the main Flash memory using a serial load from the H8.  During configuration outputs are tri-stated and pulled to a safe level by committing resistors to prevent audio and RF bursts at power up.

### IF ADC.

The IF sampling ADC is a 12 bit ADC capable of sub-sampling a 455kHz IF signal at 96kHz sampling rate.  It is connected by a serial interface to the DSP ESSI port 1  The serial data interface is clocked at 1536kHz.

### Clock Generation.

The clock generation logic is used to generate clocks for the synchronous power supplies, the ADC serial interface and the keypad microcontroller.  The clock generator logic is implemented in the FPGA.

### Logic Audio CODEC.

The Audio CODEC is an integrated ADC/DAC and audio filter device capable of full duplex operation on voice bandwidth signals at 8ksps.  The samples have a resolution of 13 bits linear.  The CODEC is connected to the DSP ESSI port 0 with serial data transfer rate of 2.048 MHz.  The CODEC is continuously powered from the 3V logic and 3V audio supplies.  Parallel control lines from the DSP is used to mute the input and output sections as required.

### Side Connector Interface & Filtering.

The side connector interface is implemented as a 20-pin ZIF flexi connector, 2 pins being redundant, yielding 18 contacts.  All outputs are filtered to limit their bandwidth to the minimum and current limited to protect them from output short circuits to ground or up to 16V.  All inputs are filtered and protected from continuous application of ground or +16V.  There is no protection against negative applied voltages.  Inputs and outputs are protected from static discharge of at least 10kV air discharge.  All inputs and outputs incorporate RF bypass filter capacitors adjacent to the connector, except for ground.

### Transceiver Interface & Filtering.

The transceiver interface is implemented as a 32-pin interboard connector.  Most of the signals crossing the interface are filtered to limit their bandwidth to the minimum consistent with correct operation.  Outputs from the control module are filtered with series resistors on the control module and grounded capacitors on the transceiver module adjacent to the connector.  Outputs from the transceiver to the control use the reverse configuration.

### Battery Interface & Filtering.

The battery interface is implemented as a three-way flexi-circuit, which is fixed by two of the control board mounting screws.  The positive power and smart battery connections incorporate RF bypass capacitors as close to the signal pins as possible mounted on the flexi circuit.  Additionally, a series ferrite capable of carrying 2A peak is mounted on the flexi close to the main power pin.

### Keypad Interface.

The keypad interface uses a 18-pin interboard connector.  The connector type is a 1.27mm pitch shrouded connector.  Some filtering is provided on this interface, but all outputs are protected from short circuits by series resistors. Where possible, inputs are also protected from damage by series resistors.

### Audio Interface.

The audio interface is implemented with a four-way wire connector directly to the integral speaker and microphone.

### Test Interface.

The control board is provided with a 18-pin 1.27mm shrouded connector used for factory test.  It provides the following functions:

- Joint Test Action group (JTAG) connector access for board test and Flash boot sector programming.

- Board reset and control access.

- Board power supply and on-off switching access.

- H8 serial debug port access.

### LED

The control board incorporates a three-color LED used for status information.  It is controlled by the FPGA to show red, green, or off.  It is optically coupled to the top face of the radio by a light pipe.  The hardware is configured such that during hardware reset before the keypad MPU software is running the LEDs are off.

### Power Consumption.

The control board operates at input voltage from 8V to 13V.  Application of voltages above 16V may damage the unit.  The mean current consumption of the control board is:

Standby mode:	       60 mA

Receive mode @ 500 mW:	     330 mA

Transmit mode @ 2W:	  1,000 mA

Transmit mode @ 5W:	  1,500 mA

## Keypad Board.

The keypad board is fixed directly into the chassis behind the keypad.  The board incorporates the keypad electronics, the LCD interface, and backlights.  The board interfaces with the main controller via a bi-directional synchronous serial interface.  Figure 4-11 shows a block diagram of the keypad board. The Keypad CCA schematic (drawing number 4200649) is contained in the schematic section at the back of this document.

### Keypad Microcontroller.

The keypad board is controlled by a Atmel AVR4414 microcontroller.  This is a flash Programmable device.  The microcontroller implements these functions:

- Keypad scanning

- PTT switch input

- Auxiliary keys input

- Volume switch input

- Channel switch input

- Emergency switch input

- Synchronous bi-directional serial interface to main controller at 100 kHz

- Keypad and LCD backlight control

The keypad microcontroller is clocked by a clock derived from the main reference clock at 1.5 MHz.  The keypad microcontroller is powered by the same 10V supply as the LCD driver device.

### LCD Power Supply.

The keypad board uses a locally generated 3.3V logic supply to power the AVR and LCD driver.  This supply is a low current (<50mA) linear regulator operating off the main radio switched 10V supply.

### Display

The radio display module is a full graphics 80x32 pixels LCD, requiring a temperature compensated differential driving voltage of about 12V and a 1/6 bias, 1/32 duty cycle driving scheme.  It is driven by a LCD driver device (Seiko Epson SED 1530).  This LCD driver has internal display RAM and copes with all the display refreshing autonomously.  Display data transfers from the main controller are only required when the display is changed, selective display RAM updates are also available to minimize serial traffic.  The LCD driver display RAM is accessed through a one way synchronous serial interface, connected in parallel with the AVR serial port.  The LCD CS input is used to differentiate between serial data for the LCD driver and

Channel Switch

Vol/On Switch

Emergncy Key

Toggle Switch

Auxiliary Keys

PTT Switch

GND  Batt    ON     10V                      Dat     Dat     Dat    INT   AVR Kypd LCD LCD

CLK       In     Out              CLK     CS   CS     A0     Reset

PTT    Aux    Tog    Emg   Vol     Chn

5

5

3

3

3       2               4      4

4x4 Keypad

AVR 4414

Microcontroller

5V

Dim

Bright

Keypad Board

4

4

16 way Flex

SPI

LED Backlights

PSU

4

6 way Flex

LCD Driver

SED1560

Control Board

LCD

32 x 80

Dot Matrix

ON Decode

On

Vol

The display driver is reset by the keypad board hardware reset line. The hardware is configured such that during hardware reset before serial LCD data is presented the LCD is blank.

Figure 4-11.  Keypad Board Block Diagram

### Keypad.

The radio keypad consists of 16 conductive rubber keys, which contact onto switch contacts on the rear of the keypad board.  Each key is provided with an LED backlight.

### Backlighting.

Simultaneous backlighting of both the LCD and keypad is provided.  The backlighting is controlled by the AVR so that it can be turned off or set to one of two brightness levels.  Brightness control is by series resistor switching, not by duty cycle control.  The hardware is configured such that during hardware reset before the keypad MPU software is running, the backlights are off.

### Switch Interface.

An interface to the top face switches and the side mounted PTT and Auxiliary switches is provided.  This interface is implemented using surface mount ZIF flexi connectors on the keypad board.  The connections to the side switches and PTT are by a separate flexi connector.

- Top SW Interface

- Switch common (earth)

- Volume switch outputs (4)

- Channel switch outputs (4)

- Toggle switch outputs (2)

- Emergency switch output

- Side SW Interface

- AUX keys (3)

- PTT

- Switch common

### Control Board Interface.

A 18-pin interboard connector is used to implement the interface to the main control board.  The signals on this interface need not be  filtered, but are protected from short circuits to ground.  All logic signals are at 0-3V Complimentary Metal-Oxide Semiconductor (CMOS) levels at the interface.  The interface carries the following functions:

- 10VCONT

- Ground

- /RADON

- Switched 10V supply

- Reset

- AVR Clock

- Serial Data Clock

- Serial Data Input

- Serial Data Output

- Key Interrupt

- LCD Chip Select

- LCD A0 (Data\command select)

- Keypad Chip Select

### Power Consumption.

The total mean power consumption of the Keypad board in normal operation with the LCD on but the backlight and indicator off, is less than 4mA from the switched 10V supply.

## Transceiver.

The Transceiver CCA is fixed directly onto the radio rear cover.  Board components are packed in acoustic insulation that improve audio quality by dampening internal vibrations/harmonics from the radio speaker.  The board is also covered by an RF shield.  The Transceiver CCA schematic (drawing number 4200675) is contained in the schematic section at the back of this document

### Interface Section.

The Interface Section schematic is page 1 of the Transceiver CCA schematic enclosed in the back of this document.  J1 is the 32-pin surface-mount transceiver interface connector.  The transceiver is powered, controlled, and monitored through this port.  The pin names and functions are delineated at Appendix D.

J2 is the transceiver “TOP RF” connector.  RF signals are transmitted and received through this surface mount RF connector.  Transmitted and received RF signals are routed from this connector through a short RF cable to the antenna of the radio.  J3 is the transceiver “SIDE RF” connector.  RF signals are transmitted and received through this surface mount connector.  Transmitted and received RF signals are routed from this connector through a short RF cable to the side RF connector of the radio.  The side connector is used when the radio is used in a vehicular adapter.  J4 is the “455 kHz IF OUT MONITOR” connector.  The 455 kHz IF is available for monitoring purposes through this surface mount RF connector when the transceiver is in the receive mode of operation.  It is intended for test and troubleshooting purposes only.

### Transmitter Section.

The Transmitter Section schematic is page 4 of the Transceiver CCA schematic enclosed in the back of this document.

#### Transmit Chain.

The pre-driver amplifier (Q7, etc.) amplifies the “TXLO” signal from the synthesizer section.  The Q7 output power is typically 13 decibel milliwatt (dBm) measured at C77/R44/ R49 node.

R44, R49, and R52 is a 3 decibel (dB) pie attenuator network.  The PIN attenuator circuit is made up of CR5, CR6, and associated components.  This circuit yields more than 50 dB of useful attenuation range.  The circuit is part of a DAC controlled closed loop system, in conjunction with the detector/power control circuit (U12, U13, and associated components), which controls the transmitter output power level.  The Power Amplifier (PA) driver amplifier (Q5, Q8, etc.) utilizes an NPN transistor stage biased class A.  Q5 provides an active bias to Q8.  The collector current of Q8 is 100 mA typically.  The Q8 output power is typically 24 dBm measured at C76/C80/T3-1 node.

The PA (Q6, Q9, etc.) uses a pair of MOSFET power transistors configured as a push-pull power amplifier.  The power transistor gates are biased via DAC output lines “PABIAS1” and “PABIAS2” through 10 K ohm resistors R42 and R43.  Each transistor is biased for 100 mA of quiescent current. The PA full output power is typically 38.5 dBm measured at the T4-5 C209 node.

The PA temperature sensor (U11) is a thermistor Integrated Circuit (IC) fitted in close proximity to the power amplifier circuit.  Under normal operating conditions, the line labeled “PATEMP” is closely monitored by the microprocessor.  In the unlikely event of overheating, the transmitter would be disabled to prevent damage to the power transistors.

The harmonic filter (C209, L30, C81, C82, L31, C83, and C84) attenuates harmonics created by the power amplifier. The harmonic filter insertion loss is 0.4 dB typically at 174 MHz.  The output of the harmonic filter connects to the 20 dB coupler (U12).  This coupler is part of a DAC-controlled closed loop system designed to set the transmitter output power level.  The insertion loss through the coupler (U12-1 to U12-3) is 0.2 dB typically.

#### Power Amplifier Control.

Transmit output power level is controlled by the Detector/Power Control circuit and the PIN Attenuator (discussed previously) circuit via “VCONTROL.”  This closed loop system is designed to keep the transmitter output power constant over variations in temperature, transmitter supply voltage (“10VT”), and RF power levels into the transmit chain.  The detector/power control circuit is made up of the 20 dB coupler (U12), an RF rectifier circuit (CR11, CR12, etc.), and an integrator (U13).  The DAC line labeled “PWRSET” at the non-inverting input of U13 sets the transmitter to the desired power level.  U12-2 “samples” the transmit signal.  The sampled RF signal is rectified by the temperature stable circuitry of CR11,CR12, etc, and is routed to the inverting input of the integrator at U13-4. The output of the integrator at U13-1, labeled “VCONTROL”, controls the PIN attenuator circuit.  Any change in transmitter output power level is automatically corrected by the loop.

#### PIN Diode Switch.

The antenna PIN diode switch is made up of CR7, CR8, CR9, CR10, and other associated components. This switch is a four-port design.  The four ports are Antenna 1 (“TOP RF”), Antenna 2 (“SIDE RF”), receive, and transmit.  Receive and transmit ports can be switched to only one of the two antenna ports.  Transmit signals are routed from the transmit/receive PIN diode switch (to be discussed in the following paragraph) to the selected antenna port.  The receive signal is routed from the selected antenna port to the transmit/receive PIN diode switch.  The antenna PIN diode switch and receiver circuits “share” current in the receive mode of operation via the signal labeled “RXSINK” at Q11 pin 3.  This approach conserves battery power.   The insertion loss through the antenna pin switch is 0.2 dB typically.

The transmit/receive PIN diode switch is made up of C127, CR13, C114, L42, C115, CR14, C116 and other associated components.  C127, C114, L42, and C115 is the 1/4 wave simulator circuit.  The 1/4 wave simulator is critical to the design of the switch.  In the transmit mode of operation CR13 and CR14 are forward biased.  C116 resonates with the internal series inductance of CR14 at 155 MHz and the receive port (“RX INPUT”) is RF shorted to ground.  With the receive port RF shorted to ground, the parallel combination of C127, C114, and L42 forms a tank circuit resonating at 155 MHz.  Consequently, the receive port appears as an open circuit to the transmit signal and is routed to the antenna PIN diode switch.  In the receive mode of operation,  CR13 and CR14 are biased off so C114/L42/C115 appears as a Low-Pass Filter (LPF) to signals at the antenna port of the switch.  The insertion loss through the transmit/receive PIN diode switch is 0.4 dB in the transmit mode and 0.2 dB in the receive mode typically.

Q10-Q19 and associated components are switching transistors used to control the antenna and transmit/ receive PIN diode switches.  The current flowing through the entire PIN diode circuit is approximately 45 mA in the transmit mode of operation.  In the receive mode of operation the transmit/receive PIN diode switch is disabled, and approximately 85 mA flows through the antenna PIN diode switch.

### Receiver Section.

The Receiver Section schematic is page 2 of the Transceiver CCA schematic enclosed in the back of this document.  The VHF signal enters into the “RX INPUT” via the PIN diode switch (discussed previously).  CR1 and CR2 are schottky protection diodes to protect the front end circuitry from RF overloads that could occur if the PIN diode switch failed to work properly or if a transmitter is very close to a receiver.  Typical insertion loss is 0.1 dB for the protection diodes.  L25/C61 form a Band-Stop Filter (BSF) at the first IF frequency of 45 MHz.  Typical insertion loss for the BSF is 15 dB at 45 MHz but less than 0.1 dB in the VHF band.

L14, L6, CR3, CR4, L7, CR27, CR28, L8, and L15 make up the Very High Frequency  (VHF) preselector Band-Pass Filter (BPF).  The BPF is inductively coupled for improved high side attenuation.  This filter provides attenuation to spurious signals such as the first image and the half-IF.  The BPF is varactor diode tuned by DAC line “RXVTF." Typical insertion loss (138-174 MHz)  is 1 dB for the VHF BPF.

The RF Amplifier (Q1, T1, etc.) utilizes “loss-less feedback” to deliver reasonable gain, low noise figure and a high third order intercept point simultaneously.  Typical gain (136-174 MHz) is 11.5 dB for the RF Amplifier.

C14, L1, C9, C15, L2, C10, C16, L3, C11, C17, and L9 form a VHF LPF.  This filter provides additional RX spurious attenuation as well as image noise attenuation.  L4, C12, L16, C25, L5, and C13 form a BSF at the first IF frequency of 45 MHz.  The insertion loss is 1.0 - 2.0 dB (136-174 MHz) typically for the cascade.  The IF BSF  insertion loss is typically 40 dB at 45 MHz but less than 0.3 dB in the VHF band.

U1 is a double-balanced mixer (DBM).  U1 converts the desired RF signal down to the first IF of 45 MHz.  High-side Local Oscillator (LO) injection is used.  Therefore, the LO is 45 MHz higher than the receiver tuned frequency.  The LO drive level is +10 dBm nominal at U1, pin 1. The conversion loss of the mixer (RF to IF) is 5.5 dB typically.

The LO signal is generated in the synthesizer section (to be discussed later).  The LO signal is designated “RXLO” on the schematic diagram.  The LO signal is routed to a LPF consisting of C31, L21, C87, C30, L20, C75, and C28.  L19 and C28 are also used to impedance match the LO port of the mixer.  The insertion loss of the VHF LO LPF is 0.3 dB typically at 174 MHz.

R4, L17, C6, L10, R5, and C23 make up the diplexer network.  This network properly terminates the DBM both in and out of band.  The diplexer also provides some additional half-IF spurious rejection.  The diplexer  insertion loss is 0.8 dB typically at 45 MHz.

There are two 45 MHz IF Amplifier circuits.  The first (Q2, T2, etc.) utilizes “loss-less feedback” to deliver reasonable gain, low noise figure and a high third order intercept point simultaneously.  The second 45 MHz IF Amplifier will be discussed later.  Typical gain is 10.5 dB for the first IF Amplifier.

There are two crystal BPFs and a second 45 MHz IF Amplifier.  The BPFs provide attenuation for the adjacent and alternate channels, and also for the second image response.  FL1 is a four-pole crystal filter with a 20 kHz bandwidth centered at 45 MHz.  FL4 is a two-pole crystal filter with a 30 kHz bandwidth centered at 45 MHz.  The second 45 MHz IF Amplifier provides high gain to prevent further degradation of receive sensitivity.  C57, L12, C18, C19, and L13 are impedance matching elements for the input of FL1.  The output of FL1 is impedance matched to the second 45 MHz IF Amplifier (Q36, etc.) by C285, L64, and C242.

The output of the second 45 MHz IF Amplifier is impedance matched to FL4 by C287, L61, C286, C237, L60, and C235.  The entire cascade provides  21 dB of gain and has a 3 dB bandwidth of 20 kHz typically.  Typical insertion loss is 1.5 dB for each crystal BPF.

The IF IC (U28) contains the second mixer and an IF amplifier chain.  The 45 MHz IF signal enters U28 at pin 6 from the crystal BPF (FL4).  FL4 is impedance matched to the IF IC input by C294, L62, and C288.  The incoming 45 MHz IF signal is mixed with the second LO (to be discussed later).  The second mixer IF output is at U28 pin 8 and the second IF frequency is 455 kHz.

The signal from U28-8 is routed to FL3.  FL3 is a ceramic BPF operating at 455 kHz.  The insertion loss of the ceramic BPF is 6 dB typically in a 1500 ohm system.

The 455 kHz IF signal enters U28-10, is amplified by a cascade of  IF amplifiers, and exits at U28-14.  The signal from U28-14 then enters a second ceramic BPF,  FL2.  FL2 is the final ceramic BPF with a typical insertion loss of 8 dB in a 1000 ohm system.  The 455 kHz IF signal enters its final stage of amplification at U29-3.  U29 is configured as a non-inverting operational amplifier and is capable of driving a 50 ohm load.  The amplifier is set for a voltage gain of  2.5.  With the receiver set to full gain the signal level at J4 (455 kHz IF OUT MONITOR) is -20 dBm  3 dB into 50 ohms with a -119 dBm unmodulated 136 MHz signal injected at J2 (TOP RF) or J3 (SIDE RF).  The second LO consists of CR26, R33, Y1, L24, C59, C226, C227, and Q35.  The oscillator is a Colpitts type with the crystal operating in the series mode.  CR24 is a varactor diode used to set the oscillator on frequency using the DAC output labeled “2ndLO."  The second LO operating frequency is 44.545 MHz (low side injection).  L65 and C223 impedance match the output of Q35 to the LPF (C289, C293, L63, and C284).  The signal is then attenuated by R282, R283, and R284 and sent on to the second mixer.  The signal level at U28-4 is -16 dBm nominal.

U4 is a voltage regulator used to power the receiver circuits.  The DC voltage appearing at U4 pin 1, labeled “RXSINK” on the schematic diagram, is routed from the antenna PIN diode switch.  As previously discussed, the PIN diode switch and the receiver circuits “share” current to reduce receive power consumption.  The control line “+3.3V RXEN” is used to enable the regulator while the transceiver is in the receive mode of operation.  The regulator is disabled during the transmit mode of operation.

### Synthesizer Section.

The Transceiver  Section schematic is page 3 of the Transceiver CCA schematic enclosed in the back of this document.

#### Synthesizer and Reference Oscillator.

U19 is a fractional-N synthesizer IC programmed  for a specific frequency by loading appropriate serial data into the IC.  It controls the receive VCO when the transceiver is in the receive mode of operation, and the transmit VCO when in the transmit mode.  The programming lines are labeled “3VSCL”, “3VSDA” and “3VSYNTHENA” on the schematic diagram.  These are all CMOS logic level inputs.  R118 (RF) and R123 (RN) are the fractional compensation and phase detector current setting resistors, respectively.  These resistors are critical to the operation of the synthesizer system and must be checked when troubleshooting around U19.  The phase detector output pins (U19-13 and U19-14) are fed to the passive loop filter (R140, C177, C172, R134, and C173) and then on to the VCO control varactor diodes (CR17/CR19) for frequency control.  The buffered, filtered output from the VCO is fed into U19-5 (RF IN) to close the phase locked loop.  The level is typically -10 dBm into U19-5.  The reference oscillator is made up of CR22, Y2, Q28, C197, and C198 and associated components.  The reference oscillator operates at 12.8 MHz.  The reference oscillator operating frequency is adjusted by varying the DC voltage at the DAC controlled line that is labeled “REFOSCMOD.”  This line is also used to modulate the reference oscillator during the transceiver’s transmit mode of operation.  The 12.8 MHz signal is fed into the synthesizer chip at U19-8 (REF IN) using a coupling capacitor, C194.  The AC signal level at U19-8 is 1 Volt, peak to peak (Vp-p) typically.

U22 is the reference oscillator temperature sensor used to monitor the temperature near Y2.  It’s output is labeled “XTALTEMP” on the schematic diagram.  This line is normally monitored by the microprocessor so the reference oscillator can be adjusted for drift due to changes in temperature.

#### Receive/Transmit VCOs and Buffer Amplifiers.

The receive VCO operates from 181-219 MHz since high side LO injection is used and the first IF is 45 MHz.  The transmit VCO operates from 136-174 MHz.  Each VCO is a Colpitts type design utilizing a low noise, bipolar transistor as the active device.  The receive VCO uses Q24 and the transmit VCO uses Q21, each in the common collector configuration.  The Colpitts capacitors are C169/C180 (receive VCO), and C137/C142 (transmit VCO).   These capacitors enable Q24 to oscillate in the

181-219 MHz frequency range and Q21 to oscillate in the 136-174 MHz frequency range.  L53 is the resonating inductor for the receive VCO and L45/L46 are the resonating inductors for the transmit VCO. CR20/CR21, and CR16/CR18 are the coarse tuning varactor diodes for the receive and transmit VCO respectively.  These diodes are used to coarse tune the VCO such that the LPF, phase detector output voltage (from U19) at TP10 equals 1.65 VDC. The receive and transmit VCOs share the coarse tuning  DAC controlled line labeled “CTUNE”.  Coarse tune DC voltage swings from approximately 1.8-22 VDC.  CR19 (receive VCO) and CR17 (transmit VCO) are the fine tuning varactor diodes controlled by U19 as was explained previously.  CR15 is the modulation varactor diode for the transmit VCO.  The output from the receive VCO is coupled off Q24-E using C174.  The output from the transmit VCO is coupled off Q21-E using C139.  The signal should be measured at the C174/R146 node (receive) and the C139/R107 node (transmit), and measures -15 dBm typically.

Q26 and Q22, and associated components, form the first VCO receive and transmit buffer amplifiers respectively.  These amplifiers buffer the VCO output from changing-output Voltage Standing Wave Ratios (VSWRs) that could “pull” the VCO off frequency.  The output from each measures -5 dBm typically.  The buffer should be measured at the C170/R141 node (receive) and the C138/R131 node (transmit).Q25 is the second buffer amplifier.  This amplifier is common to both the receive and the transmit VCOs.  R131, R141, and R142 are the combining elements used to make this possible.  This buffer outputs a signal large enough, after subsequent attenuation and filtering, to properly drive the RF IN pin of the synthesizer (U19-5 ).  The output from this buffer should be measured at the C167/C53/L52/C175 node and measures 0 dBm typically.

The output from the Q25 buffer is filtered by C175, C53, L52, and C176.  This LPF prevents the synthesizer IC (U19) from “locking on” to harmonics of the desired frequency.  The insertion loss of the LPF is 0.4 dB typically.

The signal is then split by R138, R139, and R144, and sent on to the appropriate receive or transmit final buffer amplifier.  The signal measured at R139/C150/ C186 node is -6 dBm, and the signal measured at R144/R143/C168 node is -6 dBm.

Q23 (receive) and Q27 (transmit) is the final buffer amplifier.  Q23 amplifies the signal up to the level needed to properly drive the LO port of the DBM (discussed previously).  Q27 amplifies the signal up to the level needed to properly drive the PA pre-driver (previously discussed).  The signal measured at “RXLO” is +7 dBm typically.  The signal measured at “TXLO” is +7 dBm typically.

U17, Q20, etc. form the voltage regulator for the receive and transmit VCO/Buffer amplifiers.  R67, C124, and Q20 form a “super filter” which attenuates voltage regulator noise that may otherwise degrade the synthesizer phase noise performance.

U20, U21, Q29, Q30, CR23, CR24, CR35, C199, C202, C203, etc. create a voltage multiplier.  The circuit is configured as a voltage quadrupler.  Circuit losses and output loading lower the voltage down from 24 VDC to about 22 VDC.  The driver circuit (U21) switches at about 192 kHz.  This frequency was selected so harmonics would not land at or near the second IF frequency of 455 kHz.  The 22 VDC supply is used to power the DAC supporting quad op-amp U18.

The shift register (U24) is used to control transceiver modes of operation and functions.  The line labeled “STD/SIDE” selects the desired antenna port of the transceiver.  The line labeled “TX/RX” selects either the transmit or receive mode of operation.  “+3.3V RXEN” turns the receiver on and off (previously discussed).  Q34/Q32 and Q33/Q31 enable and disable the receive and transmit VCOs and buffers respectively (discussed previously).  U16 is the voltage regulator that supplies all 5V digital circuitry on the transceiver board.

### Digital/Analog Control.

Digital/Analog Control is shown on page 1 of the Transceiver CCA schematic enclosed in the back of this document.  The transceiver board is fitted with an EEPROM (U15) (see page 4 of the schematic diagram).  The IC is used to store calibration and “curve fit” data which is needed when the transceiver is configured with the Racal 25 radio.  Each transceiver has it’s calibration and “curve fit” data stored within the EEPROM.  The calibration and “curve fit” data is written to the EEPROM at the successful conclusion of level 2 testing.  Two quad eight bit serial DACs, a quad twelve bit serial DAC, and supporting operational-amplifiers (U2, U6, U13, U18, and U30 ) control much of the transceiver, as has been discussed previously.  U32 is a 2.5 VDC reference used by the Quad twelve bit DAC and the variable IF attenuator (discussed previously).

U18D and associated components amplifies the DC signal supplied by U31-3.

As was discussed previously,  “REFOSCMOD” is the DC signal, which varies the operating frequency of the reference oscillator.  Normally under DSP and microprocessor control,  this line is used to FM modulate the reference oscillator which in turn FM modulates the RF carrier in transmit mode.  This line is used to temperature compensate the reference oscillator as well.

The DAC controlled line “TXVCOMOD” at U31-4 is transmit data normally controlled by DSP and a microprocessor.  This signal is routed to U18C and associated components (see page 5 on the schematic diagram).  U18C and associated components form an active LPF/attenuator to shape the transmit data before modulating the RF carrier in the transmit mode.  The cutoff frequency of the LPF occurs at 20 kHz.  The 1 kHz peak to peak signal level at the active LPF output (U18-8) is one fourth “TXVCOMOD” at 2.5 VDC.

The synthesizer reference oscillator and the transmit VCO are simultaneously modulated to balance the FM modulation.  We refer to this technique as “two point modulation”.  The DAC values required to balance the modulation are dependent on RF frequency.

The DC signal at U31-17 is routed to U30 and associated components.  This op-amp is configured for a voltage gain of 2.  The DC signal “VATT” controls the variable IF attenuator (discussed previously) in the receiver chain.  Under DSP and microprocessor control, the attenuator is normally set for a desired amount of attenuation by this DAC controlled signal.

Q37, Q38, Q39, Q40, Q41, and associated components are used to enable and disable the 14 dB step attenuator in the receiver chain (discussed previously).  Normally under DSP and microprocessor control, the attenuator is set to the desired state of operation via U31-13.  A logic level “1” at this pin enables the attenuator.  Conversely, a logic level “0” at this pin disables the attenuator (bypass mode).

U18B and associated components amplifies the DC signal supplied by U33-2.  As discussed previously,  “CTUNE” is the DC signal which coarse tunes the receive and transmit VCOs.  Under microprocessor control, the appropriate VCO is normally coarse tuned to a desired frequency based on “curve fit” data stored in the EEPROM (U15).  “Curve fit” data is obtained and stored in the EEPROM during coarse tune calibration procedures performed at level two testing.

The DAC controlled DC signal “2ndLO” sets the 2nd LO (discussed previously) on frequency at 44.545 MHz.  Normally under microprocessor control, the 2nd LO is set on frequency based on a DAC value stored in the EEPROM (U15).  The correct DAC value is obtained and stored in the EEPROM during the 2nd LO calibration procedure at level 2 testing.

The DAC controlled DC signal “RXVTF” appropriately sets the varactor tuned BPF (discussed previously) based on the receiver tuned frequency.  Normally under microprocessor control, the varactor tuned BPF is set based on “curve fit” data stored in the EEPROM (U15).  The “curve fit” is based on statistical data obtained during the testing of hundreds of units.

The DAC controlled DC signal “PWRSET” sets the power amplifier (discussed previously) to a desired power level.  Normally under microprocessor control, the power amplifier is set to the desired level based on “curve fit” data stored in the EEPROM (U15).  The “curve fit” data is obtained and stored in the EEPROM during transmit power calibration procedures at level 2 testing.  The power calibration procedure obtains “curve fit” data for five power level settings (0.1, 0.5, 1.0, 2.0, and 5.0 watts) over the entire transmitter operating frequency range (136-174 MHz).

The DAC controlled DC signals “PA1” and “PA2” set the gate bias for each power transistor (Q6 and Q9 respectively) in the power amplifier circuit (discussed previously).  These two signals are routed to op-amps U2 and U6 (see page 4 of the schematic diagram) which are configured for a voltage gain of 2.  The outputs at U2-1 and U6-1, labeled “PABIAS1” and “PABIAS2” respectively, are then routed to the gates of the power transistors.  The correct DAC values for the bias current are stored in the EEPROM (U15).  The correct DAC value is obtained and stored in the EEPROM during the power amplifier bias calibration procedure at level 2 testing.  Each gate is biased such that 100 mA of current flows through each power transistor with “PWRSET” set to a DAC value of zero in the transmit mode.  Level 2 software monitors the U10 output line labeled “IMONITOR” (see page 4 of the schematic diagram) when calibrating PA bias.

## External Interfaces.

The external interfaces of the radio are described as follows.

### Side Connector.

The Side Connector will be used for PC Programming, including keyfill; cloning; and audio accessory connection.  It uses 18 flush-mounted electrical contacts in a non-conductive molding.  Pin assignments and functions are delineated at Appendix D.

An additional flush mounted coaxial contact is provided to remote the antenna. (future use).

### Battery Connector.

The batteries interface with the radio through a reliable, long-wearing, easily-operated, twist-lock type connector mechanism.

### Antenna Connector

The antennas screw into an SMA female connector mounted on the top rightmost corner of the radio (as viewed from the front of the radio).

# .  ASSEMBLY/DISASSEMBLY

## General.

This chapter provides radio assembly/disassembly instruction.

## Power-On Self Test.

At radio switch-on, the H8 controller executes a number of tests to confirm correct operation.  Any errors are reported to the user through display error messages, and logged in the Flash.  The tests implemented include:

- Flash checksum Cyclic Redundancy Check (CRC),

- RAM read and write

- FPGA configuration

- DSP host interface

- Keypad interface to AVR

- Battery bus interface

- Transceiver EEPROM interface

- Synthesizer lock tests top and bottom frequencies, lock time etc

## CCA Removal and Replacement

### Disassemble the Radio

- Remove and retain the 7 Torx  (T6) screws on the back of the unit.

- Carefully lift up the rear cover.  The Transceiver CCA is attached to the rear cover.  As you lift the back cover the Transceiver CCA will disconnect from the control CCA (connected by a 32 pin connector).  Disconnect the two coaxial antenna connectors from the Transceiver CCA.

NOTE:	If not replacing the Transceiver Cca, skip to step 5.

3.	To remove the Transceiver CCA (Figure 5-1) from the back plate unscrew the 4 Phillips head screws (each has an associated washer) in the corners.

4.	Remove the brass shield.  Remove the acoustic fiberglass and save for reinstallation.

The Transceiver CCA is now free from the unit.

NOTE:	A heat sink for the Power Amp on the Transceiver CCA is integrated into the back plate.  This heat sink is coated with a heat sink dielectric compound.  Apply the compound before reattaching the Transceiver CCA to the rear cover.

Figure 5-12.  Transceiver CCA (shown in Transceiver Module)

- To remove the Control CCA (Figure 5-2), first remove the dielectric shield.  Then remove the 2 Phillips head screws on the side and the 2 Phillips head screws at each bottom corner.  Slide out the side connector flexi-cable from the connector and disconnect the connector from the speaker-microphone.  Lift out from the top end closest to the antenna, taking care not to damage the power flexi-cable.  The Control CCA is now free from the unit.

Figure 5-13.  Control CCA (shown in Control Module)

6.	To remove the Keypad CCA (Figure 5-3), remove six Phillips head screws.  Disconnect the PTT side flexi-cable (must slide connector assembly open first then remove flexi-cable), the top connector flexi cable, and the display flexi cable and lift up the Keypad CCA.

The Keypad CCA is now free from the unit.

### Reassemble the Radio.

NOTE:	When installing screws, any screw that does not have a lock washer requires that LOCTITE thread-locking adhesive is applied to the thread of the screw.  This prevents the screws from being loosened by vibration.  Do not apply LOCTITE to the chassis Torx screws when completing the re-assembly.

- Place the Keypad CCA (Figure 5-3) into the radio chassis.  Attach the six Phillips head and tighten to 3 inch-pounds.  Carefully slide the top connector flexi-cable, the display flexi-cable, and the PTT flexi-cable into the corresponding connectors on the Keypad CCA.

Figure 5-14.  Keypad CCA (shown in Control Module)

2.	Lift the side connector flexi and slide the Control CCA (Figure 5-2) into the radio chassis. (under the power flexi at the bottom of the radio.)  Gently but firmly push down on the bottom left hand corner to mate the Control CCA and Keypad CCA 64 pin connector.  Reinstall the four Phillips head screws at the corners of the CCA and tighten to 3 inch-pounds.   Attach the side connector flexi-cable to the CCA.

3.	Place the dielectric shield on top of the Control CCA.

NOTE:	The Transceiver CCA (Figure 5-1) and RF shield must be attached to the rear plate before they can be reattached to the radio (see Figure 7-24).

4.	Install the Acoustic Fiberglass shield to the Transceiver CCA.

5.	Before reattaching the Transceiver CCA, apply a dielectric gel to the heat sink attached to the rear plate.

6.	Reattach the 4 Phillips head screws through the RF Shield and Transceiver CCA holes and into the rear plate.  Tighten the screws to 2.5 inch-pounds.

7.	The easiest way to reattach the Rear Plate/ Transceiver CCA is place the radio down, then rotate the Rear Plate/Transceiver CCA 180o and put it down above the radio so that the top of the radio and the top of the Transceiver are next to each other.  This will make it easier to reattach the antenna leads.  Attach the antenna leads to the Transceiver CCA.  Verify that the RF gasket is still in its groove on the rear plate - if it is out work it back into the groove (or replace).  Flip the Rear Plate/Transceiver CCA up and over, align the connector on the Transceiver CCA with the Control CCA connector and firmly press the rear plate on to the radio chassis.  Reattach 7 Torx screws and torque the screws to 2.5 inch-pounds.

The radio has been reassembled.

# .  TROUBLESHOOTING

## Introduction.

This chapter is included to help qualified service personnel troubleshoot and repair the Racal 25 radio.  If questions or problems arise, please contact the Racal Help Line at 1-800-914-0303.  To help in troubleshooting and repairing the Racal25 radio please refer to the following sections of this Manual:

- Chapter 3.0	Software Theory of Operation

- Chapter 4.0	Hardware Theory of Operation

- Chapter 5.0	Assembly/Disassembly Instructions

- Chapter 7.0	Assembly Drawings

- Back of Document	Schematics

## How to Use This Section.

This section contains basic radio functional tests.  When performing these tests, refer to Figure 6-1.  If a basic test fails, Figure 6-1 will refer you to the appropriate individual flowchart by Figure number.  Use the individual flowchart to find and correct the problem.  Once the problem has been corrected, restart the tests from the beginning.

## Radio Functional Tests.

The tests in this section require the radio to be tested is programmed.  The suggested programming is to program all three auxiliary buttons to Hi/Lo Power, the Emergency button to Emergency, and the Toggle Switch to Zone Select.  Develop eighteen (18) Channels, three Zones, and one Bank.  Put sixteen (16) channels in Zone 1, one channel in Zone2 and one channel in Zone 3.  For all the channels, use assigned transmit and receive frequencies, turn transmit squelch off and set receive squelch to carrier, level 8.  When more than one remove and receive tasks are shown in a block, they are listed in order from most to least probable to fix the problem.  It is recommended that the remove and repair tasks are tried one at a time, and the radio retested, until the problem is fixed.

### Overall Radio Test.

This test outlined at Figure 6-15 is an overall test to check the radio, ensure it is working properly, and if not, try to determine where the problem resides.  As shown in Figure 6-15, the tests in Figures 6-2 through Figure 6-6 will be run only if a part of this test fails.

#### Power-On Test.

Using the flowchart at Figure 6-2, this test ensures that the radio turns on, the latest software version number briefly appears, a beep is heard and an operational screen appears on the radio LCD.

#### Buttons and Switches Test.

Using the flowchart at Figure 6-3, this test ensures that the PTT, auxiliary buttons, toggle switch, emergency key, keypad, on/off/volume switch, and channel switch all work.

#### Transmit Test

Using the flowchart at Figure 6-4, this test ensures the radio has the required transmit power, frequency accuracy, and deviation.

#### Receive Test

Using the flowchart at Figure 6-5, this test ensures that the radio LED works, a 1kHz tone is heard, and that SINAD is within specified limits.

#### Audio Test

This test ensures that the radio internal speaker and microphone are working.  If the radio fails this test, please return it to Racal Communications for servicing.

#### Accessories Test

Using the flowchart at Figure 6-6, this test ensures that the lithium ion battery, the AA battery pack, the external speaker/microphone, the PC Programmer, and the cloning cable are working properly.

START TEST

POWER-ON

TEST

BUTTONS

AND SWITCHES

TEST

TRANSMIT

TEST

RECEIVE

TEST

AUDIO

TEST

ACCESSORIES

TEST

END OF TEST

PASS

PASS

PASS

PASS

PASS

PASS

FAIL

FAIL

FAIL

FAIL

FAIL

FAIL

GO TO FIGURE 6-2

POWER-ON TROUBLESHOOTING

GO TO FIGURE 6-3

CONTROLS TROUBLESHOOTING

GO TO FIGURE 6-4

TRANSMIT TROUBLESHOOTING

GO TO FIGURE 6-5

RECEIVE TROUBLESHOOTING

IF THE AUDIO TEST FAILS, THE INTERNAL SPEAKER/MICROPHONE MUST BE REPLACED. RETURN TO RACAL COMMUNICATIONS FOR SERVICE.

GO TO FIGURE 6-7

ACCESSORIES TROUBLESHOOTING

RETEST

Figure 6-15.  Overall Radio Test

START TEST

RADIO DISPLAYED SOFTWARE VERSION

RADIO DISPLAYED NORMAL OPERATION

YES

YES

END OF TEST

YES

NO

DID ALL

TESTS PASS AFTER REPAIR

NO

RETEST

RADIO DISPLAY REMAINED BLANK

CHECK CABLE CONNECTIONS

ABLE TO RECTIFY PROBLEM

CALL RACAL FOR SERVICE

IS BATTERY VOLTAGE     8 - 13 VDC

CHARGE OR REPLACE BATTERY

ATTEMPT FIRMWARE RELOAD

CHECK CONTROL BOARD VOLTAGES

REMOVE AND REPLACE POWER FLEXI (3100628-501)

REMOVE AND REPLACE

CONTROL BD (4101258-501),

LCD (1600482-1), AND/OR

CONTROL-DISPLAY FLEXI (55326-1)

YES

YES

NO

BAD

G

O

O

D

GOOD

BAD

FIXED

DID NOT FIX

YES

NO

Figure 6-16.  Power-On Troubleshooting

START TEST

KEYPAD

TEST

PASS

FAIL

REMOVE AND REPLACE

KEYPAD BOARD (3100618-501)

KEYPAD (4400412-1)

RETEST

FIXED

PTT TEST

PASS

FAIL

PROGRAM CHANNELS

CHANNEL

PROGRAMMED

RED LED

ILLUMINATE

ACOUSTIC INSULATION BLOCKING

LED

REMOVE/REPLACE CONTROL BOARD (4101258-501)

NO

REINSTALL

INSULATION

AUXILIARY BUTTON

TEST

FAIL

REMOVE/REPLACE

PTT/AUX BUTTON

FLEXI (3300153-1)

YES

NO

DID

NOT

FIX

YES

YES

NO

TOGGLE

SWITCH

TEST

EMERGENCY

KEY TEST

PASS

PASS

FAIL

FAIL

REMOVE AND REPLACE

TOP SWITCH FLEXI (3300151-1)

EMERGENCY BUTTON (3400516-1)

CHECK CABLE CONNECTIONS

REMOVE AND REPLACE

KEYPAD BOARD (3100618-501)

TOGGLE SWITCH (50002)

TOP SWITCH FLEXI (3300151-1)

CONTROL BOARD (4101258-501)

GO TO SHEET 2

Figure 6-17.  Controls Troubleshooting (Sheet 1 of 2)

END OF TEST

PASS

FROM SHEET 1

ON/OFF/

VOLUME

SWITCH TEST

CHANNEL

SWITCH

TEST

REMOVE AND REPLACE

KEYPAD BOARD (3100618-501)

TOP SWITCH FLEXI (3300151-1)

VOLUME SWITCH (3600178-1)

REMOVE AND REPLACE

KEYPAD BOARD (3100618-501)

TOP SWITCH FLEXI (3300151-1)

CHANNEL SWITCH (3600178-1)

FAIL

FAIL

PASS

RETEST

Figure 6-3.  Controls Troubleshooting (Sheet 2 of 2)

START TEST

RETEST

RF POWER TEST

PASS

FAIL

PROGRAM RADIO

RADIO

PROGRAMMED

NO

FREQUENCY

TEST

FAIL

PROGRAM RADIO

RADIO

PROGRAMMED

NO

PASS

REMOVE AND REPLACE

SIDE CONNECTOR FLEXI (3300155-1)

TRANSCEIVER BOARD (4101257-501)

CONTROL BOARD (4101258-501)

REMOVE AND REPLACE

RF CABLE (2600133-2)

TRANSCEIVER BOARD (4101257-501)

ANTENNA (1600468-4)

CONTROL BOARD (4101258-501)

YES

YES

FAIL

END OF TEST

PASS

DEVIATION

TEST

REMOVE AND REPLACE

TRANSCEIVER BOARD (4101257-501)

Figure 6-18.  Transmit Troubleshooting

START TEST

RETEST

GREEN

LED TEST

PASS

FAIL

PROGRAM RADIO

RADIO

PROGRAMMED

NO

YES

REINSTALL INSULATION

NO

ACOUSTIC INSULATION BLOCKING

LED

REMOVE/REPLACE

CONTROL BOARD (4101258-501)

1kHz TONE

TEST

FAIL

PASS

REMOVE AND REPLACE

RF CABLE (2600133-1)

TRANSCEIVER BOARD (4101257-501)

CONTROL BOARD (4101258-501)

FAIL

SINAD TEST

REMOVE AND REPLACE

TRANSCEIVER BOARD (4101257-501)

RF CABLE (2600133-1)

CONTROL BOARD (4101258-501)

SIDE CONNECTOR FLEXI (3300155-1)

END OF TEST

PASS

Figure 6-19.  Receive Troubleshooting.

START TEST

RETEST

PASS

LI-ION

BATTERY TEST

PASS

FAIL

CLEAN BATTERY  CONTACTS

DOES DISPLAY

SHOW 2W

NO

REMOVE AND REPLACE

AA BATTERIES (4101287-501)

AA BATTERY REGULATOR (4101291-                 501)

DISPLAY

STILL

SHOW 2W

AA BATTERY

TEST

TURN AROUND BATTERY

or

REMOVE AND REPLACE

CONTROL BOARD (4101258-501)

LI-ION BATTERY (4101106-501)

RF POWER FLEXI (3100628-501)

DID RADIO

SHUT OFF

CHARGE or

REMOVE AND REPLACE

LI-ION BATTERY (4101106-501)

SPEAKER/

MICROPHONE

TEST

REMOVE AND REPLACE

SPEAKER/MICROPHONE (1600469-X)

PC

PROGRAMMER

TEST

CHECK

CONNECTIONS

COMMUNICATIONS PORT

REMOVE AND REPLACE

PC PROGRAMMER CABLE (4100152-               501)

CLONING

CABLE TEST

CHECK

CONNECTIONS

REMOVE AND REPLACE

CLONING CABLE (4500153-501)

END TEST

YES

NO

Figure 6-20.  Accessories Troubleshooting Flowchart

# .  SPECIFICATIONS AND PARTS LISTS

Section I - INTRODUCTION

## Parts List.

The Parts List shows the complete radio system divided into main groups.  The main groups are broken down into assemblies, subassemblies, and details.  In general, the assemblies and parts installed at the time the end item was manufactured are listed and identified in the manual.

	Figure and Index Number.  Lists the figure and index number of each part illustrated in the related figure.  The index numbers are in numerical sequence and identify each part number shown in the related figure.

	Part Number.  Lists the part number, including dash numbers, assigned to each part.

	Description.  Contains the description of all items appearing on the parts list.  The description contains modifiers necessary to identify the particular item.

	Units Per Assembly.  Contains the number of units required per assembly and/or subassembly.  If more than one assembly is required, the total number of assemblies is listed.

## Electrostatic Discharge (ESD) Sensitive Devices.

This equipment contains parts and assemblies sensitive to damage by Electrostatic Discharge (ESD).

## Specifications.

### Physical Characteristics.

Table 7-1 contains the radio physical characteristics.

| Table 7-1. Physical Characteristics |  |
| --- | --- |
| Characteristic | Measurement |
| Weight | 17.5 ounces, including battery and antenna. |
| Dimensions | 2.60" wide x 7.50" high x 1.05" deep, including battery, excluding control knobs and antenna. |
| RF Power output | Programmable on a channel by channel basis to 0.1, 0.5, 1, 2 or 5 Watts (W), limited to 2W maximum when using alkaline batteries, when it is set to 5W. |
| Audio output | 1W at maximum volume |
| Current Drain | Standby: 60 mA nominal Receive: 100-350 mA nominal, depending on volume setting Transmit: 500 -1,500 mA nominal, depending on power setting |
| Operating Temperature | -30Celsius (C) (-22o Fahrenheit (F)) to +60C (140F) |
| Storage Temperature | -40C (-40F) to +85C (185F) |
| Relative humidity | 95% non-condensing relative humidity through-out its operating temp range. |
| Immersibility | 2 meters in fresh water for 1 hour. |

### Technical Specifications

#### System

Frequency Range	136 - 174 MHz

#### Transmit

RF Output Power (0.1W, 0.5W, 1.0W, 2.0W)	± 20%

RF Output Power (5.0W)	 + 20%, -0%

Operating Frequency Accuracy	± 2 ppm (± 1 ppm typical)

Radiated Spurious Emissions	 - 13 dBm

Conducted Spurious Emissions	 - 13 dBm

Adjacent Channel Power Ratio	 70 dB @ 25kHz spacing

Intermodulation Attenuation	> + 40 dBm

Transmitter Encoder Attack Time	 100 ms

Modulation Fidelity for C4FM	± 10% FM

< 34 dB AM

Symbol Rate Accuracy	± 10 ppm

RF Output Impedance	50 ohms nominal

IF Rejection	 70 dB

Image Rejection	 70 dB

#### Receive

Radiated Spurious Emissions (at 3 meters)	 150 uV/m

Conducted Spurious Emissions	 - 57 dBm

Reference Sensitivity	 -116 dBm

 -121 dBm typical

Faded Reference Sensitivity	 -108 dBm

Frequency Tolerance	± 2 ppm (± 1 ppm typical)

Adjacent Channel Rejection	 60 dB @ 25 kHz spacing

Co-Channel Rejection	 9 dB

Intermodulation Rejection	 70 dB

Spurious Response Rejection	 70 dB

Signal Displacement Bandwidth	 1 kHz

Audio Output Distortion	 5 % ( 2 % typical)

Residual Audio Noise Ratio	 45 dB silence

 35 dB muted

Average Radiation Sensitivity	 98 dBm

BIT Error Rate	 0.01%

### Accessory Equipment

Available accessory equipment is listed in Table 7-2:

| Table 7-2. Accessory Equipment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Part Number | Application |
| Single-Unit Battery Charger | 1600426-1 | Recharges one battery at a time. |
| Six-Unit Battery Charger (AC/DC) | 1600426-2 | Recharges up to six batteries at one time, in a fielded environment. |
| Six-Unit battery Charger (AC only) | 1600426-3 | Recharges up to six batteries at one time, in an office environment. |
| Nylon Bag | 1600467-1 | A harness type support for the radio |
| Leather bag | 1600467-2 | A harness type support for the radio |
| Leather bag (for use with the alkaline battery cassette) | 1600467-4 | A harness type support for the radio |
| Standard Broadband Antenna | 1600468-1 | 6-inch whip antenna, 136-174 MHz |
| Low-Band Narrowband Antenna | 1600468-2 | 7.5-inch whip antenna, 136-152 MHz |
| Mid-Band Narrowband Antenna | 1600468-3 | 7.5-inch whip antenna, 147-163 MHz |
| High-Band Narrowband Antenna | 1600468-4 | 7.5-inch whip antenna, 158-174 MHz |
| Speaker-Microphone, AC Coupled ** | 1600469-1 | External audio accessory which includes a volume control and an earphone connector. |
| Speaker-Microphone, Balanced | 1600469-2 | External audio accessory which includes a volume control and an earphone connector. It provides a greater volume than 1600469-1. |
| Surveillance Harness ** | 1600497-1 (3-wire tan) 1600497-2 (3-wire black) 1600497-3 (2-wire tan) 1600497-4 (2-wire black) | Earpiece and wiring harness with audio connector for covert use of the radio. |
| Audio Transducer | 1600497-5 | Part of surveillance harness; transmits a signal between the radio and earpiece. |
| Wireless Earpiece | 1600497-6 | Part of surveillance harness, inductively coupled earpiece worn inside the ear. |
| Voice Tube | 1600497-7 | Part of surveillance harness; voice tube |
| Flexible Open Ear Insert | 1600497-8 | Part of surveillance harness; voice tube attachment to the ear. |
| DES Encryption Keyfill Cable ** | 3500459-501 | Allows the loading of DES Encryption Keys through the side connector. |
| Belt Clip | 40508 | Mechanical means of attaching the radio to a belt. |
| Lithium-Ion Rechargeable Battery | 4101106-501 | Standard power source for the radio. Can be recharged using the battery chargers. |
| Audio Adapter Assembly | 4101179-501 | Connects to the radio side connector to allow several audio accessories to be connected. |
| AA Battery Cassette/Top | 4101287-501 | Reusable case for ten (10) commercial alkaline batteries. |
| Cloning Cable | 4500153-501 | Allows cloning (transfer) of programming information from one radio to another. |
| PC Programmer | MA6941C | Loads programming into the radio from a PC. It includes software, manual, and a cable. |

** - Requires the side connector accessory (4101179-1) to interface to the radio.

Table 7-3.  Racal 25 Portable Radio (PRC6994HB-SYS)

| Figure & Index No. | Part Number | Description | Units per Assy |
| --- | --- | --- | --- |
| 7-1-1 | 1600468-1 | Broadband Antenna, Standard 136-174 MHz | 1 |
| 7-1-2 | 4101256-501 | Unit Assy Racal 25 portable | 1 |
| 7-1-3 | 4101106-501 | Battery Rechargeable, Lithium-Ion | 1 |
| not shown | 4101171-501 | AA Battery Cassette Clamshell (uses 10 AA batteries) | (optional) |
| not shown | 1600468-2 | Low Band Narrowband Antenna, 136 - 174 MHz | (optional) |
| not shown | 1600468-3 | Mid-Band Narrowband Antenna, 147-163 MHz | (optional) |
| not shown | 1600468-4 | High-Band Narrowband Antenna, 158-174 MHz | (optional) |

Figure 7-21.  Racal 25 Portable Radio

Table 7-4.  Racal 25 Portable Transceiver (4101126-501)

| Figure & Index No. | Part Number | Description | Units per Assy |
| --- | --- | --- | --- |
| 7-2-1 | 4101254-501 | Module Assy Ctrl/Crypto/Display | 1 |
| 7-2-2 | 4101255-501 | Module Assy Transceiver 136-174 MHz | 1 |

Figure 7-22.  Racal 25 Portable Transceiver

Figure 7-23.  Racal 25 Control/Crypto/Display Module Assembly (4101254-501), Exploded View

Table 7-5.  Racal 25 Control/Crypto/Display Module Assembly (4101254-501), Exploded View

| Figure & Index No. | Part Number | Description | Units/ Assy | Figure & Index No. | Part Number | Description | Units/ Assy |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7-3-A1 | 3100618-501 | CCA, Keypad | 1 | 7-3-48 | 4400437-1 | Top Switch, Plastic | 1 |
| 7-3-A2 | 4101258-501 | CCA, Control | 1 | 7-3-52 | 4400386-1 | Power Pin | 1 |
| 7-3-A3 | 1600482-1 | Module LCD | 1 | 7-3-53 | 3400523-1 | Insulator, Power Pin | 1 |
| 7-3-A4 | 3100628-501 | CCA, Power Flex | 1 | 7-3-54 | 3400520-1 | Insulator, Data Pin | 1 |
| 7-3-J1 | 4101160-501 | Side Connector Assy | 1 | 7-3-56 | 4400385-1 | Seal, Insul, Power Pin | 1 |
| 7-3-J2 | 60140 | Plug, Flush Mount |  | 7-3-57 | 3400518-1 | Pin, Data | 1 |
| 7-3-S1 | 50002 | Switch, Toggle | 1 | 7-3-58 | 3400519-1 | Pin, Data Cap | 1 |
| 7-3-S2, 7-3-S3 | 3600178-1 | Switch, 16 Position | 2 | 7-3-60 | 71101-2506 | Screw | 1 |
| 7-2-W1 | 3500384-502 | Cable Assy, Speaker/ Microphone | 1 | 7-3-61 | 71101-2510 | Screw | 1 |
| 7-3-W2 | 55236-11 | Cable, Flex | 1 | 7-3-62 | 74856 | Nut, Plain | 1 |
| 7-3-W3 | 2600133-1 | Cable Assy | 1 | 7-3-63 | 75016 | Washer, Lock | 1 |
| 7-3-W4 | 3300151-1 | FFC, Top Switch | 1 | 7-3-64 | 71101-2005-BLK | Screw | 4 |
| 7-3-W5 | 3300155-1 | FFC, Side Connector | 1 | 7-3-65 | SS-2400568-1 | Nut, Switch | 2 |
| 7-3-W6 | 2600133-2 | Cable Assy | 1 | 7-3-66 | SS-71464 | Washer, Crinkle | 2 |
| 7-3-W7 | 3300153-1 | FFC, PTT | 1 | 7-3-67 | 81833 | Retaining Ring | 1 |
| 7-3-22 | 71100-2004 | Screw | 6 | 7-3-68 | 91215 | Spring, Compression | 1 |
| 7-3-25 | 2600122-2 | Gasket, Fibrous | 1 | 7-3-69 | 91167 | Spring, Compression | 1 |
| 7-3-28 | SS-71460 | Washer, Lock | 10 | 7-3-70 | 81286 | O-Ring | 1 |
| 7-3-30 | 81038 | Knob, Volume | 1 | 7-3-71 | 81287 | O-Ring | 1 |
| 7-3-31 | 81037 | Knob, Channel | 1 | 7-3-72 | 81288 | O-Ring | 2 |
| 7-3-32 | 42764 | Speaker | 1 | 7-3-75 | 3400607-1 | Adhesive Preform LCD | 1 |
| 7-3-35 | 3400506-1 | Plate, Negative | 1 | 7-3-77 | 4101173-501 | Chassis Assy | 1 |
| 7-3-38 | 4400381-1 | Gasket Side Connector | 1 | 7-3-78 | 4400382-1 | Light Pipe | 1 |
| 7-3-39 | 4400416-1 | Rubber, PTT | 1 | 7-3-79 | 3400547-1 | Seal, Light Pipe | 1 |
| 7-3-40 | 4400378-1 | Cover, PTT | 1 | 7-3-80 | 2100388-501 | Side Coaxial Assy | 1 |
| 7-3-41 | 4400384-1 | Actuator, PTT | 1 | 7-3-85 | 2400781-1 | Nut Microphone Hldr | 1 |
| 7-3-44 | 4400412-1 | Keypad, 16 Button | 1 | 7-3-86 | B4009086 | Washer, Insulating | 1 |
| 7-3-45 | 3600156-1 | Window, LCD | 1 | 7-3-87 | 71100-2006 | Screw | 4 |
| 7-3-47 | 3400516-1 | Button, Emergency | 1 | 7-3-90 | 77009 | Set Screw | 2 |

7-3-81, 7-3-82, 7-3-83, and 7-3-84 are Adhesives, Sealants and Lubricants and as such are not listed.

Figure 7-24.  Racal 25 Transceiver Assembly (4101255-501)

Table 7-6.  Racal 25 Transceiver Assembly (4101255-501)

| Figure & Index No. | Part Number | Description | Units per Assy |
| --- | --- | --- | --- |
| 7-4-21 | 4101257-501 | Circuit Card Assembly, Transceiver | 1 |
| 7-4-22 | 4101259-501 | Cover Assembly, Rear, Finished | 1 |
| 7-4-23 | 4400367-1 | Cover, Rear, Rubber Seal | 1 |
| 7-4-24 | 4400368-1 | Clamp Bar | 2 |
| 7-4-25 | 71100-2004 | Screw | 4 |
| 7-4-27 | SS-71460 | Washer, Lock | 4 |
| 7-4-29 | 4400392-1 | Shield, RF | 1 |
| 7-4-30 | 91087 | Adhesive Sealant, Dow RTV 3140 | A/R |
| 7-4-31 | 91012 | Thermal Compound | A/R |
| 7-4-32 | 595012 | Insulation, Acoustic AM 90S | A/R |

Figure 7-25.  Racal 25 Keypad CCA (3100618-501)

Figure 7-26.  Racal 25 Control CCA (4101258-501)

This page left blank intentionally

Figure 7-27.  Racal 25 Transceiver Circuit Card (4101257-501), PRIMARY SIDE

Figure 7-28.  Racal 25 Transceiver Circuit Card (4101257-501), SECONDARY SIDE

Table 7-7.  Transceiver CCA (4101257-501) Bill of Materials

| Reference Designation |  |  | Part Number |  | Transceiver CCA Component Description |
| --- | --- | --- | --- | --- | --- |
| A/R | 595012 |  | INSULATION ACOUSTIC AM 90S (1 |  |
| B1 | 45270 |  | BEAD FERRITE 50@100 |  |
| CAPACITORS |  |  |  |  |
| C1 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C2 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C3 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C4 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C5 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C6 | 22565-390 |  | CERAMIC 0402 COG 39 pF |  |
| C7 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF |  |
| C8 | 25149-1R0-B-50 |  | CERAMIC 0603 COG 1.0 pF-.1pF |  |
| C9 | 25149-2R7-B-50 |  | CERAMIC 0603 COG 2.7 pF-.1pF |  |
| C10 | 25149-9R1-B-50 |  | CERAMIC 0603 COG 9.1 pF-.1pF |  |
| C11 | 25149-5R6-B-50 |  | CERAMIC 0603 COG 5.6 pF-.1pF |  |
| C12 | 22565-121 |  | CERAMIC 0402 COG 120 pF |  |
| C13 | 22565-121 |  | CERAMIC 0402 COG 120 pF |  |
| C14 | 22579-180-G-50 |  | CERAMIC 0603 COH 18 pF |  |
| C15 | SS-22555-220 |  | CERAMIC 0805 NPO 22 pF 2% 200v |  |
| C16 | 22579-220-G-50 |  | CERAMIC 0603 COH 22 pF 2 |  |
| C17 | 22579-220-G-50 |  | CERAMIC 0603 COH 22 pF 2 |  |
| C18 | 25149-8R2-B-50 |  | CERAMIC 0603 COG 8.2 pF-.1pF |  |
| C19 | SS-28058 |  | VAR CERAMIC NPO 2-10 pF SMT |  |
| C22 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C23 | 22565-151 |  | CERAMIC 0402 COG 150 pF 5 |  |
| C25 | 22580-6R2 |  | CERAMIC 0603 COG 6.2 pF +/-.25p |  |
| C26 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF10 |  |
| C27 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C28 | 22565-330 |  | CERAMIC 0402 COG 33 pF 5 |  |
| C30 | SS-22555-180 |  | CERAMIC 0805 NPO 18 pF 2% 200V |  |
| C31 | 22571-100 |  | CERAMIC 0402 COG 10 pF+/-.25pF |  |
| C34 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C38 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C43 | SS-25122-475-10 |  | TANTALUM A-CASE 4.7 uF 10% 10V |  |
| C53 | 25149-100-G-50 |  | CERAMIC 0603 COG 10 pF 2 |  |
| C56 | 22571-100 |  | CERAMIC 0402 COG 10 pF+/-.25pF |  |
| C57 | 22571-1R8 |  | CERAMIC 0402 COG 1.8 pF+/-.25pF |  |
| C59 | 22565-101 |  | CERAMIC 0402 COG 100 pF 5 |  |
| C61 | 22565-121 |  | CERAMIC 0402 COG 120 pF 5 |  |
| C67 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C68 | 25122-335-16 |  | TANTALUM A-CASE 3.3 uF 10% 16V |  |
| C69 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C70 | 22584-224 |  | CERAMIC 0603 X7R 0.22 uF10 |  |
| C71 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C72 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C73 | 22565-101 |  | CERAMIC 0402 COG 100 pF 5 |  |
| C74 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF10 |  |
| C75 | 22571-3R9 |  | CERAMIC 0402 COG 3.9 pF+/-.25pF |  |
| C76 | SS-22527-102 |  | CERAMIC 0805 NPO 1000 pF 5% 50V |  |
| C77 | 25149-100-G-50 |  | CERAMIC 0603 COG 10 pF 2 |  |
| C78 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C79 | 22565-270 |  | CERAMIC 0402 COG 27 pF 5 |  |
| C81 | SS-22554-2R7 |  | CERAMIC 0805 NPO 2.7 pF+/-.25pF |  |
| C82 | SS-22555-330 |  | CERAMIC 0805 NPO 33 pF 2% 200V |  |
| C83 | 25156-9R0-D-50 |  | CERAMIC 0805 COG 9.0 pF +/-0.5p |  |
| C84 | SS-22555-120 |  | CERAMIC 0805 NPO 12 pF 2% 200V |  |
| C85 | 25149-100-G-50 |  | CERAMIC 0603 COG 10 pF 2 |  |
| C86 | 22579-150-G-50 |  | CERAMIC 0603 COH 15 pF 2 |  |
| C87 | 22571-1R8 |  | CERAMIC 0402 COG 1.8 pF+/-.25pF |  |
| C88 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF10 |  |
| C89 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C90 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C91 | 22572-271 |  | CERAMIC 0402 X7R 270 pF 5% 50V |  |
| C92 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF10 |  |
| C93 | 22584-224 |  | CERAMIC 0603 X7R 0.22 uF10 |  |
| C94 | SS-22527-102 |  | CERAMIC 0805 NPO 1000 pF 5% 50V |  |
| C95 | 22572-332 |  | CERAMIC 0402 X7R 3300 pF 5% 50V |  |
| C96 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C97 | SS-22527-102 |  | CERAMIC 0805 NPO 1000 pF 5% 50V |  |
| C98 | SS-22527-102 |  | CERAMIC 0805 NPO 1000 pF 5% 50V |  |
| C99 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF10 |  |
| C100 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C101 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C102 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C103 | 22572-391 |  | CERAMIC 0402 X7R 390 pF 5% 50V |  |
| C104 | 22572-391 |  | CERAMIC 0402 X7R 390 pF 5% 50V |  |
| C105 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C106 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C107 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C108 | SS-25122-475-10 |  | TANTALUM A-CASE 4.7 uF 10% 10V |  |
| C109 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C110 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF10 |  |
| C111 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C112 | SS-22527-102 |  | CERAMIC 0805 NPO 1000 pF 5% 50V |  |
| C113 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C114 | 22579-100-G-50 |  | CERAMIC 0603 COH 10 pF 2 |  |
| C116 | SS-22527-561 |  | CERAMIC 0805 NPO 560 pF 5% 50V |  |
| C117 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C118 | 25148-105-25-A |  | TANTALUM A-CASE 1 uF 10% 25V |  |
| C120 | SS-25122-475-10 |  | TANTALUM A-CASE 4.7 uF 10% 10V |  |
| C121 | 25148-105-25-A |  | TANTALUM A-CASE 1 uF 10% 25V |  |
| C123 | SS-25122-475-10 |  | TANTALUM A-CASE 4.7 uF 10% 10V |  |
| C124 | SS-25122-475-10 |  | TANTALUM A-CASE 4.7 uF 10% 10V |  |
| C125 | SS-25131 |  | TANTALUM A-CASE 2.2 uF 10% 10V |  |
| C126 | 22564-153 |  | CERAMIC 0603 X7R 0.015 uF 10% 16V |  |
| C127 | 25149-100-G-50 |  | CERAMIC 0603 COG 10 pF 2 |  |
| C128 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C129 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C130 | 25149-1R0-B-50 |  | CERAMIC 0603 COG 1.0 pF +/-.1pF |  |
| C131 | 22572-332 |  | CERAMIC 0402 X7R 3300 pF 5% 50V |  |
| C132 | 22565-221 |  | CERAMIC 0402 COG 220 pF |  |
| C133 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C134 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C135 | 25149-8R2-B-50 |  | CERAMIC 0603 COG 8.2 pF?-.1pF |  |
| C136 | 25149-6R8-B-50 |  | CERAMIC 0603 COG 6.8 pF?-.1pF |  |
| C137 | 22579-220-G-50 |  | CERAMIC 0603 COH 22 pF 2 |  |
| C138 | 22579-220-G-50 |  | CERAMIC 0603 COH 22 pF 2 |  |
| C139 | 22571-4R7 |  | CERAMIC 0402 COG 4.7 pF+/-.25pF |  |
| C140 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C141 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C142 | 22579-220-G-50 |  | CERAMIC 0603 COH 22 pF 2 |  |
| C143 | 25148-106-10-B |  | TANTALUM B-CASE 10 uF 10% 10V |  |
| C144 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C145 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C146 | 22564-223 |  | CERAMIC 0603 X7R 0.022 uF 10% 16V |  |
| C147 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C148 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C149 | 25149-120-G-50 |  | CERAMIC 0603 COG 12 pF 2 |  |
| C150 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C151 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C152 | SS-25131 |  | TANTALUM A-CASE 2.2 uF 10% 10V |  |
| C153 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C154 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C155 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C156 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C157 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C158 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C159 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C160 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C161 | 22564-223 |  | CERAMIC 0603 X7R 0.022 uF 10% 16V |  |
| C162 | 25148-106-10-B |  | TANTALUM B-CASE 10 uF 10% 10V |  |
| C163 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C164 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C165 | 25149-100-G-50 |  | CERAMIC 0603 COG 10 pF 2 |  |
| C166 | 25149-9R1-B-50 |  | CERAMIC 0603 COG 9.1 pF?-.1pF |  |
| C167 | 22565-150 |  | CERAMIC 0402 COG 15 pF 5 |  |
| C168 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C169 | 22579-270-J-25 |  | CERAMIC 0603 COH 27 pF 5 |  |
| C170 | 22565-150 |  | CERAMIC 0402 COG 15 pF 5 |  |
| C171 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C172 | 22562-682 |  | CERAMIC 0603 X7R 6800 pF10% 50V |  |
| C173 | 22568-222 |  | CERAMIC 0402 X7R 2200 pF10% 25V |  |
| C174 | 22571-4R7 |  | CERAMIC 0402 COG 4.7 pF+/-.25pF |  |
| C175 | 22579-150-G-50 |  | CERAMIC 0603 COH 15 pF 2 |  |
| C176 | 22579-150-G-50 |  | CERAMIC 0603 COH 15 pF 2 |  |
| C177 | 22573-683 |  | CERAMIC 1206 X7R 0.068 uF 5%100V |  |
| C178 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C179 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C180 | 22579-180-G-50 |  | CERAMIC 0603 COH 18 pF 2 |  |
| C181 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C182 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C183 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C184 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C185 | 22565-150 |  | CERAMIC 0402 COG 15 pF 5 |  |
| C186 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C187 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C188 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C189 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C190 | SS-25133 |  | CHIP TANTALUM 2.2uF 20% 16V |  |
| C191 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF |  |
| C192 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C193 | 25148-105-25-A |  | TANTALUM A-CASE 1 uF 10% 25V |  |
| C194 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF |  |
| C195 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C196 | SS-25122-105-35 |  | TANTALUM B-CASE 1.0 uF 10% 35V |  |
| C197 | 22565-181 |  | CERAMIC 0402 COG 180 pF |  |
| C198 | 22565-151 |  | CERAMIC 0402 COG 150 pF |  |
| C199 | 22583-333 |  | CERAMIC 0603 X7R 0.033 uF 10% 25V |  |
| C200 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF 10 |  |
| C201 | SS-25122-105-35 |  | TANTALUM B-CASE 1.0 uF 10% 35V |  |
| C202 | 22583-333 |  | CERAMIC 0603 X7R 0.033 uF 10% 25V |  |
| C203 | 22583-333 |  | CERAMIC 0603 X7R 0.033 uF 10% 25V |  |
| C206 | SS-25122-105-35 |  | TANTALUM B-CASE 1.0 uF 10% 35V |  |
| C207 | SS-22527-102 |  | CERAMIC 0805 NPO 1000 pF 5% 50V |  |
| C208 | SS-22527-102 |  | CERAMIC 0805 NPO 1000 pF 5% 50V |  |
| C209 | SS-22555-270 |  | CERAMIC 0805 NPO 27 pF 2% 200V |  |
| C210 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF 10 |  |
| C211 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF 10 |  |
| C213 | SS-22543-151 |  | CERAMIC 0805 NPO 150 pF 2% 100V |  |
| C214 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF 10 |  |
| C215 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF 10 |  |
| C216 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF 10 |  |
| C217 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF 10 |  |
| C218 | SS-19350-0402 |  | RES 0402 JUMPER ZERO-OHM |  |
| C222 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF10 |  |
| C223 | 22565-180 |  | CERAMIC 0402 COG 18 pF 5 |  |
| C224 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C225 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C226 | 22565-560 |  | CERAMIC 0402 COG 56 pF |  |
| C227 | 22565-560 |  | CERAMIC 0402 COG 56 pF |  |
| C228 | 22567-103 |  | CERAMIC 0402 X7R 0.01 uF |  |
| C229 | 25150-226-25-D |  | CAP TAN D-CASE 22 uF 20% TPS |  |
| C230 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C231 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C233 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C236 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C237 | 22571-8R2 |  | CERAMIC 0402 COG 8.2 pF +/- 0.25 pF |  |
| C240 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C241 | 22567-103 |  | CERAMIC 0402 X7R .01 uF |  |
| C242 | 22565-390 |  | CERAMIC 0402 COG 39 pF |  |
| C244 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C245 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C246 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C247 | 22571-3R3 |  | CERAMIC 0402 COG 3.3 pF+/-.25 pF |  |
| C248 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C249 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C256 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C257 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C267 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C268 | SS-25131 |  | TANTALUM A-CASE 2.2 uF 10% 10V |  |
| C269 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C270 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C271 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C275 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C276 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C277 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C278 | 22564-104 |  | CERAMIC 0603 X7R 0.1 uF |  |
| C279 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C284 | 22565-910 |  | CERAMIC 0402 COG 91 pF |  |
| C285 | 22571-2R2 |  | CERAMIC 0402 COG 2.2 pF +/-.25 pF |  |
| C286 | 22571-3R3 |  | CERAMIC 0402 COG 3.3 pF +/-.25 pF |  |
| C287 | 22565-121 |  | CERAMIC 0402 COG 120 pF |  |
| C288 | 22579-150-G-50 |  | CERAMIC 0603 COH 15 pF |  |
| C289 | 22565-910 |  | CERAMIC 0402 COG 91 pF |  |
| C290 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C292 | SS-25122-475-10 |  | TANTALUM A-CASE 4.7 uF 10% 10V |  |
| C293 | 22565-120 |  | CERAMIC 0402 COG 12 pF |  |
| C295 | 22567-103 |  | CERAMIC 0402 X7R .01uF |  |
| C297 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C298 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C299 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C300 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C301 | 22572-102 |  | CERAMIC 0402 X7R 1000 pF 5% 50V |  |
| C302 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C303 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C304 | 22584-224 |  | CERAMIC 0603 X7R .22 uF |  |
| C305 | 22584-224 |  | CERAMIC 0603 X7R .22 uF |  |
| C306 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C307 | 22583-333 |  | CERAMIC 0603 X7R .033 uF 10% 25V |  |
| C308 | 22583-333 |  | CERAMIC 0603 X7R .033 uF 10% 25V |  |
| C309 | 22583-333 |  | CERAMIC 0603 X7R .033 uF 10% 25V |  |
| C310 | 22572-471 |  | CERAMIC 0402 X7R 470 pF 5% 50V |  |
| C311 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C312 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C313 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| C314 | 22564-104 |  | CERAMIC 0603 X7R .1 uF |  |
| DIODES |  |  |  |  |
| CR1 | SS-36034 |  | MMBD701 (SH)HOT CARRIER |  |
| CR2 | SS-36034 |  | MMBD701 (SH)HOT CARRIER |  |
| CR3 | 29050 |  | VARACTOR SMV1236-004 SOT |  |
| CR4 | 29050 |  | VARACTOR SMV1236-004 SOT |  |
| CR5 | 29043 |  | PIN DUAL SMP1304-004 |  |
| CR6 | 29043 |  | PIN DUAL SMP1304-004 |  |
| CR7 | 36047 |  | PIN MA4P7001F |  |
| CR8 | 36047 |  | PIN MA4P7001F |  |
| CR9 | 36047 |  | PIN MA4P7001F |  |
| CR10 | 36047 |  | PIN MA4P7001F |  |
| CR11 | SS-35616 |  | SCHOTT HSMS-2800 |  |
| CR12 | SS-35616 |  | SCHOTT HSMS-2800 |  |
| CR13 | 36047 |  | PIN MA4P7001F |  |
| CR14 | 36047 |  | PIN MA4P7001F |  |
| CR15 | SS-29031 |  | BBY31 VARACTOR |  |
| CR16 | SS-29029 |  | BBY40 VARACTOR |  |
| CR17 | 29044 |  | VARACTOR SMV1213-001 |  |
| CR18 | SS-29029 |  | BBY40 VARACTOR |  |
| CR19 | 29054 |  | VARACTOR SMV1493-011 SOD |  |
| CR20 | 29036 |  | VARACTOR SMV1207-001 |  |
| CR21 | 29036 |  | VARACTOR SMV1207-001 |  |
| CR22 | SS-29029 |  | BBY40 VARACTOR |  |
| CR23 | SS-35620 |  | BAR43S SCHOTT DUAL (DA5) |  |
| CR24 | SS-35620 |  | BAR43S SCHOTT DUAL (DA5) |  |
| CR25 | SS-35620 |  | BAR43S SCHOTT DUAL (DA5) |  |
| CR26 | 29037 |  | VARACTOR SMV1236-011 |  |
| CR27 | 29050 |  | VARACTOR SMV1236-004 SOT |  |
| CR28 | 29050 |  | VARACTOR SMV1236-004 SOT |  |
| CR35 | SS-35620 |  | BAR43S SCHOTT DUAL (DA5) |  |
| FILTERS |  |  |  |  |
| FL1 | 1600471-1 |  | CRYSTAL 45 MHz |  |
| FL2 | 37107 |  | CERAMIC 455KHz 25KHz BW |  |
| FL3 | 37121 |  | CERAMIC 455KHz 15KHz BW |  |
| FL4 | 1600479-1 |  | CRYSTAL 45 MHz 2 POLE |  |
| SOCKET CONNECTORS |  |  |  |  |
| J1 | 61100-32 |  | HDR 6mm STKG 32-PIN SMT |  |
| J2 | 60165 |  | RF OSMT/SSMT RCPT SMT |  |
| J3 | 60165 |  | RF OSMT/SSMT RCPT SMT |  |
| J4 | 60165 |  | RF OSMT/SSMT RCPT SMT |  |
| INDUCTORS |  |  |  |  |
| L1 | 43103-560-2 |  | CERAMIC 0805 WW 56 nH 2% |  |
| L2 | 43103-470-2 |  | CERAMIC 0805 WW 47 nH 2% |  |
| L3 | 43103-560-2 |  | CERAMIC 0805 WW 56 nH 2% |  |
| L4 | 43103-101-2 |  | CERAMIC 0805 WW 100 nH 2% |  |
| L5 | 43103-101-2 |  | CERAMIC 0805 WW 100 nH 2% |  |
| L6 | 3400439-5 |  | COIL AIR CORE |  |
| L7 | 43104-R47 |  | CERAMIC 1008 WW 0.47 uH 2% |  |
| L8 | 3400439-5 |  | COIL AIR CORE |  |
| L9 | 43103-560-2 |  | CERAMIC 0805 WW 56 nH 2% |  |
| L10 | 43115-331-5 |  | MLC 0805 330 nH 5% |  |
| L11 | 43104-1R2 |  | CERAMIC 1008 WW 1.2 uH 2% |  |
| L12 | 43109-R68 |  | MLC 0805 SHLD 680 nH 10% |  |
| L13 | 43109-R47 |  | MLC 0805 SHLD 470 nH 10% |  |
| L14 | 3400439-12 |  | COIL AIR CORE |  |
| L15 | 3400439-3 |  | COIL AIR CORE |  |
| L16 | 43104-1R8 |  | CERAMIC 1008 WW 1.8 uH 2% |  |
| L17 | 43103-820-2 |  | CERAMIC 0805 WW 82 nH 2% |  |
| L18 | 43083-6R8 |  | FER 1008 WW 6.8 uH 5% |  |
| L19 | 43107-12R5 |  | RF AIR CORE SMT 12.55 nH 5% |  |
| L20 | 43103-330-2 |  | CERAMIC 0805 WW 33 nH 2% |  |
| L21 | 43103-390-2 |  | CERAMIC 0805 WW 39 nH 2% |  |
| L24 | 43119-1R2 |  | FER 1008 SHLD 1.2 uH 10% |  |
| L25 | 43111-101-5 |  | CERAMIC 0603 WW 100 nH 5% |  |
| L27 | 43103-820-2 |  | CERAMIC 0805 WW 82 nH 2% |  |
| L28 | 43115-121-10 |  | MLC 0805 120 nH 10% |  |
| L29 | 43107-9R85 |  | AIR CORE SMT 9.85 nH |  |
| L30 | 3400439-11 |  | COIL AIR CORE |  |
| L31 | 3400439-10 |  | COIL AIR CORE |  |
| L32 | 43115-8R2-5 |  | MLC 0805 8.2 nH 5% |  |
| L34 | 43115-8R2-5 |  | MLC 0805 8.2 nH 5% |  |
| L35 | 43115-271-10 |  | MLC 0805 270 nH 10% |  |
| L36 | 43103-470-2 |  | CERAMIC 0805 WW 47 nH 2% |  |
| L37 | 43103-820-2 |  | CERAMIC 0805 WW 82 nH 2% |  |
| L38 | 43104-1R2 |  | CERAMIC 1008 WW 1.2 uH 2% |  |
| L39 | 43104-1R2 |  | CERAMIC 1008 WW 1.2 uH 2% |  |
| L40 | 43104-R27 |  | CERAMIC 1008 WW 0.27 uH 2% |  |
| L41 | 43104-1R2 |  | CERAMIC 1008 WW 1.2 uH 2% |  |
| L42 | 3400439-11 |  | COIL AIR CORE |  |
| L43 | 43103-820-2 |  | CERAMIC 0805 WW 82 nH 2% |  |
| L44 | SS-43093-1R5 |  | CERAMIC 1008 WW 1.5 uH 10% |  |
| L45 | 43103-101-2 |  | CERAMIC 0805 WW 100 nH 2% |  |
| L46 | 43103-101-2 |  | CERAMIC 0805 WW 100 nH 2% |  |
| L47 | SS-43093-1R5 |  | CERAMIC 1008 WW 1.5 uH 10% |  |
| L48 | 43115-560-5 |  | MLC 0805 56 nH 5% |  |
| L49 | 43115-680-5 |  | MLC 0805 68 nH 5% |  |
| L50 | 43115-680-5 |  | MLC 0805 68 nH 5% |  |
| L51 | SS-43093-1R5 |  | CERAMIC 1008 WW 1.5 uH 10% |  |
| L52 | 43115-270-5 |  | MLC 0805 27 nH 5% |  |
| L53 | 3400439-1 |  | COIL AIR CORE |  |
| L55 | SS-43093-1R5 |  | CERAMIC 1008 WW 1.5 uH 10% |  |
| L56 | 43103-680-5 |  | CERAMIC 0805 WW 68 nH 5% |  |
| L60 | 43104-1R5 |  | CERAMIC 1008 WW 1.5 uH 2% |  |
| L61 | 43115-391-10 |  | MLC 0805 390 nH 10% |  |
| L62 | 43104-R82 |  | CERAMIC 1008 WW 0.82 uH 2% |  |
| L63 | 43115-151-10 |  | MLC 0805 150 nH 10% |  |
| L64 | 43104-1R0 |  | CERAMIC 1008 WW 1.0 uH 2% |  |
| L65 | 43109-R68 |  | MLC 0805 SHLD 680 nH 10% |  |
| L66 | 43083-1R2 |  | FER 1008 WW 1.2 uH 5% |  |
| TRANSISTORS |  |  |  |  |
| Q1 | 32098 |  | MMBR941 LOW-NOISE NPN |  |
| Q2 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q5 | SS-32089-3906 |  | MMBT3906 PNP |  |
| Q6 | 32635 |  | PTF10027 MOSFET_N-CHAN |  |
| Q7 | 32098 |  | MMBR941 LOW-NOISE NPN |  |
| Q8 | SS-32597 |  | MRF5812 NPN RF |  |
| Q9 | 32635 |  | PTF10027 MOSFET_N-CHAN |  |
| Q10 | 32631 |  | Si2301DS MOSFET_P-CH |  |
| Q11 | 32631 |  | Si2301DS MOSFET_P-CH |  |
| Q12 | 32631 |  | Si2301DS MOSFET_P-CH |  |
| Q13 | 32631 |  | Si2301DS MOSFET_P-CH |  |
| Q14 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q15 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q16 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q17 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q18 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q19 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q21 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q22 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q23 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q24 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q25 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q26 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q27 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q28 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q31 | 32631 |  | Si2301DS MOSFET_P-CH |  |
| Q32 | 32631 |  | Si2301DS MOSFET_P-CH |  |
| Q33 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q34 | SS-32564 |  | BSS123 FET N-CHAN |  |
| Q35 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q36 | 32099 |  | MMBR901 RF AMPLIFIER NPN |  |
| Q46 | SS-32089-3904 |  | MMBT3904 NPN |  |
| RESISTORS |  |  |  |  |
| R1 | 19388-10R0 |  | 0402 FILM 10 OHM 1% 0.06W |  |
| R2 | 19388-10R0 |  | 0402 FILM 10 OHM 1% 0.06W |  |
| R3 | 19388-2671 |  | 0402 FILM 2.67K OHM 1% 0.06W |  |
| R4 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1% 0.06W |  |
| R5 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1% 0.06W |  |
| R6 | 19388-2671 |  | 0402 FILM 2.67K OHM 1% 0.06W |  |
| R7 | 19388-3011 |  | 0402 FILM 3.01K OHM 1% 0.06W |  |
| R8 | 19388-3320 |  | 0402 FILM 332 OHM 1% 0.06W |  |
| R9 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R10 | 19388-3011 |  | 0402 FILM 3.01K OHM 1% 0.06W |  |
| R11 | 19388-3320 |  | 0402 FILM 332 OHM 1% 0.06W |  |
| R12 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1% 0.06W |  |
| R31 | 19388-2003 |  | 0402 FILM 200K OHM 1% 0.06W |  |
| R33 | 19388-1821 |  | 0402 FILM 1.82K OHM 1% 0.06W |  |
| R36 | SS-19360-61R9 |  | 0805 FILM 61.9 OHM 1% 0.06W |  |
| R37 | SS-19360-61R9 |  | 0805 FILM 61.9 OHM 1% 0.06W |  |
| R38 | SS-19360-56R2 |  | 0805 FILM 56.2 OHM 1% 0.06W |  |
| R39 | 19388-2431 |  | 0402 FILM 2.43K OHM 1% 0.06W |  |
| R41 | 19388-6811 |  | 0402 FILM 6.81K OHM 1% 0.06W |  |
| R42 | 19388-1002 |  | 0402 FILM 10K OHM 1% 0.06W |  |
| R43 | 19388-1002 |  | 0402 FILM 10K OHM 1% 0.06W |  |
| R44 | 19388-3010 |  | 0402 FILM 301 OHM 1% 0.06W |  |
| R45 | 19388-3320 |  | 0402 FILM 332 OHM 1% 0.06W |  |
| R46 | 19388-1001 |  | 0402 FILM 1K OHM 1% 0.06W |  |
| R47 | 19388-5620 |  | 0402 FILM 562 OHM 1% 0.06W |  |
| R48 | 19388-3921 |  | 0402 FILM 3.92K OHM 1%.06W |  |
| R49 | 19388-18R2 |  | 0402 FILM 18.2 OHM 1%.06W |  |
| R50 | 19388-22R1 |  | 0402 FILM 22.1 OHM 1%.06W |  |
| R51 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1%.06W |  |
| R52 | 19388-3010 |  | 0402 FILM 301 OHM 1%.06W |  |
| R53 | 19388-5620 |  | 0402 FILM 562 OHM 1%.06W |  |
| R54 | 19388-5620 |  | 0402 FILM 562 OHM 1%.06W |  |
| R55 | 19388-3921 |  | 0402 FILM 3.92K OHM 1%.06W |  |
| R57 | 19388-3920 |  | 0402 FILM 392 OHM 1%.06W |  |
| R58 | 19388-1621 |  | 0402 FILM 1.62K OHM 1%.06W |  |
| R59 | 19388-1621 |  | 0402 FILM 1.62K OHM 1%.06W |  |
| R60 | 19388-6810 |  | 0402 FILM 681 OHM 1%.06W |  |
| R62 | 19388-5620 |  | 0402 FILM 562 OHM 1%.06W |  |
| R63 | 19388-4750 |  | 0402 FILM 475 OHM 1%.06W |  |
| R64 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R66 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1%.06W |  |
| R67 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1%.06W |  |
| R68 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R69 | 19388-1001 |  | 0402 FILM 1K OHM 1%.06W |  |
| R70 | 19388-1001 |  | 0402 FILM 1K OHM 1%.06W |  |
| R71 | 19388-2002 |  | 0402 FILM 20K OHM 1%.06W |  |
| R72 | 19388-1821 |  | 0402 FILM 1.82K OHM 1%.06W |  |
| R73 | 19388-1821 |  | 0402 FILM 1.82K OHM 1%.06W |  |
| R74 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R75 | 19388-2211 |  | 0402 FILM 2.21K OHM 1%.06W |  |
| R76 | SS-19360-43R2 |  | 0805 FILM 43.2 OHM 1%.06W |  |
| R77 | SS-19360-43R2 |  | 0805 FILM 43.2 OHM 1%.06W |  |
| R78 | SS-19360-47R5 |  | 0805 FILM 47.5 OHM 1%.06W |  |
| R79 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R80 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R81 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R82 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R83 | SS-19360-43R2 |  | 0805 FILM 43.2 OHM 1%.06W |  |
| R84 | SS-19360-43R2 |  | 0805 FILM 43.2 OHM 1%.06W |  |
| R85 | SS-19360-47R5 |  | 0805 FILM 47.5 OHM 1%.06W |  |
| R86 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R87 | 19388-10R0 |  | 0402 FILM 10 OHM 1%.06W |  |
| R89 | 19388-1103 |  | 0402 FILM 110K OHM 1%.06W |  |
| R91 | 19388-2003 |  | 0402 FILM 200K OHM 1%.06W |  |
| R92 | 19388-7501 |  | 0402 FILM 7.5K OHM 1%.06W |  |
| R93 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R94 | 19388-1823 |  | 0402 FILM 182K OHM 1%.06W |  |
| R95 | 19388-1502 |  | 0402 FILM 15K OHM 1%.06W |  |
| R96 | 19388-4321 |  | 0402 FILM 4.32K OHM 1%.06W |  |
| R97 | 19388-3921 |  | 0402 FILM 3.92K OHM 1%.06W |  |
| R98 | 19388-2372 |  | 0402 FILM 23.7K OHM 1%.06W |  |
| R99 | 19388-47R5 |  | 0402 FILM 47.5 OHM 1%.06W |  |
| R100 | 19388-47R5 |  | 0402 FILM 47.5 OHM 1%.06W |  |
| R101 | 19388-2211 |  | 0402 FILM 2.21K OHM 1%.06W |  |
| R102 | 19388-1001 |  | 0402 FILM 1K OHM 1%.06W |  |
| R103 | 19388-1820 |  | 0402 FILM 182 OHM 1%.06W |  |
| R104 | 19388-4321 |  | 0402 FILM 4.32K OHM 1%.06W |  |
| R105 | 19388-4751 |  | 0402 FILM 4.75K OHM 1%.06W |  |
| R106 | 19388-22R1 |  | 0402 FILM 22.1 OHM 1%.06W |  |
| R107 | 19388-10R0 |  | 0402 FILM 10 OHM 1%.06W |  |
| R109 | 19388-3320 |  | 0402 FILM 332 OHM 1%.06W |  |
| R110 | 19388-5621 |  | 0402 FILM 5.62K OHM 1%.06W |  |
| R111 | 19388-10R0 |  | 0402 FILM 10 OHM 1%.06W |  |
| R112 | 19388-5110 |  | 0402 FILM 511 OHM 1%.06W |  |
| R113 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R114 | 19388-1004 |  | 0402 FILM 1M OHM 1%.06W |  |
| R115 | 19388-4750 |  | 0402 FILM 475 OHM 1%.06W |  |
| R116 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R117 | 19388-1101 |  | 0402 FILM 1.1K OHM 1%.06W |  |
| R118 | 19388-6810 |  | 0402 FILM 681 Ohm 1%.06w |  |
| R119 | 19388-1000 |  | 0402 FILM 100 Ohm 1%.06w |  |
| R120 | 19388-1301 |  | 0402 FILM 1.3k Ohm 1%.06w |  |
| R121 | 19388-1004 |  | 0402 FILM 1M OHM 1%.06W |  |
| R122 | 19388-1210 |  | 0402 FILM 121 OHM 1%.06W |  |
| R123 | 19388-3572 |  | 0402 FILM 35.7K OHM 1%.06W |  |
| R124 | 19388-47R5 |  | 0402 FILM 47.5 OHM 1%.06W |  |
| R125 | 19388-47R5 |  | 0402 FILM 47.5 OHM 1%.06W |  |
| R126 | 19388-1000 |  | 0402 Film 100 OHM 1% 0.06W |  |
| R127 | 19388-22R1 |  | 0402 FILM 22.1 OHM 1%.06W |  |
| R128 | 19388-47R5 |  | 0402 FILM 47.5 OHM 1%.06W |  |
| R129 | 19388-1004 |  | 0402 FILM 1M OHM 1%.06W |  |
| R130 | 19388-2211 |  | 0402 FILM 2.21K OHM 1%.06W |  |
| R131 | 19388-16R2 |  | 0402 FILM 16.2 OHM 1%.06W |  |
| R132 | 19388-1820 |  | 0402 FILM 182 OHM 1%.06W |  |
| R133 | 19388-2211 |  | 0402 FILM 2.21K OHM 1%.06W |  |
| R134 | 19388-4752 |  | 0402 FILM 47.5K OHM 1%.06W |  |
| R135 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R136 | 19388-1820 |  | 0402 FILM 182 OHM 1%.06W |  |
| R137 | 19388-4321 |  | 0402 FILM 4.32K OHM 1%.06W |  |
| R138 | 19388-16R2 |  | 0402 FILM 16.2 OHM 1%.06W |  |
| R139 | 19388-16R2 |  | 0402 FILM 16.2 OHM 1%.06W |  |
| R140 | 19388-2212 |  | 0402 FILM 22.1K OHM 1%.06W |  |
| R141 | 19388-16R2 |  | 0402 FILM 16.2 OHM 1%.06W |  |
| R142 | 19388-16R2 |  | 0402 FILM 16.2 OHM 1%.06W |  |
| R143 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1%.06W |  |
| R144 | 19388-16R2 |  | 0402 FILM 16.2 OHM 1%.06W |  |
| R145 | 19388-4751 |  | 0402 FILM 4.75K OHM 1%.06W |  |
| R146 | 19388-10R0 |  | 0402 FILM 10 OHM 1%.06W |  |
| R147 | 19388-3011 |  | 0402 FILM 3.01K OHM 1%.06W |  |
| R148 | 19388-2670 |  | 0402 FILM 267 OHM 1%.06W |  |
| R149 | 19388-3320 |  | 0402 FILM 332 OHM 1%.06W |  |
| R150 | 19388-5621 |  | 0402 FILM 5.62K OHM 1%.06W |  |
| R151 | 19388-22R1 |  | 0402 FILM 22.1 OHM 1%.06W |  |
| R152 | 19388-5110 |  | 0402 FILM 511 OHM 1%.06W |  |
| R153 | 19388-22R1 |  | 0402 FILM 22.1 OHM 1%.06W |  |
| R154 | 19388-4750 |  | 0402 FILM 475 OHM 1%.06W |  |
| R155 | 19388-8250 |  | 0402 FILM 825 OHM 1%.06W |  |
| R156 | 19388-1101 |  | 0402 FILM 1.1K OHM 1%.06W |  |
| R157 | 19388-1000 |  | 0402 Film 100 OHM 1% 0.06W |  |
| R158 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R159 | 19388-1210 |  | 0402 FILM 121 OHM 1%.06W |  |
| R160 | 19388-10R0 |  | 0402 FILM 10 OHM 1%.06W |  |
| R161 | 19388-1001 |  | 0402 FILM 1K OHM 1%.06W |  |
| R162 | 19388-1001 |  | 0402 FILM 1K OHM 1%.06W |  |
| R163 | 19388-3572 |  | 0402 FILM 35.7K OHM 1%.06W |  |
| R164 | 19388-2003 |  | 0402 FILM 200K OHM 1%.06W |  |
| R166 | 19388-4322 |  | 0402 FILM 43.2K OHM 1%.06W |  |
| R167 | 19388-3921 |  | 0402 FILM 3.92K OHM 1%.06W |  |
| R169 | 19388-2743 |  | 0402 FILM 274 K OHM 1%.06W |  |
| R171 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R172 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R173 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R175 | 19388-1303 |  | 0402 FILM 130K OHM 1%.06W |  |
| R177 | 19388-4322 |  | 0402 FILM 43.2K OHM 1%.06W |  |
| R178 | 19388-1004 |  | 0402 FILM 1M OHM 1%.06W |  |
| R179 | 19388-2372 |  | 0402 FILM 23.7K OHM 1%.06W |  |
| R180 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R182 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R183 | 19388-1004 |  | 0402 FILM 1M OHM 1%.06W |  |
| R184 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R185 | 19388-4752 |  | 0402 FILM 47.5K OHM 1%.06W |  |
| R186 | 19388-4752 |  | 0402 FILM 47.5K OHM 1%.06W |  |
| R187 | 19388-1003 |  | 0402 FILM 100K OHM 1%.06W |  |
| R188 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R189 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R191 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R192 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R193 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R194 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R195 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R196 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R197 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R198 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R203 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R206 | 19388-1002 |  | 0402 FILM 10K OHM 1%.06W |  |
| R212 | 19388-1822 |  | 0402 FILM 18.2K OHM 1%.06W |  |
| R213 | 19388-1301 |  | 0402 FILM 1.3K OHM 1%.06W |  |
| R214 | 19388-1001 |  | 0402 FILM 1K OHM 1%.06W |  |
| R215 | 19388-10R0 |  | 0402 FILM 10 OHM 1%.06W |  |
| R217 | 19388-1212 |  | 0402 FILM 12.1K OHM 1%.06W |  |
| R218 | 19388-2671 |  | 0402 FILM 2.67K OHM 1%.06W |  |
| R219 | 19388-1212 |  | 0402 FILM 12.1K OHM 1%.06W |  |
| R220 | 19388-3011 |  | 0402 FILM 3.01K OHM 1%.06W |  |
| R222 | 19388-1000 |  | 0402 FILM 100 OHM 1%.06W |  |
| R224 | 19388-1001 |  | 0402 FILM 1K OHM 1%.06W |  |
| R225 | 19388-6810 |  | 0402 FILM 681 OHM 1%.06W |  |
| R227 | 19388-1211 |  | 0402 FILM 1.21K OHM 1%.06W |  |
| R229 | 19388-2211 |  | 0402 FILM 2.21K OHM 1%.06W |  |
| R230 | 19388-3320 |  | 0402 FILM 332 OHM 1%.06W |  |
| R236 | 19388-1331 |  | 0402 FILM 1.33K OHM 1%.06W |  |
| R237 | 19388-1740 |  | 0402 FILM 174 OHM 1%.06W |  |
| R238 | 19388-1501 |  | 0402 FILM 1.5K OHM 1%.06W |  |
| R240 | 19388-4750 |  | 0402 FILM 475 OHM 1%.06W |  |
| R241 | 19388-4750 |  | 0402 FILM 475 OHM 1%.06W |  |
| R250 | 19388-1501 |  | 0402 FILM 1.5K OHM 1%.06W |  |
| R251 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1%.06W |  |
| R252 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R253 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R254 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R255 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R256 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R257 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R258 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R259 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R260 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R261 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R262 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R263 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R264 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R265 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R266 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R267 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R269 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R270 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R272 | 19388-1003 |  | 0402 FILM 100K OHM 1% 0.06W |  |
| R273 | 19388-1000 |  | 0402 FILM 100 OHM 1%.06W |  |
| R274 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R275 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R276 | 19388-51R1 |  | 0402 FILM 51.1 OHM 1%.06W |  |
| R277 | SS-19360-56R2 |  | 0805 FILM 56.2 OHM 1%.06W |  |
| R278 | SS-19360-56R2 |  | 0805 FILM 56.2 OHM 1%.06W |  |
| R279 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R280 | 19388-2002 |  | 0402 FILM 20K OHM 1%.06W |  |
| R281 | 19388-2002 |  | 0402 FILM 20K OHM 1%.06W |  |
| R282 | 19388-68R1 |  | 0402 FILM 68.1 OHM 1%.06W |  |
| R283 | 19388-1500 |  | 0402 FILM 150 OHM 1%.06W |  |
| R284 | 19388-68R1 |  | 0402 FILM 68.1 OHM 1%.06W |  |
| R287 | 19388-1004 |  | 0402 FILM 1M OHM 1%.06W |  |
| R288 | 19388-1004 |  | 0402 FILM 1M OHM 1%.06W |  |
| R289 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R290 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R291 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R292 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R293 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R294 | 19388-1000 |  | 0402 FILM 100 OHM 1% 0.06W |  |
| R295 | 19388-8662 |  | 0402 FILM 86.6K OHM 1%.06W |  |
| R296 | 19388-3323 |  | 0402 FILM 332K OHM 1%.06W |  |
| R297 | 19388-10R0 |  | 0402 FILM 10 OHM 1% 0.06W |  |
| R298 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R299 | SS-19350-0402 |  | 0402 JUMPER ZERO-OHM |  |
| R300 | SS-19360-1211 |  | 0805 FILM 1.21K OHM 1%.06W |  |
| R301 | SS-19360-1211 |  | 0805 FILM 1.21K OHM 1%.06W |  |
| REFERENCE MATERIAL |  |  |  |  |
| REF | 4101257-ASSY |  | ASSY DWG TRANSCEIVER |  |
| REF | 4200675 |  | SCHEM DIAG TRANSCEIVER |  |
| TRANSFORMERS |  |  |  |  |
| T1 | 1600462-1 |  | XFRMR ASSY RX 136-174 MHz |  |
| T2 | 1600463-1 |  | XFRMR ASSY RX 30-88 MHz |  |
| T3 | 1600507-1 |  | XFRMR ASSY 136-174 MHz |  |
| T4 | 1600508-1 |  | XFRMR ASSY 136-174 MHz |  |
| INTEGRATED CIRCUITS |  |  |  |  |
| U1 | 84586 |  | MIXER EMRS-1A SMT |  |
| U2 | 365455 |  | LMC7111 OP AMP SMT |  |
| U3 | 365460 |  | LP2980A-3.3 LDO REG SMT |  |
| U4 | 365460 |  | LP2980A-3.3 LDO REG SMT |  |
| U6 | 365455 |  | LMC7111 OP AMP SMT |  |
| U11 | 365437 |  | LM50B CENT TEMP SENSOR |  |
| U12 | 1600458-1 |  | COUPLER ASSY 136-174_MHZ |  |
| U13 | 365438 |  | LMC7101A OP AMP LOW-POWER |  |
| U14 | 365413 |  | LP2980A-5.0 LDO REG SMT |  |
| U15 | 365440 |  | 24C64 8K X 8 SERIAL EEPROM |  |
| U16 | 365413 |  | LP2980A-5.0 LDO REG SMT |  |
| U17 | 365499 |  | MIC5205 VR ADJ 1 SOT-23-5 |  |
| U18 | SS-365293 |  | LM2902 QUAD OP-AMP SMT |  |
| U19 | 365423 |  | SA7025DK SYNTH FRACT-N 1GHz |  |
| U20 | 365499 |  | MIC5205 VR ADJ 1 SOT-23-5 |  |
| U21 | 365621 |  | MIC4416 DRVR MOSFET |  |
| U22 | 365437 |  | LM50B CENT TEMP SENSOR |  |
| U24 | 365459-595 |  | 74HC595 SHFT REG 8BIT TSSOP |  |
| U27 | 365455 |  | LMC7111 OP AMP SMT |  |
| U28 | 365513 |  | AD607A 3V RX IF SUBSYS SSOP |  |
| U29 | 365701 |  | OPA343NA SINGLE OP-AMP |  |
| U31 | 365588 |  | MAX525B SER QUAD DAC 12-BIT |  |
| U32 | 365591 |  | MAX873B PREC VLTG_REF 2.5V |  |
| U33 | 365590 |  | MAX534B DAC QUAD 8-BIT SER |  |
| U34 | 365590 |  | MAX534B DAC QUAD 8-BIT SER |  |
| U35 | 365078-125 |  | 74HCT125 QUAD 3-ST DRIVER |  |
| CRYSTAL OSCILLATOR |  |  |  |  |
| Y1 | 1600407-1 |  | CRYSTAL QUARTZ 44.545 MHz |  |
| Y2 | 1600406-1 |  | CRYSTAL QUARTZ 12.8 MHz |  |

This page left blank intentionally

Figure 7-29. Control Circuit Card (4101258-501), PRIMARY SIDE

Figure 7-30.  Control Circuit Card (4101258-501), SECONDARY SIDE

Table 7-8.  Control CCA (4101258-501) Bill of Materials

| Reference Designator | Part Number | Control CCA Component Description |
| --- | --- | --- |
| CAPACITORS |  |  |
| C1 | 22551-105 | CERAMIC 1210 Z5U 1.0uF 20% 25V |
| C2 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C3 | 22572-392 | CERAMIC 0402 X7R 3900 pF 5% 50V |
| C4 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C5 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C6 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C7 | 25146-226-10 | TANTALUM C-CASE 22 uF 10% 10V |
| C8 | 25146-475-20 | TANTALUM B-CASE 4.7 uF 10% 20V |
| C9 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C10 | 22565-680 | CERAMIC 0402 COG 68 pF 5% |
| C11 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C12 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C13 | 22564-104 | CERAMIC R 0603 X7R 0.1 uF 10% |
| C14 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C15 | 22564-333 | CERAMIC 0603 X7R 0.033 uF 10% 16V |
| C16 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C17 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C18 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C19 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C20 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C21 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C22 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C23 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C24 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C25 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C26 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C27 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C28 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C29 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C30 | 22551-105 | CERAMIC 1210 Z5U 1.0 uF 20% 25V |
| C31 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C32 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C33 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C34 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C35 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C36 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C37 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C38 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C39 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C40 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C41 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C42 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C43 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C44 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C45 | 22565-220 | CERAMIC 0402 COG 22 pF 5% |
| C46 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C47 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C48 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C49 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C50 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C51 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C52 | 22572-102 | CERAMIC 0402 X7R 1000 pF 5% 50V |
| C53 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C54 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C55 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C56 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C57 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C58 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C59 | 22562-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C60 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C61 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C62 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C63 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C65 | 25146-226-10 | TANTALUM C-CASE 22 uF 10% 10V |
| C66 | 25146-475-20 | TANTALUM B-CASE 4.7 uF 10% 20V |
| C67 | 25146-475-20 | TANTALUM B-CASE 4.7 uF 10% 20V |
| C68 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C69 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C70 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C71 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C72 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C73 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C74 | 25146-107-6 | TANTALUM R-CASE 100 uF 10% 6V |
| C75 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C76 | 22565-470 | CERAMIC 0402 COG 47 pF 5% |
| C77 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C78 | 25146-226-10 | TANTALUM C-CASE 22 uF 10% 10V |
| C79 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C80 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C81 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C82 | 22572-681 | CERAMIC 0402 X7R 680pF 5% 50V |
| C83 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C84 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C85 | 22567-682 | CERAMIC 0402 X7R 6800 pF 10% 16V |
| C86 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C87 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C88 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C89 | 22564-473 | CERAMIC 0603 X7R 0.047 uF 10% 16V |
| C90 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C91 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C92 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C93 | 25146-226-10 | TANTALUM C-CASE 22 uF 10% 10V |
| C94 | 25146-226-10 | TANTALUM C-CASE 22 uF 10% 10V |
| C95 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C96 | 22551-105 | CERAMIC 1210 Z5U 1.0 uF 20% 25V |
| C97 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C98 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C99 | 22551-105 | CERAMIC 1210 Z5U 1.0 uF 20% 25V |
| C100 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C101 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C102 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C103 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C104 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C105 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C106 | 22565-680 | CERAMIC 0402 COG 68 pF 5% |
| C107 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C108 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C109 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C110 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C111 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C112 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C113 | 25146-686-10 | TANTALUM R-CASE 68 uF 10% 10V |
| C114 | 25146-107-6 | TANTALUM R-CASE 100 uF 10% 6V |
| C115 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C116 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C117 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C118 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C119 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C120 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C121 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C122 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C123 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C124 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C125 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C126 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C127 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C128 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C129 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C132 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C133 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C134 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C135 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C136 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C137 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C138 | 25146-226-10 | TANTALUM C-CASE 22 uF 10% 10V |
| C139 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C140 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C141 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C142 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C143 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C144 | 25146-475-20 | TANTALUM B-CASE 4.7 uF 10% 20V |
| C145 | 25146-226-20 | TANTALUM R-CASE 22 uF 10% 20V |
| C146 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C147 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C148 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C149 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C150 | 22551-105 | CERAMIC 1210 Z5U 1.0 uF 20% 25V |
| C151 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C152 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C153 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C154 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C155 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C156 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C157 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C158 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C159 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C160 | 22572-102 | CERAMIC 0402 X7R 1000 pF 5% 50V |
| C161 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C162 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C163 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C164 | 25146-475-20 | TANTALUM B-CASE 4.7 uF 10% 20V |
| C165 | 22565-101 | CERAMIC 0402 COG 100 pF 5% |
| C166 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C167 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| DIODES |  |  |
| D1 | 41020 | LAMP LED GREEN/RED SMT |
| D2 | RCL-999389 | BAR43C SCHOTTKY |
| D3 | 35628 | SCHOTT CMDSH2-3 SMT |
| D4 | 35624 | SCHOTT DUAL CMPSH-3A SMT |
| D5 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| D6 | 35628 | SCHOTT CMDSH2-3 SMT |
| D7 | 35628 | SCHOTT CMDSH2-3 SMT |
| D8 | 35625 | RECT SCHOTT CMSH2-40 SMT |
| D9 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| D10 | 36005 | RECT SCHOTT ZHCS1000 SMT |
| D11 | RCL-999389 | BAR43C SCHOTTKY |
| D12 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| D13 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| D14 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| D15 | 35628 | SCHOTT CMDSH2-3 SMT |
| D16 | 35628 | SCHOTT CMDSH2-3 SMT |
| D17 | 35628 | SCHOTT CMDSH2-3 SMT |
| D18 | 36005 | RECT SCHOTT ZHCS1000 SMT |
| D19 | 35628 | SCHOTT CMDSH2-3 SMT |
| D20 | 35628 | SCHOTT CMDSH2-3 SMT |
| D21 | 35628 | SCHOTT CMDSH2-3 SMT |
| D22 | 35628 | SCHOTT CMDSH2-3 SMT |
| D23 | 35628 | SCHOTT CMDSH2-3 SMT |
| D24 | 35628 | SCHOTT CMDSH2-3 SMT |
| D25 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| D26 | 33614 | ZENER BZX84C12 12V SMT |
| D27 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| D28 | 35628 | SCHOTT CMDSH2-3 SMT |
| D29 | 35628 | SCHOTT CMDSH2-3 SMT |
| D30 | SS-35620 | BAR43S SCHOTT DUAL (DA5) |
| FILTERS |  |  |
| FL1 | 45232 | BEAD CHIP FERRITE 3216 |
| FL2 | 45232 | BEAD CHIP FERRITE 3216 |
| SOCKET CONNECTORS |  |  |
| J1 | 610348-9-2 | HDR SHRD 18 PIN .050" SMT |
| J2 | 610349-9-2 | RCP SHRD 18 PIN .050" SMT |
| J3 | 61101-32 | RCPT 6mm STKG 32-PIN SMT |
| INDUCTORS |  |  |
| L1 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| L2 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| L3 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| L4 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| L5 | 43083-1R2 | FER 1008 WW 1.2 uH 5% |
| L6 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| L7 | 43083-1R2 | FER 1008 WW 1.2 uH 5% |
| L8 | 43123-330 | PWR 1608 WW 33 uH 20% |
| L9 | SS-43083-100 | FER 1008 WW 10 uH 5% |
| L10 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| L11 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| L12 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| PLUG CONNECTORS |  |  |
| P1 | 61106-4 | HDR R-ANG 4-POS SMT |
| P2 | 61125-20 | FFC R-ANG 0.8mm 20-POS |
| TRANSISTORS |  |  |
| Q1 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q2 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q3 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q4 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q5 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q6 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q7 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q8 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q9 | 32603 | BSS84 DMOS FET P-CHANNEL |
| Q10 | SS-32089-3906 | MMBT3906 PNP |
| Q11 | SS-32550 | 2N7002 MOSFET N-CHANNNEL |
| Q12 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q13 | 32603 | BSS84 DMOS FET P-CHANNEL |
| Q14 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| Q15 | SS-32564 | BSS123 FET N-CHANNEL |
| Q16 | SS-32550 | 2N7002 MOSFET N-CHANNEL |
| RESISTORS |  |  |
| R1 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R2 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R3 | 19388-2213 | 0402 FILM 221K OHM 1% 0.06W |
| R4 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R5 | 19388-1502 | 0402 FILM 15K OHM 1% 0.06W |
| R6 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R7 | 19388-5621 | 0402 FILM 5.62K OHM 1% 0.06W |
| R8 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R9 | 19388-4751 | 0402 FILM 4.75K OHM 1% 0.06W |
| R10 | 19388-4750 | 0402 FILM 475 OHM 1% 0.06W |
| R11 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R12 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R13 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R14 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R15 | 19388-2211 | 0402 FILM 2.21K OHM 1% 0.06W |
| R16 | 19388-2212 | 0402 FILM 22.1K OHM 1% 0.06W |
| R17 | 19388-1502 | 0402 FILM 15K OHM 1% 0.06W |
| R18 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R19 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R20 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R21 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R22 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R23 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R24 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R25 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R26 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R27 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R28 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R29 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R30 | 19388-1502 | 0402 FILM 15K OHM 1% 0.06W |
| R31 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R32 | SS-19350-0402 | 0402 JUMPER ZERO-OHM |
| R33 | SS-19350-0402 | 0402 JUMPER ZERO-OHM |
| R34 | 19388-47R5 | 0402 FILM 47.5 OHM 1% 0.06W |
| R35 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R36 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R37 | 19388-1004 | 0402 FILM 1M OHM 1% 0.06W |
| R38 | 19388-2212 | 0402 FILM 22.1K OHM 1% 0.06W |
| R39 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R40 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R41 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R42 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R43 | 19388-2212 | 0402 FILM 22.1K OHM 1% 0.06W |
| R44 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R45 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R46 | 19388-47R5 | 0402 FILM 47.5 OHM 1% 0.06W |
| R47 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R48 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R49 | 19388-4752 | 0402 FILM 47.5K OHM 1% 0.06W |
| R50 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R51 | 19388-1823 | 0402 FILM 182K OHM 1% 0.06W |
| R52 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R53 | 19388-2213 | 0402 FILM 221K OHM 1% 0.06W |
| R54 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R55 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R56 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R57 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R58 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R59 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R60 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R61 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R62 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R63 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R64 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R65 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R66 | 19388-3322 | 0402 FILM 33.2K OHM 1% 0.06W |
| R67 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R68 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R69 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R70 | 19348-4R7 | 1206 FILM 4.7 OHM 5% 1/8W |
| R71 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R72 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R73 | 19388-47R5 | 0402 FILM 47.5 OHM 1% 0.06W |
| R74 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R75 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R76 | 19348-4R7 | 1206 FILM 4.7 OHM 5% 1/8W |
| R77 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R78 | 19388-2743 | 0402 FILM 274 K OHM 1% 0.06W |
| R79 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R80 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R81 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R82 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R83 | 19388-5623 | 0402 FILM 562K OHM 1% 0.06W |
| R84 | 19388-5623 | 0402 FILM 562K OHM 1% 0.06W |
| R85 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R86 | 19388-5111 | 0402 FILM 5.11K OHM 1% 0.06W |
| R87 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R88 | 19388-56R2 | 0402 FILM 56.2 OHM 1% 0.06W |
| R89 | 19388-56R2 | 0402 FILM 56.2 OHM 1% 0.06W |
| R90 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R91 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R92 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R93 | 19388-5620 | 0402 FILM 562 OHM 1% 0.06W |
| R94 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R95 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R96 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R97 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R98 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R99 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R100 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R101 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R102 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R104 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R106 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R107 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R108 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R109 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R110 | 19388-56R2 | 0402 FILM 56.2 OHM 1% 0.06W |
| R111 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R112 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R113 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R114 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R115 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R116 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R117 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R118 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R119 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R120 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R121 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R122 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R123 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R124 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R125 | 19388-47R5 | 0402 FILM 47.5 OHM 1% 0.06W |
| R126 | 19388-2211 | 0402 FILM 2.21K OHM 1% 0.06W |
| R127 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R128 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R129 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R130 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R131 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R132 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R133 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R134 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R135 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R136 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R137 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R138 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R139 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R140 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R141 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R142 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R143 | 19388-2211 | 0402 FILM 2.21K OHM 1% 0.06W |
| R144 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R145 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R146 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R147 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R148 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R149 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R150 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R151 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R152 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R153 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R154 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R155 | 19388-4751 | 0402 FILM 4.75K OHM 1% 0.06W |
| R156 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R157 | 19388-8251 | 0402 FILM 8.25K OHM 1% 0.06W |
| R158 | 19388-2212 | 0402 FILM 22.1K OHM 1% 0.06W |
| R159 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R160 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R161 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R162 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R163 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R164 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R165 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R166 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R167 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R168 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R169 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R170 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R171 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R172 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R173 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R174 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R175 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R176 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R177 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R178 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R179 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R180 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R181 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R182 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R183 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R184 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R185 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R186 | 19388-47R5 | 0402 FILM 47.5 OHM 1% 0.06W |
| R188 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R189 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R190 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R191 | 19388-4751 | 0402 FILM 4.75K OHM 1% 0.06W |
| R193 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R194 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R195 | SS-19350-0402 | 0402 JUMPER ZERO-OHM |
| R196 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R197 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R198 | 19388-2211 | 0402 FILM 2.21K OHM 1% 0.06W |
| R199 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R200 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R201 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R202 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R203 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R204 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R205 | 19388-4751 | 0402 FILM 4.75K OHM 1% 0.06W |
| R206 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R207 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R208 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R209 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R210 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R211 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R212 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R213 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R214 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R215 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R216 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R217 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R218 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R219 | 19388-1002 | 0402 FILM 10K OHM 1% 0.06W |
| R220 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R221 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R222 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R223 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R224 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R225 | 19388-1503 | 0402 FILM 150K OHM 1% 0.06W |
| R226 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| REFERENCE MATERIAL |  |  |
| REF | 1700302-1 | CONTROL LOADING SOFTWARE RACAL |
| REF | 4101258-ASSY | ASSEMBLY DRAWING CCA CONTROL |
| REF | 4200676 | SCHEMATIC DIAGRAM CONTROL |
| THERMISTOR |  |  |
| TH1 | 19386 | THRMSTR miniSMD050-2_CKT PROT |
| INTEGRATED CIRCUITS |  |  |
| U1 | 365438 | LMC7101A OP AMP LOW-POWER |
| U2 | 365413 | LP2980A-5.0 LDO REG SMT |
| U3 | 365458 | TLV320 AUDIO PROCESSOR |
| U4 | 365436 | LM4861 AMP AUDIO 0.5-WATT |
| U5 | 365584 | MAX825T RESET GENERATOR |
| U6 | 365585 | TC55V1001 SRAM 128K x 8 |
| U7 | 365690 | DSP56309 DIG SIG PROC 80MIP |
| U8 | 365595-14 | 74LVC14 HEX INV SCHM TRIG |
| U9 | 365692 | TE28F800B3 ROM FLASH 8Mbit |
| U10 | 365582 | H8/3044 uCONTROLER |
| U11 | 365460 | LP2980A-3.3 LDO REG SMT |
| U12 | 365569 | MIC5205 VR 3.3V 1% SOT-23-5 |
| U13 | 365421 | LTC1434IGN 450MA DC/DC CONV |
| U15 | 365697 | LT1118CST-5 VOLT REG 5V |
| U16 | 365436 | LM4861 AMP AUDIO 0.5-WATT |
| U17 | 365435 | EPF8282VTC100 PLD |
| U18 | 365698 | 74HC4053 TRIP 2-CHA ANA MUX |
| U19 | 365583 | MAX213E RS-232 XCVR 5V |
| U20 | 32610 | XSTR Si4953DY DUAL MOSFET P-CH |
| U21 | 365696 | LP2980A-3.6 LDO REG SMT |
| U22 | 365413 | LP2980A-5.0 LDO REG SMT |
| U23 | 32610 | XSTR Si4953DY DUAL MOSFET P-CH |
| U24 | 365592 | MAX872 PREC VLTG REF 2.5V |
| U25 | 365589 | MAX1247A ADC 4-CH_12-BIT |
| U26 | 32610 | XSTR Si4953DY DUAL MOSFET P-CH |
| CRYSTAL OSCILLATOR |  |  |
| X1 | 37044 | TCXO 12.288 MHz +/-2.5PPM 3V |

Figure 7-31.  Keypad Circuit Card (3100618-501) - PRIMARY SIDE

Figure 7-32.  Keypad Circuit Card (3100618-501) - SECONDARY SIDE

Table 7-9.  Keypad CCA (3100618-501) Bill of Materials

| Reference Designator | Part Number | Keypad CCA Component Description |
| --- | --- | --- |
| CAPACITORS |  |  |
| C2 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C3 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C4 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C5 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C6 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| C7 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C8 | 25146-225-25B | TANTALUM B-CASE 2.2 uF 10% 25V |
| C9 | 22564-104 | CERAMIC 0603 X7R 0.1 uF 10% |
| C10 | 22567-103 | CERAMIC 0402 X7R 0.01 uF 10% |
| DIODES |  |  |
| D1 | 35624 | SCHOTT DUAL CMPSH-3A SMT |
| D2 | 35624 | SCHOTT DUAL CMPSH-3A SMT |
| D3 | 35628 | SCHOTT CMDSH2-3 SMT |
| D4 | 35628 | SCHOTT CMDSH2-3 SMT |
| D5 | 35628 | SCHOTT CMDSH2-3 SMT |
| D6 | 35628 | SCHOTT CMDSH2-3 SMT |
| D7 | 35628 | SCHOTT CMDSH2-3 SMT |
| D8 | 35628 | SCHOTT CMDSH2-3 SMT |
| D9 | 35628 | SCHOTT CMDSH2-3 SMT |
| D13 | 35628 | SCHOTT CMDSH2-3 SMT |
| D14 | 35628 | SCHOTT CMDSH2-3 SMT |
| D15 | 35628 | SCHOTT CMDSH2-3 SMT |
| D16 | 35628 | SCHOTT CMDSH2-3 SMT |
| LIGHT EMMITTING DIODES |  |  |
| DS1 | 41081 | LAMP LED GREEN SMT |
| DS2 | 41081 | LAMP LED GREEN SMT |
| DS3 | 41081 | LAMP LED GREEN SMT |
| DS4 | 41081 | LAMP LED GREEN SMT |
| DS5 | 41081 | LAMP LED GREEN SMT |
| DS6 | 41081 | LAMP LED GREEN SMT |
| DS7 | 41081 | LAMP LED GREEN SMT |
| DS8 | 41081 | LAMP LED GREEN SMT |
| DS9 | 41081 | LAMP LED GREEN SMT |
| DS10 | 41081 | LAMP LED GREEN SMT |
| DS11 | 41081 | LAMP LED GREEN SMT |
| DS12 | 41081 | LAMP LED GREEN SMT |
| DS13 | 41081 | LAMP LED GREEN SMT |
| DS14 | 41081 | LAMP LED GREEN SMT |
| DS15 | 41081 | LAMP LED GREEN SMT |
| DS16 | 41081 | LAMP LED GREEN SMT |
| SOCKET CONNECTOR |  |  |
| J1 | 610349-9-2 | CONN RCP SHRD 18 PIN .050" SMT |
| INDUCTOR |  |  |
| L1 | 45231-Y601B | BEAD FERRITE MMZ 1608 |
| PLUG CONNECTORS |  |  |
| P1 | 61125-20 | FFC R-ANG 0.8mm 20-POS |
| P2 | 61125-11 | FFC R-ANG 0.8mm_11-POS |
| P3 | 61125-11 | FFC R-ANG 0.8mm_11-POS |
| TRANSISTORS |  |  |
| Q5 | SS-32550 | 2N7002 MOSFET N-CH |
| Q6 | SS-32089-3904 | MMBT3904 NPN |
| Q7 | SS-32089-3904 | MMBT3904 NPN |
| Q8 | SS-32089-3904 | MMBT3904 NPN |
| Q9 | SS-32089-3904 | MMBT3904 NPN |
| Q10 | SS-32089-3904 | MMBT3904 NPN |
| Q11 | SS-32089-3904 | MMBT3904 NPN |
| Q13 | 32603 | BSS84 DMOS FET P-CHAN |
| Q17 | SS-32089-3904 | MMBT3904 NPN |
| RESISTORS |  |  |
| R1 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R2 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R7 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R8 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R9 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R10 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R12 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R14 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R15 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R16 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R17 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R18 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R19 | 19388-1000 | 0402 FILM 100 HM 1% 0.06W |
| R23 | 19388-1501 | 0402 FILM 1.5K OHM 1% 0.06W |
| R25 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R27 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R28 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R29 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R30 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R31 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R36 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R37 | 19388-1003 | 0402 FILM 100K OHM 1% 0.06W |
| R39 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R41 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R42 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R43 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R44 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R47 | 19388-6810 | 0402 FILM 681 OHM 1% 0.06W |
| R48 | 19388-3321 | 0402 FILM 3.32K OHM 1% 0.06W |
| R49 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R50 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R52 | 19388-1000 | 0402 FILM 100 OHM 1% 0.06W |
| R57 | 19388-10R0 | 0402 FILM 10 OHM 1% 0.06W |
| R58 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R59 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R60 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R61 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R62 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R63 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R64 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| R65 | 19388-1001 | 0402 FILM 1K OHM 1% 0.06W |
| REFERENCE MATERIAL |  |  |
| REF | 3100618-ASSY | ASSEMBLY DRAWING KEYPAD BOARD |
| REF | 4200649 | SCHEMATIC DIAGRAM KEYPAD |
| INTEGRATED CIRCUITS |  |  |
| U1 | 365681 | AT90S4414 8-BIT AVR uCONTROLLER |
| U2 | 365460 | LP2980A-3.3 LDO REG SMT |

APPENDIX A.  MENU STRUCTURE

SCAN

SQUELCH

MONITOR

INDIVIDUAL

CALL ID

EMERGENCY

ALERTS

ZONE

PRIORITY 1

SCAN

PRIORITY 2

SCAN

GLOBAL

SEARCH

SCAN

RECEIVED

CHANNEL

ADD/REMVE

CHANNEL

DISPLAY

TX/RX

SQUELCH

BAR

GRAPH

SET

RADIO

INDIVIDUAL CALL

SILENT

MODE

ALERT

MODE

EMERGENCY

CALL

NO TX PROGRAMMED

(RX ONLY)

KEY LOSS

TX

INHIBIT

TX TIMEOUT

RADIO TEST FAIL

WARNING LOW BATTERY

RADIO

LOCKED

OVERALL RADIO MENU

RADIO MMI PROGRAMMING MENU

ENCRYPTION

(OPTIONAL

PASSWORD)

SCAN

SELECT

HOME

PROGRAM

(OPTIONAL

PASSWORD)

SCAN

MODE

PRIORITY

ZONE

GLOBAL

CHANNEL

SCAN

ZEROIZE

DEFAULT

KEY

LISTS

PASSWRD

MESSAGE

COVERT

(OPTIONAL

PASSWORD)

SPEAKER

ALARMS

KEY BEEPS

LED

DEFAULT

KEY SELECT

SCLIST

PRIORITY

CHANNEL 2

SEARCH

BANK

PHONE

ID CALL

TONE

TALKGR'UP

TALKAR'ND

BKLITE

PRIORITY

CHANNEL 1

ZONE

SCAN

"PROGRM" (PROGRAM) GLOBAL MENU

GLOBAL

CHANNEL

LISTS

SCAN

BACKLIGHT

TIME OUT

TIMER

SET

SCAN LIST

SET DELAY

REPEATER

DELAY

BRIGHT

DIM

BACKLIGHT

DELAY

(0 - 300 seconds in 30 second intervals)

SET DELAY

HOME

CHANNEL

SET

(USE CHANNEL KNOB)

OFF

(0 - 9 seconds in 1 second intervals)

DISPLAY

ALPHA

NUMBER

PRE/DE

EMPHASIS

ON

OFF

TRANSMIT

INHIBIT

CARRIER

CARRIER+O

TONE

TONE+O

EMERG’CY

CHANNEL

SET

(USE CHANNEL KNOB)

ALERT

PSSWORD

(FUTURE USE)

(FUTURE USE)

NORMAL

SILENT

NAC

NAC+O

NONE

(FUTURE USE)

"PROGRM" (PROGRAM) CHANNEL MENU (ANALOG CHANNEL)

SCLIST

GLOBAL

CHANNEL

LISTS

SCAN

CHANNEL ID

SET

(USE CHANNEL KNOB)

ALPHA TAG

SET

MODE

ANALOG

12.5 kHz

25.0 kHz

DISABLE

ENABLE

ENCRYPT’N

SET

KEY

B/W

RX FREQ'CY

ENTER

RXSQ MODE

NONE

NOISE

DCS

CTCSS

(USE NUMERIC KEYPAD)

LEVEL

CODE

TONE

TX

FREQ'CY

ENTER

(USE NUMERIC KEYPAD)

TXSQ MODE

NONE

DCS

CTCSS

CODE

TONE

LOW

POWER

0.1 WATT

1 WATT

5 WATT

HIGH

POWER

PASSWORD

0.5 WATT

2 WATT

"PROGRM" (PROGRAM) CHANNEL MENU (DIGITAL CHANNEL)

SCAN LIST

GLOBAL

CHANNEL

LISTS

SCAN

CHANNEL ID

SET

(USE CHANNEL KNOB)

ALPHA TAG

SET

MODE

DIGITAL

DISABLE

ENABLE

ENCRYPT’N

SET

KEY

RX

FREQUENCY

ENTER

RXSQ

MODE

MONITOR

NORMAL

SELECTIVE

IND CALL

(USE NUMERIC KEYPAD)

NAC

NAC

TGID

TX

FREQUENCY

ENTER

(USE NUMERIC KEYPAD)

TXSQ MODE

NONE

NAC

TGID

LOW

POWER

0.1 WATT

1 WATT

5 WATT

HIGH

POWER

B/W

12.5 kHz

25.0 kHz

(FUTURE)

TGID

USER ID

IND CALL

0.5 WATT

2 WATT

PASSWORD

"PROGRM" (PROGRAM) LISTS MENU

GLOBAL

CHANNEL

LISTS

SCAN

SCLISTS

PSSWRD

Reserved for Future Use

"PROGRM" (PROGRAM) SCAN MENU

GLOBAL

CHANNEL

LISTS

SCAN

SWITCH

CHANNEL

SCAN LIST

PRIORITY

CHANNEL 1

SCAN DELAY

LAST RX

CHANNEL

HOME

CHANNEL

TX REVERT

PRIORITY

CHANNEL 2

SET

SET

SET

(USE CHANNEL KNOB)

(USE CHANNEL KNOB)

(ENTER 0 - 7 USING NUMERIC KEYPAD)

PSSWRD

"PROGRM" (PROGRAM) SCAN LIST MENU

GLOBAL

CHANNEL

LISTS

SCAN

SCLISTS

PSSWRD

Reserved for Future Use

"PROGRM" (PROGRAM) PASSWORD MENU

SCAN LIST

GLOBAL

CHANNEL

LISTS

SCAN

PASSWORD

OLD

PASSWORD

NEW

PASSWORD

CONFIRM

PASSWORD

(USE NUMERIC KEYS)

(USE NUMERIC KEYS)

(USE NUMERIC KEYS)

APPENDIX B.  DEFINITIONS

Alert Mode - Display and audio properties used when the Emergency key is pressed.  In Normal mode, the display will flash "EMERGENCY" and an audio tone will be heard.  In Silent mode, the display will be blank and no audio tone will be heard.

Backlight - Light behind the keypad and LCD that allows a user to see the keys/LCD during dark/nighttime conditions.

Backlight Delay - The time that the backlight will remain on after the last keypad activity.

Bandwidth - A small range of frequencies around a transmit or receive frequency in which a message can be received or transmitted.

Bank - A group of zones.  There are up to four (4) Banks per radio.  Up to 16 zones can be distributed through the four Banks.

Channel - A memory location with defined: receive, transmit, squelch, modulation, and power settings.  There are 256 channels per radio.

Channel Locked - An indicator that channel parameters cannot be programmed using the LCD and keypad.  The channel parameters can only be changed using a PC Programmer.

Channel Scan - Will scan all "scan list" channels in a given 16 channel zone.   The scan starts on Home Channel, checks each channel in the Scan Plan, returns to the Home Channel, and then scans each channel in the Scan Plan.

Continuous Tone-Controlled Squelch System (CTCSS) Tone - A sub-audible tone superimposed on an analog signal, used to reduce interference from traffic and background noise.

Digital Coded Squelch (DCS) - A digital variable superimposed on a digital signal, used to reduce interference from traffic and background noise.

Dual-Tone Multiple-Frequency (DTMF) - A signaling scheme, used by the telephone system, in which two voice band tones are generated for each keypad key press.

Global Search - Will scan all frequencies programmed into the radio, regardless of scan list designation.

Initial Synchronization - The length of time required for the radio to perform encryption synchronization.

Monitor Receive Squelch - The radio will receive any intelligible analog transmission.

Monitor Timer - The amount of time the radio will stay on a channel picked up during scan before the radio reverts back to Scan Mode.

Network Access Code (NAC) - Selective squelch for digital mode.  The NAC's primary purpose is to allow a user access to a repeater network.  In radio-to-radio communications, these codes can be used to eliminate interference from other traffic and background noise.

Normal Receive Squelch - The radio will receive any transmission that has the correct NAC.

Priority 1 Scan - The Priority1 channel will be sampled during scanning, receive of an active channel, or standby.  Activity on the Priority1 channel will override all other modes except emergency.

Priority 2 Scan - The secondary priority channel is sampled in a similar fashion to the Priority1 channel.  Activity on the Priority2 channel will override all other modes except Emergency and Priority1.

Receive Only Channel - A feature which does not allow outgoing transmissions on the channel.  It is used for channels for which transmission is prohibited (i.e., weather channels).  If the PTT is pressed on a Receive Only channel, "RX ONLY" will appear on the display.

Repeater Delay - Delay timer used to prevent a radio from receiving it's own transmission from a tactical repeater.

Scan Delay - The amount of time that the scanner dwells on an active receive channel after the carrier has dropped.  This prevents another message from being received before a response can be made.

Scan Reply - The amount of time allowed, if a PTT press interrupts the Scan Delay timer to assure time for a reply to a received message.

Scan List- A group of channels in a zone that have been designated as active scan list channels.  Channels can be added or deleted from the scan list using the PC Programmer or the radio keypad.

Scan Revert Channel - The transmit channel the radio reverts to when the PTT is pressed during or following a scanned message.

Search Mode - The radio will scan for and open on carrier only regardless of CTCSS, DCS, or Digital ID.

Selective Receive Squelch - The radio will receive any transmission that has the correct NAC and TGID.

Shadow Channel - The radio of primary channels, each of which can have up to seven shadow channels.  A shadow channel has the same transmit and receive frequencies, options,. and transmit power levels as it's primary channel.  Individual shadow channels can be configured for different channel types (analog or digital), bandwidth (12.5 or 25 kHz for analog channels), squelch modes, P25 NACs (digital channel only), and encryption key (only one primary or shadow channel can have CVSD DES (conventional analog encryption) enabled).  When properly configured, shadow channels can be created to allow a user to hear all transmissions on a receive/transmit frequency regardless of channel type, bandwidth, squelch mode, or encryption.  Shadow channels can be created and shadow channels parameters can be edited using the MA6941C PC Programmer.

Talk Group Identifier (TGID) - Selective squelch for digital mode.  The TGID is used to group users into functional teams.

Transmit Inhibit and Override - A feature used to stop users from talking over other radio conversations.  There are three options CARRIER prevents transmission if any activity is detected on the channel; TONE prevents transmission on an active channel with a squelch code other than your own; and NAC prevents transmission on an active channel with the same NAC.  There is a quick-key override feature available, which allows a user to override the transmit inhibit state by quick-keying the radio (i.e., two PTT presses within a short time frame).

Transmit Timeout - Prevents inadvertent or prolonged transmit operations that can significantly shorten battery life.

Zone - A group of channels.  There are a maximum of 16 Zones per radio and each zone can contain up to 16 channels. Three (3) zones can be selected by the toggle switch or 16 zones by the radio keypad.

Zone Scan List - A group of Zones in a Bank that have been designated as active scan list zones.  Zones can be added or deleted from the scan list using the PC Programmer or the radio keypad.

APPENDIX C.  SIGNAL TONES

| TONE | SIGNAL | CAUSE: |
| --- | --- | --- |
| Brief Low-Pitched | Key Press Error | Invalid key is pressed. |
|  | Failed Power-On Self Test (POST) | Radio fails Power-On Self Test (POST). |
|  | Transmit Time-Out Warning | Transmit time-out about to interrupt PTT. |
|  | Empty Channel Warning | No Rx/Tx frequencies programmed for the channel. |
| Steady Low-Pitched | Transmit Time-Out Timed Out | Transmit time is exceeded and the PTT is still pressed. |
|  | Transmit Inhibit | PTT switch is pressed and there is activity on the transmit channel. |
|  | Invalid Mode | No programmed data on the selected channel. |
|  | Radio Locked | Radio locks after three consecutive wrong password attempts. |
| Brief Medium-Pitched | Key press | Valid key press is accepted by the radio. |
|  | Radio Passed POST | Radio passed POST. |
|  | Clear Voice being received | Radio is receiving a clear signal. |
| Repeated Medium-Pitched | Emergency Call State | Emergency button is pressed. |
|  | Key Error | Encryption is selected but no key is present. |
| Brief High-Pitched | Low Battery | Battery current falls below a preset value. |
| Repeated High-Pitched | Individual Call | An individual call is received. |

APPENDIX D.  INTERFACES

Side Connector Pins and Functions

| PIN | SIGNAL | FUNCTION | FUNCTION |
| --- | --- | --- | --- |
| 1 | EXTMIC or /WE | GENERAL or FILL | Microphone input or Key transfer Indicator input, asserted by voltage < 0.8VDC. |
| 2 | EXTMICB or KEY | GENERAL or FILL | Microphone bias or bi-directional Key Data, LVTTL levels. |
| 3 | GND | GENERAL | Ground reference. |
| 4 | WRU | GENERAL | Voltage input identifies external device. |
| 5 | EXTSPKR+ or KEYLOAD | GENERAL or FILL | Balanced Audio Output, 500mW into 8 Ohms or LVTTL level LOW Output when Keyloader is connected. |
| 6 | EXTPTT or KID | GENERAL or FILL | Push-to-talk input, asserted by voltage closure to Ground or Pseudo-random Key Insert Data, LVTTL level input. |
| 7 | /RADOFF | GENERAL | Power switch override input. Ground forces off. 3-5VDC forces on. |
| 8 | EXTSPKR- | GENERAL | Balanced Audio Output, 500mW into 8 Ohms. |
| 9 | SQL | GENERAL | Squelch, LVTTL Level HIGH output on valid receive signal. |
| 10 | 232CLK | Data Port | Synchronous clock output to PC Terminal (DCE), RS-232 level (>+3V=ZERO, <-3V=ONE) |
| 11 | 232TXD | Data Port | Transmit data line input (DCE), RS-232 level (>+3V=ZERO, <-3V=ONE) |
| 12 | 232RXD | Data Port | Receive data line output (DCE), RS-232 level (>+3V=ZERO, <-3V=ONE) |
| 13 | 232RTS | Data Port | Data port control input (DCE), RS-232 level (>+3V=ON, <-3V=OFF) |
| 14 | 232DTR | Data Port | Data port control input (DCE), RS-232 level (>+3V=ON, <-3V=OFF) |
| 15 | 232CTS | Data Port | Data port control output (DCE), RS-232 level (>+3V=ON, <-3V=OFF) |
| 16 | PCRXD | Programmer | Receive data line output (DCE), RS-232 level (>+3V=ZERO, <-3V=ONE) |
| 17 | PCTXD | Programmer | Transmit data line input (DCE), RS-232 level (>+3V=ZERO, <-3V=ONE) |
| 18 | EXTPWR | EXT DC Power I/O. | Output current limited to 50mA (TBR). Input voltage limited to 13.8VDC |

Transmitter Interface Pins and Functions

| PIN | NAME | FUNCTION |
| --- | --- | --- |
| 1, 2 | “10VT” | Transmit mode supply voltage, +10 VDC nominal. |
| 3, 4 | “10VA” | Supply voltage, +10 VDC nominal. |
| 5, 6 | GND | Ground. |
| 7 | XTALTEMP | Reference Oscillator temperature monitor. |
| 8 | PATEMP | PA temp monitor. |
| 9 | IMONITOR | PA current monitor. |
| 10 | RSSI | Buffered “IFAGC”. |
| 11 | OOL | Synthesizer Out-of-Lock indicator. |
| 12 | 3VEEPWP | EEPROM write protect. |
| 13 | 3VSCL | EEPROM serial clock. |
| 14 | 3VSDA | EEPROM serial data. |
| 15 | 5VSDA | Serial data. |
| 16 | 5VSYNTHENA | Synthesizer write enable. |
| 17 | 5VSCL | Serial clock. |
| 18 | RCLK | Shift register clock. |
| 19, 20, 21 | GND | Ground. |
| 22 | IFOUT | Receiver 455 kHz IF output. |
| 23, 24, 25 | GND | Ground. |
| 26 | 4.5VSW | 4.5V supply voltage from switches. |
| 27 | LDA | 12-bit DAC chip select. |
| 28 | 5VLDAC | 8-bit DAC chip select. |
| 29 | GND | Ground. |
| 30 | MODCLK | 12-bit DAC serial clock. |
| 31 | GND | Ground. |
| 32 | MODSDI | 12-bit DAC serial data. |

APPENDIX E.  SOFTWARE UPDATE

The radio software can be updated in the radio using a PC and the PC Programming cable.  To update the software , complete the following steps.  The update software (Updatexxx.exe) will either be sent via Email or in a zipped format on a 3.5" disk.  The Emailed version can either be run directly from the Email or loaded on the hard drive and run from there.  The disk copy must be extracted, using unzip software, onto the hard drive and run from there.

- Connect the radio side connector to the PC COM port using the PC Programmer Cable.  Ensure that the radio is turned Off.

- Double click on the Updatexxx.exe file. (xxx is the software version number).

- A Racal Radio Software Update screen will appear as shown below.

- Select the correct COM port (COM1 is the Default) and the highest baud rate the PC will support (9,600 is the default, 38,400 is recommended).

- Turn On the radio.

- Click on Update.  The radio may sound a click or a beep and the radio screen will go blank.  The Radio update screen will provide status.

DO NOT TURN OFF THE COMPUTER OR THE RADIO OR DISCONNECT THE RADIO FROM THE COMPUTER, UNTIL THE UPLOAD PROCESS IS COMPLETE.

- When the process is complete, the window will display "Update Completed Successfully", the radio will beep, and the Operation screen will reappear in the radio display.  Push Exit to exit the software update screen.

- If updating the radio software deletes the channel programming, reload the radio configuration using the PC Programmer.

TROUBLESHOOTING

- If the "Failed to receive frame from radio" error occurs, there is either a problem with the PC-radio connection, the radio battery is dead, or the radio update was disrupted (radio or PC turned off or disconnected).

- First, connect a known good battery to the radio and try again.

- If it still does not work, check to make sure the PC and computer are properly connected.  Ensure the correct COM port is selected in the Racal Radio Software Update screen and that the PC supports the chosen baud rate and try again.

- If it still does not work, the radio firmware needs to be reset before the software can be uploaded.

- Short side connector pins 17 and 18 (back two on the bottom row) while turning On the radio (the recommended method of shorting pins is touching both at once with tweezers).

- The radio may either click or beep within one minute (depending on volume).

- Reconnect the radio to the PC.

- Click on Update.

5.	If the program still does not work, either the radio is bad or the software is corrupted.  Call Racal Customer Support at 1-800-914-0303.

APPENDIX F.  GLOSSARY

AC	Alternating Current

ADC	Analog to Digital Converter

AGC	Automatic Gain Control

AM	Amplitude Modulation

APCO	Association of Public Safety Communications Officials

BATBUS	Battery Serial Bus

BPF	Band Pass Filter

Bps	bits per second

BSF	Band Stop Filter

BW	Bandwidth

C	Celsius

C4FM	Compatible 4-level Frequency Modulation

CAI	Common Air Interface

CCA	Circuit Card Assembly

CMOS	Complimentary Metal-Oxide Semiconductor

CODEC	Coder/Decoder

CPU	Central Processing Unit

CQPSK	Compatible Quadrature Phase Shift Keying

CRC	Cyclic Redundancy Check

CS	Chip Select

CTCSS	Continuous Tone-Controlled Squelch System

CTS	Clear to Send

CVSD	Continuously Variable Slope Delta

DAC	Digital to Analog Converter

dB	decibel

dBm	decibel milliwatt

DBM	Double-Balanced Mixer

DC	Direct Current

DCS	Digital Coded Squelch

DDMC	Digital Data Mode Control

DES	Digital Encryption Standard

DMA	Direct Memory Access

DSP	Digital Signal Processing

DTMF	Dual-Tone Multiple-Frequency

DTR	Data Terminal Ready

EEPROM	Electronically Erasable Programmable Read-Only Memory

EIA	Electronic Industries Alliance

EOM	End of Message

ESD	Electrostatic Discharge

ESSI	Enhanced Synchronous Serial Interface

EXTPTT	External Push-To-Talk

F	Fahrenheit

FDMA	Frequency Division Multiple Access

FIFO	First In First Out

FIR	Finite Impulse Response

FM	Frequency Modulation

FPGA	Field Programmable Gate Array

FS	Frame Synchronization

FSK	Frequency Shift Keying

Hz	Hertz

IC	Integrated Circuit

ICD	Interface Control Document

ID	Identification

IF	Intermediate Frequency

IIR	Infinite Impulse Response

IMBE	Improved Multi-Band Excitation

I/O	Input/Output

ISR	Interrupt Service Routine

JTAG	Joint Test Action Group

kbps	kilobits per second

kHz	kiloHertz

ksps	kilo-samples per second

KVL	Key Variable Loader

LCD	Liquid Crystal Display

LED	Light Emitting Diode

Li-ion	Lithium Ion

LO	Local Oscillator

LPF	Low Pass Filter

LUT	Look-Up Table

mA	milliAmpere

mAH	milliAmpere Hour

MHz	MegaHertz

MLA	Modulation Level Analyzer

MMI	Man-Machine Interface

MPU	Micro-Processor Unit

ms	milliseconds

NAC	Network Access Code

NID	Network Identification

OFB	Output Feedback

OOL	Out-of-Lock

PA	Power Amplifier

PC	Personal Computer

PCB	Printed Control Board

PDS	Product Development Specification

PLL	Phased Locked Loop

POST	Power On Self-Test

ppm	parts per million

PSU	Power Supply Unit

PTT	Push-To-Talk

PWM	Pulse Width Modulation

RAM	Random Access Memory

RF	Radio Frequency

ROM	Read-Only Memory

RSSI	Received Signal Strength Indicator

RTS	Request to Send

Rx	Receive

SBCF	Single-Bit Cipher Feedback

SMA	Sub-Miniature series A

Stby	Standby

TGID	TalkGroup Identification

TIA	Telecommunications Industry Association

Tx	Transmit

V	Volt

VAC	Volts, Alternating Current

VCO	Voltage Controlled Oscillator

VDC	Volts, Direct Current

VHF	Very High Frequency

VOCODER	Voice Coder/Decoder

VOGAD	Voice-Operated Gain Attenuating Device

Vp-p	Volt peak-to-peak

VSWR	Voltage Standing Wave Ratio

W	Watts

WRU	Who Are You

SCHEMATICS

The following schematics are included:

4200649	Keypad CCA(1 page)

4200675	Transceiver CCA(4 pages)

4200676	Control CCA (6 pages)

All schematics are subject to change without notice.

Part Number 84328, Rev. A

Racal Communications, Inc.

5 Research Place

Rockville, MD 20850

RCI@racalcomm.com