Thales 25

Remote Commands

Revision History

| Date | Version | Description | Author |
| --- | --- | --- | --- |
| 05/08/03 | 0.1 | Initial Creation | JParis |
| 08/15/03 | 0.2 | Update for release 6.0 | JParis |
| 11/17/03 | 0.3 | Add changes from PDF Infusion comments | JParis |
| 12/03/03 | 0.4 | Add changes for channel position selection | JParis |
|  | 01 | Initial PDF Infusion revision number | JParis |
|  | 02 | Update for release 7.0 Add /gbaud and /gps commands and update /scan with P1CHan and P2CHan | JParis |

### Remote Command List

//		Repeats last successful command.  Especially useful with toggle commands.

/ANTENNA 	Selects top or side RF antenna port

/AUDIOCAL	Set the audio cal value

/BANK		Set, creates, or programs desired Bank

/BAUD		Sets RS-232 baud rate

/CAL		Hardware Calibration Reload

/CHAN		Sets, creates, or programs desired channel or programs new channel

/CG		Selects a crypto group and / or configures a crypto group

/DREG		Enables/Disables Data Registration

/DTMF		Produces DTMF tones 0-9,*,# when radio is in TX mode

/ECHO		Sets the remote command echo mode

/EMERGENCY	Sets / Enables the emergency channel

/ERASE		Erase database information

/FREQ		Sets desi	red frequency.  Also used with /chan command for programming

/GBAud		Sets the default baud rate for the radio

/GPS		Sets up the GPS settings or displays the GPS parameters

/IDCALL	Enables or disables an ID call

/IMUTE		Mutes or un-mutes the internal speaker

/KEY		Sets, creates or programs a radio key

/KEYSET	Sets, creates or programs a radio keyset

/MODE		Sets radio to desired operating mode.  Also used with /chan command for programming

Also used for BER testing

/NAC		Sets desired P25 NAC value.  Also used with /chan command for programming

/OTAR		Enables/Disables OTAR mode

/PTT		Sets PTT state to ON or OFF

/PWR		Sets desired Power level.  Also used with /chan command for programming

/REKEY	Initiates a radio re-key operation

/SCan	Sets scanning enable, type, and priority channels

/SHADOW	Sets shadow channel and shadow channel parameters

/SQUELCH	Sets desired squelch mode and value.  Also used with /chan command for programming

/TAG		Programs desired channel nametag

/TAlkaround	Enable / Disables talkaround.  Also used with /chan command for programming

/Tkgrp		Loads/queries pick list talk groups

/TOT		Sets transmit timeout

/Txsq		Loads/queries pick list AD squelch modes

/Unitid		Loads/queries pick list unit ids

/USRID		Sets the user id value for the radio

/VER		Generates report containing radio version and identification details

/VOL		Sets volume.  Also used for P25 deviation testing

/XT		Shifts reference oscillator table

/ZONE		Sets, creates, or programs desired zone (also may be used to generate scanlists)

### Introduction

This document describes the Remote Commands available on the Thales 25 radio.

Attempts have been made to provide additional remote programming capability, while maintaining backward compatibility to all previous commands.  However, the following significant changes should be noted that may affect older commands:

- Many commands now require a space between the command and the parameter.  The older commands were more forgiving of spaces.  This change was necessary to provide structure to the remote command format

- The older commands provided temporary programming changes.  This has been changed to permanent programming changes.  This was necessary to eliminate some architectural problems with the temporary changes.  Note that the same effect can be realized without reprogramming unwanted channels by limiting changes to one known channel.

One significant change has been the addition of a radio power up status message which tells the user that the radio has finished its power up sequence.  This message is “OK” and will be displayed when the power up sequence is finished.

### Interface

The interface to the radio is through an RS-232 serial port and the PC Programming cable.  The radios default configuration is 9600-8-N-1 and it will accept standard ASCII characters sent using a standard terminal program.

The commands shown in this document may be sent to the radio after receiving the initial “OK” message from the radio on completion of the power up sequence.  There are three responses that can be received from the radio after issuing a command, these are “OK”, “ERROR” and “?”.  The “OK” message will be received after successful completion of an entered command.  The “ERROR” message will be received after unsuccessful completion of an entered command and the “?” message which is also an indication of unsuccessful completion will be received if a control character is entered.

In order to prevent errors, commands should be entered one at a time and the “OK” response should be received before sending the next command to the radio.  This will provide command synchronization between the sender and the radio and provide positive feedback to the sender that the command was executed properly.

### Commands

//			Repeat Last Command

Command: //

Syntax: //

Parameters:

None

Description:

Repeats last successful command.

Example:

/ptt toggle

PTT = On

OK

//

PTT = Off

OK

//

PTT = On

OK

/ANTENNA 		Antenna Select

Command: ANTenna

Syntax:  /ANT [ Top | Side | STd ]

Parameters:

STd		- Selects the standard (top) antenna port.

Side		- Selects the side connector antenna port.

Top		- Selects the standard (top) antenna port.

Description:

Select Antenna port on radio.  One of the selections must be entered with this command.

Example(s):

/ant top

OK

/ANT T

OK

/ant side

OK

/AUDIOCAL 		Audio Calibration Value Select

Command: AUDIOcal

Syntax:  /AUDIO [ # ]

Parameters:

None		- Displays current audio calibration value

#		- Sets the audio calibration value to #

Description:

Displays or sets the audio calibration value for the radio.

Example(s):

/audio

Audio Cal Value = 100

OK

/audio 0

Set Audio Cal Value = 0

OK

/BANK		Bank Select / Program

Command: Bank

Syntax: /Bank [ # | NEW [#] ] [ TAG  "<name>" ] [ ZONE # [ slot # | delete # ] ]

Parameters:

None		- Displays current bank status

#	- Specifies a bank number to select where the valid values are 1 – 4 for the main bank and 5 for the event bank.

NEW       	- Specifies a new bank to select.

#      	- Specifies new bank to be created with the specified bank number.

TAG         	- Displays the selected bank tag.

"<name>"	- Sets the selected bank's tag to <name>, quotes are required.

ZONE #	- Sets zone # to this bank

Slot #	- Sets zone # to slot # of this bank (Slot numbers 1 – 16)

Delete #	- Deletes zone # from this bank

Description:

Selects, creates, or programs desired bank.  When creating a new bank the minimum options required to add a bank is shown in the example.  You may specify the bank ID value after the new option or the radio will choose the next available value.  In order to position a zone within a bank you must specify the zone and zone number and the slot and slot number in the command as shown.  In order to delete a zone from a bank you must supply the zone and zone number in the command along with the position number to delete after the delete option as shown.

Example:

/bank new tag "BNK 2"

Bank ID:    2

Tag = BNK 2

OK

/bank zone 1 slot 2

Bank ID:    1

Zones: 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

OK

/bank zone 1 delete 2

Bank ID:    1

Zones: 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

OK

/BAUD		Baud Rate Select

Command: BAUD

Syntax: /BAUD [ 4800 | 9600 | 19200 ]

Parameters:

None		- Displays current baud rate

- - sets baud rate to 4800

- - sets baud rate to 9600

19200		- sets baud rate to 19200

38400		- sets baud rate to 38400

Description:

Sets RS232 baud rate.  Note that  attached device must also be configured to match.  The remaining RS232 parameters for the port that are not settable are: 8 data bits, No parity, 1 stop bit, and no flow control.

Example:

/baud

Baudrate: 9600 Bits/S

OK

/CAL		Hardware Calibration Reload

Command: CAL

Syntax: /CAL

Parameters:

None		- Reloads hardware calibration values.

Description:

Forces a reload of the hardware calibration table.

Example:

/CAL

OK

/CHAN	Channel / Shadow Channel Select and Program

Command: CHan

Syntax: /CHan [ # | NEW | Pos # ] [ SHadow # | NEW ] [{Opt Param- see list below}]

Select Param:

None		- Displays current channel status

# 	- Used in conjunction with other options to specify which channel to apply the option.

NEW     - Specifies a new channel to select.

#   - Specifies new channel to be created with the specified channel

number.

Pos #   - Specifies position # to select within the current zone. Where

the # is in the range of 1 to 16.

Opt Param:

TAG     - Display

"<name>" - Sets the selected channel's tag to <name>, quotes are required.

FReq		- See Frequency Command.

MODE	- See Mode command.

SQuelch	- See Squelch command.

NAC		- See Nac command.

PWR		- See Pwr Command

TAlkaround

Enable	- Sets Channel Talkaround option to enabled

Disable 	- Sets Channel Talkaround option to disabled (default)

Toggle	- Toggles Channel Talkaround option

SCan

Enable	- Sets Channel Scanlist option to enabled

Disable 	- Sets Channel Scanlist option to disabled (default)

Toggle	- Toggles Channel Scanlist option

Otar

Enable	- Sets Channel Otar option to enabled

Disable	- Sets Channel Otar option to disabled (default)

Toggle	- Toggles Channel Otar option

Key

id		- Sets Channel key to key id

SLn

id		- Sets Channel sln to sln id

Encryption

Enable	- Sets Channel encryption option to enabled

Disable 	- Sets Channel encryption option to disabled (default)

Toggle	- Toggles Channel encryption option

CRyptlock

Enable	- Sets Channel encryption lock option to enabled

Disable 	- Sets Channel encryption lock option to disabled (default)

Toggle	- Toggles Channel encryption lock option

Shadow	- See Shadow Command

Description:

Used to select channels or shadow channels.  All valid channel parameters can also be set.  For shadow channels, options, frequencies, and power parameters may not be changed and you cannot select the ID number for the shadow channel.  In order to add a new shadow to a channel not currently selected you must specify the channel number in the command line.  When creating a new channel the minimum parameters to enter are “tag”, “freq” and “mode” as shown in the example below.  The POS # option is used to select a channel in the current zone for reception or transmission.  The radio must have channels loaded into at least one zone and have the zone selected in order to switch between channels using remote commands.

Example:

/chan new tag "CH 1" freq 157.275 mode an

Chan ID:    1

Tag = CH 1

Rx Freq = 157275000

Tx Freq = 157275000

Mode = Analog Narrow

OK

/chan

Chan ID:    1

Tag = CH 1

Mode = Analog Narrow

Scan: Enabled

Rx Freq = 157275000

Squelch Rx mode: Noise

Rx Level: 2

Tx Freq = 157275000

Squelch Tx mode: None

OK

/chan new 2 tag "CH 2" fr rx 145.25 tx 145.75 mode dg sq rx select nac 123 pwr lo 0.1w hi 5w

Chan ID:    2

Tag = CH 2

Rx Freq = 145250000

Tx Freq = 145750000

Mode = P25

Squelch Rx mode: Select

Rx Talkgroup: 1

Rx NAC = 0x7B = 123

Tx NAC = 0x7B = 123

High Power = 5w

Low Power = 0.1w

OK

/chan new 4 tag "CH 4" fr 147.75 mode dg sq rx select nac 123 sc enable otar enable key 1 pwr lo 0.1w hi 5w

Chan ID:    4

Tag = CH 4

Rx Freq = 147750000

Tx Freq = 147750000

Mode = P25

Squelch Rx mode: Select

Rx Talkgroup: 1

Rx NAC = 0x7B = 123

Tx NAC = 0x7B = 123

Scan: Enabled

Otar: Enabled

Key: 1

High Power = 5w

Low Power = 0.1w

OK

/chan shadow new tag "SH 1" mode an sq ctcss 67

Chan ID:    5

Shadow: 1

Tag = SH 1

Mode = Analog Narrow

Squelch Rx mode: CTCSS

Rx Tone:  67.0 XZ

Squelch Tx mode: CTCSS

Tx Tone:  67.0 XZ

OK

/CG		Crypto Group Control

Command: CG

Syntax: /CG [ # |  NEW [ # ] | TAG <name> | KEYSET | KEYSELect ]

Parameters:

none		- Displays current crypto group status

#		- Specifies the crypto group to select

NEW		- Creates a new crypto group

#		- Creates a new crypto group with # id

TAG		- Sets the selected crypto group’ name

name	- Sets the selected crypto group's tag to <name>, quotes are required.

KEYSET	- Modifies the selected crypto group’s keyset

ACtive	- Selects the active keyset for the selected crypto group

SLot	- Used to set a keyset to a slot in the crypto group

#		- Sets a keyset to slot # in the selected crypto group

DElete	- Used to delete a keyset from the selected crypto group

KEYSELECT	- Sets the keyselects within a crypto group

#		- Sets keyselect # in a crypto group

slot		- Sets a keyselect to a slot in the crypto group

#		- Sets keyselect to slot # in the crypto group

DElete	- Deletes a keyselect from a slot in the crypto group

Description:

Used to set / configure a crypto group in the radio.  The minimum needed to generate a new crypto group is the NEW option as shown below.  The other options can be added after the crypto group is created.  When deleting a keyset or keyselect the command requires the keyset or keyselect number and the DELETE option to follow.

Example:

/cg new 1 tag "CG 1" keyset 1 slot 1 keyset 1 active

Crypto Group ID:    1

Tag = CG 1

KeySets: 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

Active Keyset: 0

KeySets: 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

Active Keyset: 1

OK

/cg 1 keyset 2 slot 3

Crypto Group ID:    1

KeySets: 1, 0, 2, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

Active Keyset: 1

OK

/cg keyset 2 delete

Crypto Group ID:    1

KeySets: 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

Active Keyset: 1

OK

/cg keyselect 1 slot 1

Crypto Group ID:    1

Keyselects: 1, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL

OK

/cg new

Crypto Group ID:    3

OK

/cg

Crypto Group ID:    3

Tag = CRPG 03

KeySets: 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

Active Keyset: 0

Keyselects: NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL, NUL

OK

/DREG	Data Registration Control (OTAR ONLY)

Command: DREG

Syntax: /DREG [ ENABLE | DISABLE | TOGGLE ]

Parameters:

none		- Displays current dreg status

Enable	- Enables the dreg mode.

Disable	- Disables the dreg mode.

Toggle	- Toggles the dreg status

Description:

Used to enable or disable the channels data registration mode.

Example:

/DREG

Data Reg: Disabled

OK

/DREG ENABLE

Data Reg: Enabled

OK

/DREG TOGGLE

Data Reg: Disabled

OK

/DTMF	DTMF Control

Command: DTMF

Syntax: /DTMF [ # ]

Parameters:

#		- Produces DTMF tone for # entered ( 0 – 9, 10 = *, 11 = # , 12 = A, 13 = B, 14 = C, 15 = D)

Description:

Used to produce DTMF tones while transmitting.  The valid entries are 0 – 9 for the number keys, 10 for the * (star) key, 11 for the # (hash) key, 12 for the A key, 13 for the B key, 14 for the C key and 15 for the D key.

Example:

/DTMF 1

OK

/DTMF 10

OK

/ECHO	Echo Control

Command: ECHo

Syntax: /ECH [ ON | OFf ]

Parameters:

none		- Displays current echo status

ON		- Sets the echo mode ON.

OFf		- Sets the echo mode OFF.

Description:

Used to turn the echo mode on or off (Default = OFF).  This will turn off the echoing of  a remote command and return only OK or ERROR instead of the info associated with the remote command.

Example:

/ECH

ECHO = Off

OK

/ECH OFF

ECHO = Off

OK

/ECH ON

ECHO = On

OK

/EMERGENCY	Emergency Channel Control

Command: EMERgency

Syntax: /EMER [ # | ON | OFf ]

Parameters:

none		- Displays current emergency channel ID

#		- Sets the emergency channel to channel ID #

ON		- Sets the emergency channel mode ON.

OFf		- Sets the emergency channel mode OFF.

Description:

Used to turn the emergency channel on or off and also to set the radio emergency channel to the channel ID entered.

Example:

/EMER

OK

/EMER 108

Emergency Channel ID = 108

OK

/ERASE	Erase Database Info

Command: ERASE

Syntax: /ERASE [ all | chan [all | #] | zone [all | #] | bank [all | #] | key [all | #] | keyset [all | # [sln #]] | cryptogrp [all | #] | unitid [ all | # ] | tkgrp [ all | # ] | txsq [ all | 3 ] | fire all ]

Parameters:

all		- Erases entire radio database

chan		- Erases channel information

all	- Erases all channel information from radio database

#	- Erases channel # information from radio database

zone		- Erases zone information

all	- Erases all zone information from radio database

#	- Erases zone # information from radio database

bank		- Erases bank information

all	- Erases all bank information from radio database

#	- Erases bank # information from radio database

key		- Erases key information

all	- Erases all key information from radio database

#	- Erases key # information from radio database

keyset		- Erases keyset information

all	- Erases all keyset information from radio database

#	- Erases keyset # information from radio database

sln	- Erases sln information from the radio database

#	- Erases sln # information from the radio database

cryptogroup	- Erases cryptogroup information

all	- Erases all cryptogroup information from radio database

#	- Erases cryptogroup # information from radio database

unitid		- Erases unitid information

all	- Erases all unitid information from radio database

#	- Erases unitid # information from radio database

tkgrp		- Erases tkgrp information

all	- Erases all tkgrp information from radio database

#	- Erases tkgrp # information from radio database

txsq		- Erases txsq information

all	- Erases all txsq information from radio database

#	- Erases txsq # information from radio database

fire all	- Erases all firefeatures information from radio database

Description:

Used to erase database information from radio.

Example:

/ERASE chan 11

OK

/FREQ	Frequency Select and Program

Command: FReq

Syntax: /FReq [ # | Rx [#] | Tx [#] ]

Parameters:

None		- Returns current frequency status

#		- Sets the selected channels RX and TX frequency to specified #.

Rx		- Displays the selected channels RX frequency.

Rx #		- Sets the selected channels RX frequency to specified #.

Tx		- Displays the selected channels TX frequency.

Tx #		- Sets the selected channels TX frequency to specified #.

Description:

Sets RX and / or Transmit frequency on the currently selected channel.  Frequency # must be in the range of  136 to 174 MHz.  Decimal points are omitted, and frequency is automatically set in the frequency range 136 to 174 MHz, if possible.

Example:

/freq

Rx Freq = 168600000

Tx Freq = 168600000

OK

/fr 145

Rx Freq = 145000000

Tx Freq = 145000000

OK

/fr t 155

Tx Freq = 155000000

OK

/fr r

Rx Freq = 145000000

OK

/GBAUD	Global Default Baud Rate

Command: GBAud

Syntax: /GBAud [ # ]

Parameters:

None		- Returns current default baud rate if on else returns OK

#		- Sets the default baud rate to #.

Description:

Used to set or display the current default baud rate for the radio.

Example:

/gbaud

Baudrate: 9600 Bits/S

OK

/gbaud 9600

Baudrate: 9600 Bits/S

OK

/GPS	GPS Parameters

Command: GPS

Syntax: /GPS [ dest [#] | delay [#] | on_ptt [enable | disable | toggle] | on_dlay [enable | disable | toggle] | sap [#] | src [tag] |  ]

Parameters:

None		- Returns current GPS settings if on else returns OK

dest		- Sets the destination radio ID to specified #.

delay		- Sets the GPS update interval entered in seconds, valid range is: 3 – 999 seconds, default

is 60 seconds.

on_ptt		- Sets the GPS update on ptt to enabled or disabled, default is disabled.

on_dlay	- Sets the GPS update on delay to enabled or disabled, default is enabled.

sap		- Sets the sap used to send and receive the GPS messages, valid range is: 0 – 63,

default is 48.

src		- Set the source tag for the radio, maximum of 8 characters.

Description:

Used to setup the GPS parameters or display the GPS settings.

Example:

/gps

GPS Source = 0000004

GPS Dest = 1

GPS On_ptt = Disabled

GPS On_dlay = Enabled

GPS Delay = 3 Secs.

GPS SAP = 0x30

OK

/gps on_dlay toggle

GPS On_dlay = Disabled

OK

/IDCALL	Individual Call

Command: IDCall

Syntax: /IDCall [ # | on [#] | off ]

Parameters:

None		- Returns current ID call status if on else returns OK

#		- Sets the selected destination ID to specified #.

on		- Sets ID call mode on.

on #		- Sets ID call mode on and the selected destination ID to specified #.

off		- Sets ID call mode off.

Description:

Used to enable or disable an ID call and to set up the destination ID for an ID call on a P25 channel.

Example:

/idcall

IDCALL ON   Destination = 0001131

OK

/idcall on 1234

IDCALL ON   Destination = 0001234

OK

/idcall off

IDCALL OFF

OK

/IMUTE		Internal Speaker Control

Command: IMUTE

Syntax: /IMUTE [ ON | OFf |  Toggle ]

Parameters:

none		- Displays current IMUTE status

ON		- Sets the internal speaker OFF.

OFf		- Sets the internal speaker ON.

Toggle	- Toggles internal speaker setting.

Description:

Sets Internal speaker ON or OFF (default is ON).

Example:

/imute

Internal Speaker = Unmuted

OK

/imute on

Internal Speaker = Muted

OK

/KEY	Key Control

Command: KEY

Syntax: /KEY [ # | NEW [ # | WARMSTART ] | TAG name | ALGorithm type | DATa X X X X | TYPe value | ID # ] ]

Parameters:

none			- Displays current key status

NEW			- Create a new key

#			- Create a new key with id #

WARMSTART	- Create a new warmstart key (ID 65)

TAG			- Sets the selected key’s tag

name		- Sets the selected key's tag to <name>, quotes are required.

ALGorithm		- Sets the selected key’s algorithm

type		- Sets the selected key’s algorithm to <type> ( DES )

DATa			- Sets the selected key’s data

X X X X	- Sets the selected key’s data to X X X X where X is a Word (16 bits) entered in decimal format

TYPe			- Sets the selected key’s type

value		- Sets the selected key’s type to <value> ( KEK, TEK )

ID			- Sets the selected key’s ID

#			- Sets the selected key’s ID to # ( 16 bits )

Description:

Used to set key parameters or generate a new key in the radio.  When creating a new key you must enter the TAG value as a minimum.  The other values can be entered later or entered all at once as shown below.  The algorithm has only one value, DES, which is the default when entering keys.  The option is included for future expansion.

Example:

/key new 1 tag "KEY 1" data 257 257 257 257 type TEK id 1

Key ID:    1

Tag = KEY 1

Keydata = ****, ****, ****, ****

Type: TEK

Keyid = 0x0001 (1)

OK

/key

Key ID:    1

Tag = KEY 1

Type: TEK

Alg: DES

Keyid = 0x0001 (1)

Keydata = ****, ****, ****, ****

OK

/KEYSET	Keyset Control

Command: KEYSET

Syntax: /KEYSET [ # | NEW [ # ] | TAG name | ALGorithm type | TYPe value | KEY # [ SLOT # ] ] ]

Parameters:

none			- Displays current keyset status

NEW			- Create a new keyset

#			- Create a new keyset with id #

TAG			- Sets the selected keyset’s tag

name		- Sets the selected keyset's tag to <name>, quotes are required.

ALGorithm		- Sets the selected keyset’s algorithm

type		- Sets the selected keyset’s algorithm to <type> ( DES )

TYPe			- Sets the selected keyset’s type

value		- Sets the selected keyset’s type to <value> ( KEK, TEK )

KEY			- Adds a key to the selected keyset

#			- Adds key # to the selected keyset

SLOT #		- Adds key to slot # in the keyset

Description:

Used to set keyset parameters or generate a new keyset in the radio.  You cannot add keys to the keyset when creating the keyset with the NEW command but must add them after creating as shown below.

The algorithm has only one value, DES, which is the default when entering keys.  The option is included for future expansion.

Example:

/keyset new 1 tag "KEYSET 1" type TEK

Keyset ID:    1

Tag = KEYSET 1

Type: TEK

OK

/keyset 1 key 1 slot 2

Keyset ID:    1

Keys: 0, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

OK

/keyset1 key 2 slot 1

Keyset ID:    1

Keys: 2, 1, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0, 0

OK

/MODE	Mode Control

Command: MODe

Syntax: /MODE [ AN | AW | DIGITAL | BER [ ON | OFF | TOGGLE ] ]

Parameters:

none		- Displays current mode status

AN		- Sets the current channel to Analog Narrow (12.5 KHz)

AW		- Sets the current channel to Analog Wide (25 KHz)

DIGITAL	- Sets the current channel to Project 25 digital mode.

BER		- Sets radio to output frame errors for BER measurements

ON		- Enables the BER output mode

OFF	- Disables the BER output mode

TOGGLE	- Toggles the BER output mode

Description:

Sets the radios current channel to selected MODE.  Note that squelch parameters should be set after a mode change.  Radio must already be in P25 MODE for /MODE BER to be activated.

Example:

/mode an

Mode = Analog Narrow

OK

/mode digital

Mode = P25

OK

/NAC		NAC Programming

Command: NAC

Syntax: /NAC [ # | Rx [#] | Tx [#] ]

Parameters:

none		- Displays current NAC status

#		- Sets RX and TX NAC to #

Rx		- Displays current RX NAC value

Rx [#]		- Sets RX NAC value to #

Tx		- Displays current TX NAC value

Tx [#]		- Sets TX NAC value to #

Description:

Programs or displays NAC values in hexidecimal (0x#) and decimal (#) as shown below.

Example:

/nac

Rx NAC = 0x293 = 659

Tx NAC = 0x293 = 659

OK

/nac r 123

Rx NAC = 0x7B = 123

OK

/nac tx 0x221

Tx NAC = 0x221 = 545

OK

/OTAR	OTAR Control (OTAR ONLY)

Command: OTAR

Syntax: /OTAR [ ENABLE | DISABLE | TOGGLE | MN [ # ] ]

Parameters:

none		- Displays current otar status

Enable	- Enables the otar mode.

Disable	- Disables the otar mode.

Toggle	- Toggles the otar status

MN		- Sets/Displays the OTAR message number range

#		- Sets message number range to # ( Default is 65535 )

Description:

Used to enable or disable the channels OTAR ( Over The Air Re-keying ) mode.

Example:

/OTAR

Otar: Enabled

MN Range: 65535

OK

/OTAR ENABLE

Otar: Enabled

OK

/OTAR TOGGLE

Otar: Disabled

OK

/PTT	PTT Control

Command: PTt

Syntax: /PTt [ ON | OFf | Toggle ]

Parameters:

ON          - Turns PTT on.

OFf         - Turns PTT off.

Toggle      - Toggles the current state of PTT on -> off, off -> on.

Description:

This command is used to enable/disable transmit mode.

Example:

/ptt

PTT = Off

/ptt toggle

PTT = On

OK

//

PTT = Off

OK

/PWR	Power Control

Command: PWR

Syntax: /PWR [ #w | Lo [#w ]| Hi [#w] ]

Parameters:

#w		- Sets the transmit power to #w.

Lo 		- Sets the transmit power to the channel's low setting.

Hi 		- Sets the transmit power to the channel's high setting.

Description:

This command allows temporarily setting the current channels power level.  The choices are the channels lo or hi power settings or a value entered.   The entered power setting must be one of the following: 0.1w, 0.5w, 1w, 2w, 5w.  The power setting will reset to the selected radio power setting of lo or hi on a channel change.

Example:

/pwr

PWR = Low

OK

/pwr hi

PWR = High

OK

/pwr 0.1w

PWR = 0.1w

OK

/REKEY	OTAR RE-KEY Control (OTAR ONLY)

Command: REKEY

Syntax: /REKEY

Parameters:

None		- Initiates a radio re-key

Description:

This command causes the radio to initiate an Over The Air Re-key on an OTAR enabled channel.

Example:

/REKEY

OK

/SCan	Scan Control and Programming

Command: SCan

Syntax: /SCan [ ON | OFf | Toggle | Mode [ SCan | Zone | SEarch ]| Priority [ PR1 | PR2 | OFf ] | MONitor |

REPly | DELay | REVert [Home | Rx | Sel ] | P1Chan # | P2Chan #  ]

Parameters:

ON	- Turns scan mode on.

OFf	- Turns scan mode off.

Toggle	- Toggles the current state of scan on -> off, off -> on.

Mode	- Displays the current scan mode.

SCan 	- Set the scan mode to scan channels within the current zone.

Zone 	- Set the scan mode to scan channels within the current bank.

SEarch 	- Set the scan mode to scan all channels defined.

PScan	- Turns priority scan on/off

ON	- Turns priority scan on

OFF	- Turns priority scan off

Priority 	- Displays the current priority mode.

PR1	- Turns on Priority 1 scan.

PR2 	- Turns on Priority 2 scan, turns on Priority 1 + Priority 2 if PScan is on.

OFf	- Turns Priority scan off.

#	- Sets priority timer. Valid values for time are: 0.5, 1.0, 1.5, 2.0, 2.5,

3.0, 3.5, 4.0, 4.5, 5.0, 5.5, 6.0, 6.5, 7.0, 7.5, 8.0, 8.5, 9.0, 9.5

MONitor	- Sets Scan monitor timer (INT range 0-90)

REPly	- Sets Scan reply timer (INT range 0-20)

DELay	- Sets Scan delay timer (INT range 0-15)

REVert	- Displays the current revert mode

Home	- Sets Scan Revert mode to Home

Rx	- Sets Scan Revert mode to Last Received

Sel	- Sets Scan Revert mode to Last Selected

P1CHan #	- Sets the P1 channel for the current bank where # is the channel ID

P2CHan #	- Sets the P2 channel for the current bank where # is the channel ID

Description:

Sets and programs all global scan parameters and states.  If scan is currently on then it must be turned off before a new scan mode can be selected.

Example:

/scan

Scan = Off

Scan Mode = ZONE

Scan Reply Timer: 5 Sec

Scan Monitor Timer: 5 Sec

Scan Delay Timer: 2 Sec

Scan Priority Mode = PR1

Scan Priority Timer = 500 mSec

OK

/scan on mode sc mon 2

Scan Mode = SCAN

Scan Monitor Timer: 2 Sec

OK

/scan

Scan = On

Scan Mode = SCAN

Scan Reply Timer: 5 Sec

Scan Monitor Timer: 2 Sec

Scan Delay Timer: 2 Sec

Scan Priority Mode = PR1

Scan Priority Timer = 500 mSec

OK

/SHADOW	Shadow Channel Control

Command: SHADOW

Syntax: /SHADOW [ #  | revert [ rx | lastrx | sel | lastsel ] | delay [ # ] ]

/CHAN SHADOW NEW {…}  for creating new shadow channel only

Parameters:

#	- Shadow channel selection

revert	- Set/Display revert mode

rx	- Revert to last RX channel

lastrx	- Revert to last RX channel

sel	- Revert to last selected channel

lastsel	- Revert to last selected channel

delay	- Set/Display shadow delay timer

#	- Set shadow delay timer to # seconds ( 0 – 15 )

new	- Create a new shadow channel

Description:

Used to select / display shadow channel and to set / display the revert mode and shadow timer delay value.  In order to create a new shadow channel the /CHAN command is used passing in the “SHadow new” parameter with one of the above options.  The shadow channel ID will be selected by the radio.

Example:

/SHADOW

Chan ID:  108

Shadow: 0

OK

/SHADOW 1		(change to first shadow channel)

Chan ID:  114

Shadow: 1

OK

/SHADOW revert	(display revert mode)

Chan ID:  108

Shadow: 0

Shadow Revert Mode = Last RX

OK

/SHADOW delay	(display delay timer value)

Chan ID:  108

Shadow: 0

Shadow Delay Timer: 3 Sec

OK

/CHAN SHADOW new tag "S3" mode dg

OK

/SQUELCH	Squelch Control and Programming

Command: SQuelch

Syntax: /SQ [ Rx | Tx ] [ Ctcss # | Dcs # | Monitor | NOIse # | NONe | NORmal | Select # ]

Parameters:

RX		- Selects RX squelch

TX		- Selects TX squelch

Ctcss #		- Selects analog squelch mode ctcss with value #

Dcs #		- Selects analog squelch mode dcs with value #

Monitor		- Selects digital rx squelch mode monitor

NOIse		- Selects analog noise squelch wth value #

NONe		- Selects analog tx squelch mode none

NORmal	- Selects digital rx squelch mode normal

Select #		- Selects digital squelch mode select with talkgroup value #

Description:

Used to set / program all squelch parameters.  DCS values must be entered with a leading zero (0)  as in the example below since these values are in octal format.  The digital select squelch mode may be combined with the Tx and Rx options to set these to individual values as shown below.

Example:

/sq nac

Squelch Rx mode: Select

Rx Talkgroup: 65535

Squelch Tx mode: Select

Tx Talkgroup: 65535

/sq ctcss 67			(channel was P25)

Error

/mode an			(changed channel to Analog channel)

Mode = Analog Narrow

OK

/sq ctcss 67			(able to set CTCSS tones)

Squelch Rx mode: CTCSS

Rx Tone:  67.0 XZ

Squelch Tx mode: CTCSS

Tx Tone:  67.0 XZ

OK

/sq tx D 0114

Squelch Tx mode: DCS

Tx Code: 114

OK

/sq tx select 1 rx select 2

Squelch Rx mode: Select

Rx Talkgroup: 2

Squelch Tx mode: Select

Tx Talkgroup: 1

OK

/TAG		Tag Programming

Command: TAG

Syntax: /{CHan | Zone | BAnk TAG [“<name>”]

Parameters:

none		- Displays current TAG value

“<name>”		- Sets TAG to “name”.  Quotes are required

Description:

Reprograms Tag value.  Must be secondary parameter of channel, zone, or bank.

Example:

/chan tag

Chan ID:   22

Tag = bob 3

OK

/chan tag "newname"

Chan ID:   22

Tag = newname

OK

/zone tag

Tag = ANALOGNB

3    ANALOGNB

OK

/TAlkaround	Global Talkaround Control

Command: TAlkaround

Syntax: /TAlkaround [ Enable | Disable | Toggle ]

Parameters:

none		- Displays current global talkaround status

Enable	- Enables global talkaround.  Note that channel talkaround must also be enable to work.

Disable	- Disabled global talkaround

Toggle	- Toggles global talkaround state.

Description:

Sets global talkaround state

Example:

/TALK

Talkaround: Enabled

OK

/TALK TOGGLE

Talkaround: Disabled

OK

/TKGRp	Pick List Talkgroup Control ( Fire Feature )

Command: TKGRp

Syntax: /TKGRp [ Create [ tag | id | p25tkid ] | List ]

Parameters:

None		- Lists current talkgroups loaded in radio

Create	- Creates a new entry into the talkgroup pick list

tag	- An 8 character tag for the entry

id	- ID for this entry, this will be used for both the reference number in the list (pick ID) and  the database ID.  If not specified, the next available ID will be used.  Valid rage=1-99.

p25tkid	- P25 talkgroup ID.  Valid range=1-65536.

List  		- Lists current talkgroups loaded in radio

Description:

Loads/Lists the Talkgroup entries in the radio.  Use the erase command to delete a talkgroup entry.

Example:

/tkgrp create tag=fire1 p25tkid=19

Create a new Talkgroup entry

OK

/TOT	Transmit Timeout Control

Command: TOT

Syntax: /TOT [ # | OFF ]

Parameters:

none		- Displays current TOT value

#		- Sets TOT value to #, (INT 0 to 300, increments 30, 0 = infinite)

OFF		- Sets TOT value to infinite (0)

Description:

Sets Transmit Timeout value.  Valid numbers are integers 0 through 300, in increments of 30.  Numbers not in increments of 30 will be rounded down to the next valid number.  TOT value of 0 disables turnaround (infinite transmit allowed)

Example:

/tot

TX Timeout: 270 Sec

OK

/tot 45

TX Timeout: 30 Sec

OK

/TXSQ	Pick List A/D Transmit Squelch Control ( Fire Feature )

Command: TXSQ

Syntax: /TXSQ [ Create [ tag | id | a_mode | a_value | tx_nac ] | List ]

Parameters:

None		- Lists current txsq parameters loaded in radio

Create	- Creates a new entry into the txsq pick list

tag	- An 8 character tag for the entry

id	- ID for this entry, this will be used for both the reference number in the list (pick ID) and  the database ID.  If not specified, the next available ID will be used.  Valid rage=1-32.

a_mode	- Analog squelch mode

0 = none

1 = CTCSS

2 = DCS

a_value	- Analog Squelch value, index into the ctcss_tone_value or octal_dcs_value array, depending on value entered.  The values are shown in Appendix A for reference.

tx_nac	- Digital TX NAC value

List  		- Lists current txsq parameters loaded in radio

Description:

Loads/Lists the A/D Transmit squelch entries in the radio.  Use the erase command to delete a tx squelch entry

Example:

/txsq create tag=DCS023 a_mode=2 a_value=1 tx_nac=200

/txsq create tag=DCS025 a_mode=2 a_value=2 tx_nac=201

/txsq create tag=DCS026 a_mode=2 a_value=3 tx_nac=202

/txsq create id=32 tag=DCS754 a_mode=2 a_value=82 tx_nac=212

Create a new Txsq entry

ID=32

OK

/UNITid	Pick List UnitID Control ( Fire Feature )

Command: UNITid

Syntax: /UNITid [ Create [ tag | id | p25id ] | List ]

Parameters:

None		- Lists current Unit ID entries loaded in radio

Create	- Creates a new entry into the unitid pick list

tag	- An 8 character tag for the entry

id	- ID for this entry, this will be used for both the reference number in the list (pick ID) and  the database ID.  If not specified, the next available ID will be used.  Valid rage=1-99.

p25id	- The P25 radio unit ID

List  		- Lists current Unit ID entries loaded in radio

Description:

Loads/Lists the Unit ID entries in the radio.  Use the erase command to delete a unitid entry

Example:

/unitid create tag=steve p25id=74

/unitid create tag=bill p25id=52525

/unitid create tag=joe p25id=49343

/unitid create tag=ken p25id=1

/unitid create id=99 tag=mike p25id=2

Create a new Unitid entry

ID=99

OK

/USRID	User ID Value

Command: USRID

Syntax: /USRID [#]

Parameters:

none		- Displays current USRID value

#		- Sets USRID value to #

Description:

Sets User ID value.  Valid numbers are integers 1 through 9,999,999.

Example:

/USRID 4200

UserID = 4200

OK

/VER	Version Display

Command: VER

Syntax: /VER

Parameters:

none		- Displays radio version status

Description:

Displays version status

Example:

/ver

Racal 25 Radio - Thales Communications Inc.

ESN            : A012580000000101

Factory Serial : A012580000000101

Boot           : P6725-27-01-02.00.000 V4.0 Control compatible

Control        : P6725-27-02-00.08.049 TCI Racal25 Software

DSP            : P6725-27-03-05.00.002 UNDEFINED

Keypad         : P6725-27-04-02.04.000 Relm AVR build

Control Build  : Jan 15 2001 16:02:34

Radio Serial   : 00230

Radio Config   : 0x00000007

OK

/VOL	Volume Control

Command: VOL

Syntax: /VOL [ # | + |  -  ]

Parameters:

none		- Displays current volume setting

#		- Sets the volume setting to # ( 0-15 ).

+		- Increments volume position by 1.

-		- Decrements volume position by 1.

Description:

This command sets the radio volume level between the limits of zero (mute) and 15.

Example:

/vol 0

Volume = 0

OK

/vol +

Volume = 1

OK

/vol 10

Volume = 10

OK

/XT			Frequency Offset

Command: XT

Syntax: /XT [ # ]

Parameters:

none		- Displays current XT value

#		- Sets the crystal tune value to # (-32768 to +32768).

Description:

Sets crystal tune value to specified value.  It is always referenced from 0, and corresponds to the 12 –bit DAC, and not directly to Hz values.

Example:

/xt

XTune Offset = 10

OK

/xt 20

XTune Offset = 20

OK

/ZONE	Zone Control and Programming

Command: Zone

Syntax: /Zone [ # | NEW [#] | Pos # ] [ TAG ["<name>"] ] [ SCan Enable | Disable ] [ CHAN # [ slot # | delete ] ]

Parameters:

#		- Specifies a zone number to select.

NEW		- Specifies a new zone to select.

#	- Specifies new zone to be created with the specified zone number.

Pos #   	- Specifies position # to select within the current bank. Where

the # is in the range of 1 to 16.

TAG		- Displays the selected zone's tag.

"<name>"  	- Sets the selected zone's tag to <name>, quotes are required.

SCan

Enable	- Sets the Zone scanlist option to enabled

Disable 	- Sets the Zone scanlist option to disabled

CHAN #	- Sets channel # to this zone

Slot #	- Sets channel # to slot # of this zone (Slot numbers 1 – 16)

Delete	- Deletes channel # from this zone

Description:

Selects and programs the ZONE parameters in the radio.  When creating a new zone the minimum options required to add a zone is shown in the example.  You may specify the zone ID value after the new option or the radio will choose the next available value.  In order to position a channel within a zone you must specify the channel and channel number and the slot and slot number in the command as shown.  In order to delete a channel from a zone you must supply the channel and channel number in the command along with the position number to delete after the delete option as shown.

Example:

/zone 4

OK

/zone new 3 tag "ZONE 3"

Zone ID:    3

Tag = ZONE 3

OK

/zone 1 chan 2 slot 2

Zone ID:    1

Channels: 1, 3, 2, 11, 12, 13, 14, 15, 9, 7, 29, 30, 1, 0, 0, 0

OK

/zone 1 chan 2 delete 3

Zone ID:    1

Channels: 1, 3, 0, 11, 12, 13, 14, 15, 9, 7, 29, 30, 1, 0, 0, 0

OK

### Appendix A:

| Value | CTCSS Tone |  | Value | DCS Code |
| --- | --- | --- | --- | --- |
| 0 | "NONE " |  | 0 | 0 |
| 1 | " 67.0 XZ" |  | 1 | 23 |
| 2 | " 69.3 WZ" |  | 2 | 25 |
| 3 | " 71.9 XA" |  | 3 | 26 |
| 4 | " 74.4 WA" |  | 4 | 31 |
| 5 | " 77.0 XB" |  | 5 | 32 |
| 6 | " 79.7 WB" |  | 6 | 43 |
| 7 | " 82.5 YZ" |  | 7 | 47 |
| 8 | " 85.4 YA" |  | 8 | 51 |
| 9 | " 88.5 YB" |  | 9 | 54 |
| 10 | " 91.5 ZZ" |  | 10 | 65 |
| 11 | " 94.8 ZA" |  | 11 | 71 |
| 12 | " 97.4 ZB" |  | 12 | 72 |
| 13 | "100.0 1Z" |  | 13 | 73 |
| 14 | "103.5 1A" |  | 14 | 74 |
| 15 | "107.2 1B" |  | 15 | 114 |
| 16 | "110.9 2Z" |  | 16 | 115 |
| 17 | "114.8 2A" |  | 17 | 116 |
| 18 | "118.8 2B" |  | 18 | 125 |
| 19 | "123.0 3Z" |  | 19 | 131 |
| 20 | "127.3 3A" |  | 20 | 132 |
| 21 | "131.8 3B" |  | 21 | 134 |
| 22 | "136.5 4Z" |  | 22 | 143 |
| 23 | "141.3 4A" |  | 23 | 152 |
| 24 | "146.2 4B" |  | 24 | 155 |
| 25 | "151.4 5Z" |  | 25 | 156 |
| 26 | "156.7 5A" |  | 26 | 162 |
| 27 | "162.2 5B" |  | 27 | 165 |
| 28 | "167.9 6Z" |  | 28 | 172 |
| 29 | "173.8 6A" |  | 29 | 174 |
| 30 | "179.9 6B" |  | 30 | 205 |
| 31 | "186.2 7Z" |  | 31 | 223 |
| 32 | "192.8 7A" |  | 32 | 226 |
| 33 | "203.5 M1" |  | 33 | 243 |
| 40 | "206.5 8Z" |  | 34 | 244 |
| 34 | "210.7 M2" |  | 35 | 245 |
| 35 | "218.1 M3" |  | 36 | 251 |
| 36 | "225.7 M4" |  | 37 | 261 |
| 41 | "229.1 9Z" |  | 38 | 263 |
| 37 | "233.6 M5" |  | 39 | 265 |
| 38 | "241.8 M6" |  | 40 | 271 |
| 39 | "250.3 M7" |  | 41 | 306 |
| 42 | "254.8 0Z" |  | 42 | 311 |
|  |  |  | 43 | 315 |
|  |  |  | 44 | 331 |
|  |  |  | 45 | 343 |
|  |  |  | 46 | 346 |
|  |  |  | 47 | 351 |
|  |  |  | 48 | 364 |
|  |  |  | 49 | 365 |
|  |  |  | 50 | 371 |
|  |  |  | 51 | 411 |
|  |  |  | 52 | 412 |
|  |  |  | 53 | 413 |
|  |  |  | 54 | 423 |
|  |  |  | 55 | 431 |
|  |  |  | 56 | 432 |
|  |  |  | 57 | 445 |
|  |  |  | 58 | 464 |
|  |  |  | 59 | 465 |
|  |  |  | 60 | 466 |
|  |  |  | 61 | 503 |
|  |  |  | 62 | 506 |
|  |  |  | 63 | 516 |
|  |  |  | 64 | 532 |
|  |  |  | 65 | 546 |
|  |  |  | 66 | 565 |
|  |  |  | 67 | 606 |
|  |  |  | 68 | 612 |
|  |  |  | 69 | 624 |
|  |  |  | 70 | 627 |
|  |  |  | 71 | 631 |
|  |  |  | 72 | 632 |
|  |  |  | 73 | 654 |
|  |  |  | 74 | 662 |
|  |  |  | 75 | 664 |
|  |  |  | 76 | 703 |
|  |  |  | 77 | 712 |
|  |  |  | 78 | 723 |
|  |  |  | 79 | 731 |
|  |  |  | 80 | 732 |
|  |  |  | 81 | 734 |
|  |  |  | 82 | 743 |
|  |  |  | 83 | 754 |